PRE-QUALIFICATION PROGRAM

Work Type Definition / Submittal Requirements

# Work Type Definition

## 10.1 Photogrammetric Mapping

### Description

Photogrammetric mapping is a methodology by which precision flown aerial photographs, accurate camera and ground control data are combined with data capture to produce planimetric and topographic maps.  This work type includes the following:

- Evaluation of project control to ensure adequate control is present to produce mapping within the specified accuracy requirements.

- Production of an aero-triangulation solution that bridges the ground survey and provides control for photogrammetric mapping.

- Collection of planimetric (cultural) features.

- Production of a Digital Terrain Model (DTM) that will support Detail Design activities for transportation projects (1’ C.I.).

- Production of digital Orthophotos (as defined:  seamless orthorectified digital raster mosaics) for design or pre-design purposes.

### Standards and Specifications

Standards and specifications required for a project under this work type may include the following:

- All work completed must be in accordance with the Performance Based Specifications and Procedural Based Specifications identified in the contract for a specific project.

- Approved map files in Microstation Design File format. Production and delivery of 100% clean, edited digital data in Microstation Design File format is required.  Click here for CADD Standards [www.dot.state.mn.us/caes/cadd]

- Approved terrain model files in Microstation .DGN Graphics File format. Production and delivery of 100% clean, edited digital data in Microstation .DGN Graphics File format is required.

- Aero-triangulation, Digital Elevation Model (DEM), and Orthophoto specifications in www.dot.state.mn.us/surveying/Photo/photospecs.html  take precedence over the Surveying and Mapping Manual [www.dot.state.mn.us/surveying/SurvMapManuals/SM_Manual2007.pdf]

### Provided by MnDOT

Information to be supplied by MnDOT for a project may include the following:

- Project designation, location, limits and match file requirements.

- Coordinate Datum/Projection/Adjustment.

- Photo Index, Control Scheme, Contact Prints, Digital Images, if applicable.

- Calibration reports for aerial film camera(s).

### Provided by Consultant

Deliverables to be supplied by the consultant for a project may include the following:

- Photogrammetric maps per specific project specifications.

- Orthophoto Production

- Aero-triangulation solution production, planimetric feature compilation and/or Digital Terrain Model (DTM) compilation for MnDOT transportation projects and Minnesota local units of government projects.

- Photo Index with all control points used marked.

- Evaluation Stage:

  - Map content, feature portrayal, format and level placement.

  - Terrain model structure, format and level placement.

- Final Submittal Stage:

  - Aero-triangulation solution files, QA-QC data and error residual files.

  - Final camera station positions from least squares adjustment, along with orientation parameters ,,.

  - Aero-triangulation report containing narrative about the project and list of ground control points not used or identified as being suspect.

  - Orthophoto Index.

  - Raster Files of Project Orthophotos JPEG compressed with embedded overviews.

  - A “File Limits Diagram” showing the limits, boundary and name for each project file.

  - All contact prints, annotated with control data, which were utilized as part of the project.

  - Copy of photo index with all control points, used in project, marked on it.

# Work Type Submittal Requirements

A consultant firm becomes pre-qualified based on the qualifications of the personnel that are employed by the firm.

| ### Key Personnel Requirements | ### Description |
| --- | --- |
| - Minimum Number of Staff | At least one Civil Engineer, Land Surveyor, or Certified Photogrammetrist with demonstrated experience in this work. |
| - Professional Certification / Licensure | All work for this work type must be completed under the direct supervision of one of the following: Licensed Professional Civil Engineer Licensed Professional Land Surveyor Certified Photogrammetrist ASPRS Certified Photogrammetrist |

| ### Work Type Submittal Requirements | ### Description |
| --- | --- |
| - Resume and Relevant Project Experience Form - Form PQ1 - [Submit in Microsoft Word format] | - Complete Parts 1, 1A, 2 and 3 of Form PQ1 Part 1: Fill out general information and name(s) of personnel Part 1A: The firm’s staff must demonstrate a minimum of 5 years professional experience in the activities associated with this work type. Demonstrate satisfactory experience on at least two but no more than 5 substantial (at least a 20 model urban project) projects in the last 3 years in which personnel utilized aero-triangulation solution production, planimetric (cultural) feature compilation, Digital Terrain Model compilation and Ortho-photo production for transportation type engineering projects/county-township-city municipal category projects. Part 2: Examples may be requested by the work type committee at a later time. Part 3: Provide the following in this order: - Electronic Firm Personnel Profile (Note: All personnel profiles will be categorized by 0-5 yrs., 5-10 yrs., 10-15 yrs, 15-20 yrs and >20 yrs): - Full Time Employees (FTE’s) of Company or Photogrammetric Division (categorized). - FTE’s in Aero-triangulation (categorized). - FTE’s in Compilation (categorized). - FTE’s in Edit/ Programming (categorized). - FTE’s in Ortho-production (categorized). - Hardware Profile - Number and type of workstations available for A-T. - Number and type of workstations available for planimetric & DTM compilation. - Number and type of workstations available for Ortho-production. - Software and Production Technique Profile (QA/QC Plan) (Word or Excel spreadsheet format compatible): - Detailed description of software and production techniques used to perform and certify Aero-triangulation solutions. - Detailed description of software and production techniques used to collect and certify planimetric feature and Digital Terrain Model compilation. - Detailed description of software and production techniques used to produce and certify Ortho-photos. - Project Profile: Submit an electronic spreadsheet file (Excel or Word spreadsheet format compatible) that lists transportation and municipal mapping projects that have been completed within the last 3 years. In addition, include the following information for each project: - Project Owner - Project Description. - Photo Scale. - Aero Triangulation model count. - Planimetric & DTM Compilation model count. - Ortho-photo model count. - Total Project Cost. - Total Project Duration (months). |
| - Project Example Requirements - [Submit in PDF format] | Upon further request. |
| - Proof of Certification / Licensure [Submit in PDF format] | Provide a current copy of the applicable Professional Certification/ Licensure. |
| - CADD Requirements - [Submit in PDF format] | Complete and sign form indicting the firm is capable of using MnDOT Level 2 CADD Standards: Affidavit for MnDOT’s Level 2 CADD Standards |

| ### Work Type Submittal Instructions | ### Description |
| --- | --- |
| - Instructions for Submittal [Each file should be saved in the format identified.] | Submit 5 flash drives that include the following individual files or folders in this order: Resume and Relevant Project Experience Form - Form PQ1 [Submit in Microsoft Word format] Project Example Requirements [Not Required at this Time] Proof of Professional Certification/Licensure [Submit in PDF format] CADD Requirements |