### 3890	Compost

### 3890.1	SCOPE

Provide compost material for soil amendment for landscape planting or turf establishment.

### 3890.2	REQUIREMENTS

Provide material from vendors listed on the Approved/Qualified Products List.

Provide compost meeting the US Composting Council Seal of Testing Assurance Program and requirements for animal or plant based feedstocks or the following

(1)	Consists of a natural humus product derived from the aerobic decomposition of organic wastes.

(2)	Considered mature and useable when 60 percent decomposition has been achieved as determined by an ignition-loss analysis test method and any one additional test method including the Solvita test value of equal to or greater than 5.  This means that the compost product has no offensive smell, no identifiable organic materials, and will not reheat to more than 20 °F above the ambient temperature.

(3)	Produced by a process to further reduce pathogens (PFRP) and weed seeds, and the process is verified by fecal coliform or Salmonella sp. tests, where applicable.

(4)	Compost foreign particle restrictions up to 3 percent at 0.16 in apply to the shredded pieces from the plastic bags used to transport feedstocks to the composting facility, but will be considered acceptable if visible in the finished product.

(5)	Meets the Minnesota Pollution Control Agency requirements for allowable levels of inherent contaminants (Minnesota Rules Chapter 7035.2836 subp. 6 Sec A), or the Code of Federal Regulations, Title 40, section 503.13(b)(3), amended for mercury.

Provide compost that is registered for sale with the State of Minnesota.  Do not mix the compost with materials that do not comply with the Minnesota Rules Chapter 7045 (Hazardous Waste).  Compost must not exceed 10 percent of the Minnesota Pollution Control Agency's Superfund residential soil cleanup guidelines, termed Soil Reference Values or SRVs (i.e. 10% of individual chemical or chemical mixture Hazard Index, Hazard Quotient, or acceptable cancer risk level).  No chemical contaminant, including pesticides, can be present in concentrations that would result in toxic effects to soil organisms, plants, or animals that reside in or on the composted soil areas or use the treated area for food or shelter.  At the time of delivery to the project, the compost shall be in a condition considered safe for exposure to dusts during handling.

A	Grade 1 Compost

Provide Grade 1 compost for use in turf establishment meeting the following requirements and characteristics:

(1)	Nutrient rich type,

(2)	Derived from the decomposition of animal material and animal byproducts

(3)	Texture similar to a highly organic soil, and

(4)	Meeting the requirements of Table 3890-1.

| Table 3890-1 Grade 1 Compost Requirements |  |
| --- | --- |
| Requirement | Range |
| Organic matter content | ≥ 30% |
| C/N ratio | 6:1 – 20:1 |
| NPK ratios* | 2:2:1 – 4:4:2 wet weight |
| pH | 5.5 – 8.0 |
| Moisture content | 35% – 55% |
| Bulk density | 700 lb per cu. yd – 1,600 lb per cu. yd |
| Inert material | ≤ 3% at 0.15 in |
| Soluble salts | ≤ 10 mmho per cm |
| Germination test ║ | 80% – 100% |
| Screened particle size | ≤ ¾ in |
| * To obtain the nitrogen, phosphorus, or potassium levels specified, the compost may be fortified with commercial fertilizer. ║ Germination test must list the species of Cress or lettuce seed used. |  |

B	Grade 2 Compost

Provide Grade 2 compost as a landscape planting medium and meeting the following requirements:

(1)	Derived from the decomposition of leaves, yard wastes,  or Source Separated Organic Material

(2)	Texture similar to a shredded peat, and

(3)	Meeting the requirements of Table 3890-2:

| Table 3890-2 Grade 2 Compost Requirements |  |
| --- | --- |
| Requirement | Range |
| Organic matter content | ≥ 30 % |
| C/N ratio | 6:1 – 20:1 |
| NPK ratios | 1:1:1 maximum wet weight |
| pH | 5.5 – 8.5 |
| Moisture content | 35% – 55% |
| Bulk density | 700 lb per cu. yd – 1,600 lb per cu. yd |
| Inert material * | < 3% at 0.15 in |
| Soluble salts | ≤ 10 mmho per cm |
| Germination test ║ | 80% – 100% |
| Screened particle size | ≤ ¾ in |
| * Includes plastic bag shreds. ║ Germination test must list the species of Cress or lettuce seed used. |  |

Source Separated Organic Material

Facilities accepting  SSOM for composting are required to follow the US Composting Council Seal of Testing Assurance Program.

C	Grade 3 Compost

Provide Grade 3 as a blend of Grade 2 compost and no greater than 10 percent Grade 1 compost.

### 3890.3	SAMPLING AND TESTING

Before delivery of the compost to the project, submit to the Engineer certified test reports and a manufacturer’s Certificates of Compliance.

If federal or state chemical specific requirements conflict, provide compost meeting the most stringent requirement.