Scenario #3

Multi-Unit Building With a Benefited Common Area & Some Unit-Specific Areas of Frequent Human Use

Benefited Common Area

Units      3, 7, 11

Units         4, 8, 12

Multi-Unit Building

Units                       1, 5, 9

Units                  2, 6, 10

Proposed Noise Barrier

Roadway

Assumptions for this example:

- Three-story building with 4 units per floor (12 units total with 2 units on roadway side and 2 units on common area side of building).

- The common area is an area of frequent human activity available to all building residents (i.e., a pool).

- The building owner does not live in the building.

- Units 1-8 have unit-specific areas of frequent human use (i.e. balcony or patio).  Units 9-12 do not have unit-specific areas of frequent human use.

- Applicable noise abatement criteria are approached or exceeded in the common area or the common area has at least a 5dBA increase with design year traffic.

- The feasible and cost-effective noise abatement measure obtains at least a 5 dBA reduction in the noise level in the common area and at least one benefited receiver gets a 7dBA reduction.

| Voting Scenarios |  |  |  |
| --- | --- | --- | --- |
| Unit | Voting Scenario I | Voting Scenario II | Voting Scenario III |
| 1 (Roadway side & benefited receptor) | Yea (Res. 2 pts./BR) | Yea | Nay |
| 2 (Roadway side & benefited receptor) | Yea (Res. 2 pts./BR) | Yea | Nay |
| 3 (Common area side & benefited receptor) | Yea (Res. 2 pts./BR) | Yea | Nay |
| 4 (Common area side & benefited receptor) | Yea (Res. 2 pts./BR) | Yea | Nay |
| 5 (Roadway side & benefited receptor) | No response (Res. 2 pts./BR) | Yea | Nay |
| 6 (Roadway side & benefited receptor) | No response (Res. 2 pts./BR) | Yea | Nay |
| 7 (Common area side but not benefited receptor) | Not entitled to vote. | Not entitled to vote. | Not entitled to vote. |
| 8 (Common area side but not benefited receptor) | Not entitled to vote. | Not entitled to vote. | Not entitled to vote. |
| 9 | Not entitled to vote. | Not entitled to vote. | Not entitled to vote. |
| 10 | Not entitled to vote. | Not entitled to vote. | Not entitled to vote. |
| 11 | Not entitled to vote. | Not entitled to vote. | Not entitled to vote. |
| 12 | Not entitled to vote. | Not entitled to vote. | Not entitled to vote. |
| Building Owner | Nay | Nay | Yea |
| Totals |  |  |  |
| Total Possible Points | 12 (2 pts./BR*6 BR for residents) + 28 ((4pts/BR *6 BR) + 4 pts. for benefited common area for owner) = 40 | 40 | 40 |
| Points for Yeas | 8 | 12 | 28 |
| Points for Nays | 28 | 28 | 12 |
| 50% Total Possible Points | 20 | 20 | 20 |
| Does the barrier get built? | No | No | Yes |

Notes:

- All benefited residents in this multi-unit building are considered “first row”; including those on the common area side of the building and those on above ground floors.

- Cost effectiveness calculation: Every unit has access to the common area and the common area is benefited, therefore, the cost threshold for the noise abatement measure is calculated as $43,500*12 units = $522,000.    All of the units are, in effect, benefited receptors for purposes of establishing the cost threshold because the common area gets a 5dBA (or more) reduction from the proposed noise abatement measure.  However, only the benefited receptors that have balconies or patios are solicited for votes.

Res. = Resident

Pts. = Points

NR = No Response

BR = Benefited Resident