DIVISION ST

Section	P

No.	Item

ST-1	(2013) SAFETY CABLE	1

ST-2	(2104) REMOVING MISCELLANEOUS STRUCTURES	1

ST-3	(2564) TRAFFIC SIGNS AND DEVICES	1

ST-4	(3352) SIGNS, DELINEATORS AND MARKERS	1

DELETE WHEN DONE: To Update Table of Contents Right Click on the index above and choose Update Field.  If given a choice choose “Update Entire Table”.

DELETE WHEN DONE: Statements highlighted in yellow are guidelines or instructional in nature.  Remove these notes before completing the spec.  When appearing at the top left of a provision, it pertains to the paragraph immediately beneath the note, as well as any indented items following it.

DELETE WHEN DONE: Words highlighted in green are fields that may need to be modified or removed before completion of the spec, such as contact information, city names, sign numbers, or charts within the plan.

I hereby certify that the Special Provisions for traffic sign construction (Division ST) contained in this proposal were prepared by me or under my direct supervision and that I am a duly licensed Professional Engineer under the laws of the State of Minnesota.

_________________________________________                            Engineer Name

Lic.  No.  XXXXX                    Date MM/DD/YY

age

No.

-ST

-ST

-ST

-ST

# DIVISION ST

# (2013) SAFETY CABLE

This work shall consist of furnishing and installing safety cable on overhead sign structures in accordance with the applicable provisions of MnDOT 2564, the Plan and the following:

## CONSTRUCTION REQUIREMENTS

For the Type OH signs designated in the Plan, the Contractor shall furnish and install brackets, aircraft cable and all necessary hardware to assemble and attach a safety cable as detailed in the Plan.

## METHOD OF MEASUREMENT AND BASIS OF PAYMENT

Payment for Item No. 2013.603 (SAFETY CABLE) at the contract price per LINEAR FOOT shall be compensation in full for all work, materials and costs involved in performing the work as specified above and in the Plan.

# (2104) REMOVING MISCELLANEOUS STRUCTURES

This work shall consist of removing or salvaging miscellaneous structures in accordance with the provisions of MnDOT 2104 and the following:

## CONSTRUCTION REQUIREMENTS

Use for projects that include LOGO signs.

The Contractor shall give 14 calendar days advance notice to Dave DeSutter, General Manager, Minnesota Logos, at 952-895-8079 to have him arrange to remove the Logo signs designated in the Plan as “SALVAGE BY OTHERS”.

### Remove Sign Walkway

Use for walkways on cantilevers or sign bridges without lighting systems.

For Sign(s) OH XX-XX, the Contractor shall:

Remove the walkway, including the walkway supports.

Repair cut and/or damaged galvanized surfaces in accordance with ASTM A780.

Materials used for repair shall be applied according to manufacturer’s instructions.

After removal of the walkway from the sign truss:

Furnish high strength bolts, nuts and washers meeting the requirements of MnDOT 3391.2B and galvanized in accordance with MnDOT 3392 for each open hole in the sign truss.

Install the high strength, bolts, nuts and washers in the open holes in the sign truss and tighten in accordance with MnDOT 2402.3G2c.

Use for walkways for signs on bridges without lighting systems.

For Sign(s) OH XX-XX, the Contractor shall:

Cut each walkway support so that 2 inches of each support extend beyond the face of the sign panel and remove the walkway supports attached to the walkway and the feed point identification plate.

Repair cut and/or damaged galvanized surfaces in accordance with ASTM A780.

Materials used for repair shall be applied according to manufacturer’s instructions.

Use for walkways with lighting systems on cantilevers or sign bridges.

For Sign(s) OH XX-XX, the Contractor shall:

Disconnect the power cable at the source of power (as indicated in the Plan) and the sign base and tape the ends.  If the source of power is a service cabinet, disconnect the power conductors from the circuit breaker(s) and shut off the circuit breaker(s).

Contact John Pedersen at Metro Traffic Lighting Operations (651-234-7849) for final acceptance of the disconnection at the source of power.

Resplice and waterproof any power cable connections required to maintain roadway lighting in accordance with MnDOT 2545.

Disconnect and remove the conductors between the sign lighting fixtures and the switch and between the switch and the sign base.  Remove fuse connectors where encountered in the sign base.

Disconnect and remove the conduit between the sign lighting fixtures and the switch (if not attached to the sign post) and between the switch and the conduit stub on the sign post and cap the conduit stub.

Remove the switch.  If the switch is mounted on the sign post, cap the opening in the sign post to the satisfaction of the Engineer.

Dismantle the sign lighting fixtures and ballasts (if encountered) from the sign structure.

Completely remove the walkway, including the walkway supports and the feed point identification plate.

Repair cut and/or damaged galvanized surfaces in accordance with ASTM A780.

Materials used for repair shall be applied according to manufacturer’s instructions.

After removal of the walkway from the sign truss:

Furnish high strength bolts, nuts and washers meeting the requirements of MnDOT 3391.2B and galvanized in accordance with MnDOT 3392 for each open hole in the sign truss.

Install the high strength, bolts, nuts and washers in the open holes in the sign truss and tighten in accordance with MnDOT 2402.3G2c.

### Remove Grout - Sign Footing

The Contractor shall remove all in place grout between the base plate and the sign footing for signs indicated in the Plan.  Care should be taken so as not to damage the in place anchor bolts, nuts, washers and conduit.

### Remove Sign Lighting System

Use for removing existing and installing new lighting on cantilever or sign bridge (safety cable and lighting specs are not included in masterspec document)

For Sign(s) OH XX-XX, the Contractor shall:

Disconnect the power conductors at the source of power (as indicated in the Plan) and the sign base.  Both ends shall be taped for reuse for the new sign lighting system.  Resplice and waterproof any power cable connections at the source of power required to maintain roadway lighting in accordance with MnDOT 2545.

Contact John Pedersen at Metro Traffic Lighting Operations (651-234-7849) for final acceptance of the disconnection at the source of power.

Disconnect and remove the conductors between each sign lighting fixture, the switch and the sign base, including any fuse connectors in the sign base.

Disconnect and remove all conduit between the sign lighting fixtures, the switch and conduit stub on the sign post.

Remove the switch.  If the switch is mounted on the sign post, cap the opening in the sign post to the satisfaction of the Engineer.

Dismantle the sign lighting fixtures and ballasts (if encountered) from the sign structure.

Remove the feed point identification plate.

Use for bridge mounted sign with conduit and SOP above

For Sign(s) OH XX-XX, the Contractor shall:

Disconnect the power conductors at the source of power (as indicated in the Plan) and the switch and tape the conductors at the source of power.  Resplice and waterproof any power cable connections at the source of power required to maintain roadway lighting in accordance with MnDOT 2545.

Contact John Pedersen at Metro Traffic Lighting Operations (651-234-7849) for final acceptance of the disconnection at the source of power.

Disconnect and remove conduit between the switch and the conduit stub in the bridge.  Cut the power conductors extending out of the conduit stub so that 12 inches of each conductor extends out of the stub.  Tape each power conductor, loop and insert into the conduit stub and cap the conduit stub.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and ballasts (if encountered) from the sign structure.

Remove the feed point identification plate.

Use for bridge mounted sign with junction box as SOP

For Sign(s) OH XX-XX, the Contractor shall:

Disconnect the power conductors at the source of power (as indicated in the Plan) and the switch.  Resplice and waterproof any power cable connections at the source of power required to maintain roadway lighting in accordance with MnDOT 2545.

Contact John Pedersen at Metro Traffic Lighting Operations (651-234-7849) for final acceptance of the disconnection at the source of power.

Remove the power conductors between the source of power and the switch.

Disconnect and remove conduit between the switch and the conduit stub in the bridge and cap the conduit stub.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and ballasts (if encountered) from the sign structure.

Remove the feed point identification plate.

Use if pullbox or streetlight on mainline is the SOP (this is the old bridge mounted method)

For Sign(s) OH XX-XX, the Contractor shall:

Disconnect the power conductors at the source of power (as indicated in the plan) and at the switch.  Resplice and waterproof any power cable connections at the source of power required to maintain roadway lighting in accordance with MnDOT 2545.

Contact John Pedersen at Metro Traffic Lighting Operations (651-234-7849) for final acceptance of the disconnection at the source of power.

Cut the power cable at the condulet on top of the 2 inch R.S.C. attached to the bridge pier so that 12 inches of cable extends out of the condulet and tape the cable end.  Remove the power cable and cable straps between the condulet and the switch.  Plug holes from removed hangers in concrete with epoxy grout.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and ballasts (if encountered) from the sign structure.

Remove the feed point identification plate.

Upon removal of the sign structure from the bridge, fill anchorage holes in concrete with epoxy grout and/or plug holes in steel with galvanized high strength steel bolts, washers and nuts.

### Remove Sign Type A

For Sign(s) A-XX, the Contractor shall completely remove the concrete footings or H-pile footings.

### Remove Sign Type C and Type D

For each mast arm mounted sign being removed, the Contractor shall remove the bracketing and hardware attaching each sign panel to the traffic signal mast arm.

For each sign mounted on a light fixture, the Contractor shall remove the bracketing and hardware attaching each sign panel to the light fixture.

For each bridge rail mounted sign being removed, the Contractor shall remove the bridge rail mounted sign structure and mounting hardware and fill anchorage holes in concrete with epoxy grout.

For each bridge mounted sign being removed, the Contractor shall remove the sign panel, mounting hardware and bridge mounted sign structure and fill anchorage holes in concrete with epoxy grout and plug holes in steel with galvanized high strength steel bolts, washers and nuts upon removal of the bridge mounted sign structure.

Use for sign replacement projects.

The Contractor shall schedule the work so that no sign is removed (that is being replaced by a new sign) unless the new sign is installed the same work day.

### Remove Sign Type EA and EO

The Contractor shall remove and dispose of the sign panel, post clips, and flanged channel posts.

### Remove Sign Type OH

For Sign(s) OH XX-XX and OH XX-XX, the Contractor shall:

Disconnect the power conductors at the source of power (as indicated in the plan) and in the sign base.  Resplice and waterproof any power cable connections at the source of power required to maintain roadway lighting in accordance with MnDOT 2545.  Shut off the circuit breaker(s) in service cabinets.

Contact John Pedersen at Metro Traffic Lighting Operations (651-234-7849) for final acceptance of the disconnection at the source of power.

Completely remove the concrete footing(s).  Bury the end of the power cable and abandon in place as directed by the Engineer

### Remove Sign Type OH (Bridge Mounted)

For Sign(s) OH XX-XX, the Contractor shall:

Disconnect the power conductors at the source of power (as indicated in the plan) and at the switch.  Resplice and waterproof any power cable connections at the source of power required to maintain roadway lighting in accordance with MnDOT 2545.

Contact John Pedersen at Metro Traffic Lighting Operations (651-234-7849) for final acceptance of the disconnection at the source of power.

Do not include if bridge is being removed.

Cut the power cable at the condulet on top of the 2 inch R.S.C. attached to the bridge pier so that 12 inches of cable extends out of the condulet and tape the cable end.  Remove the power cable and cable straps between the condulet and the switch.  Plug holes from removed hangers in concrete with epoxy grout.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Remove the feed point identification plate.

Do not include if bridge is being removed or re-decked.

Upon removal of the sign structure from the bridge, fill anchorage holes in concrete with epoxy grout and/or plug holes in steel with galvanized high strength steel bolts, washers and nuts.

Use for bridge redecking projects when fascia beams will remain inplace.

During removal of the sign structure(s) from the bridge, take care not to damage the fascia beam webs.

Use for bridge redecking projects when fascia beams will remain inplace and holes will not be reused.

For Sign(s) OH XX-XX, fill the holes in the fascia beams with epoxy grout.

Use for bridge redecking projects when fascia beams will remain inplace and holes will be reused.

For Sign(s) OH XX-XX, shall not fill the holes in fascia beams; holes will be reused to mount new structures.

Use if junction box is the SOP

For Sign(s) OH XX-XX, the Contractor shall:

Disconnect the power conductors at the source of power (as indicated in the Plan) and the switch.  Resplice and waterproof any power cable connections at the source of power required to maintain roadway lighting in accordance with MnDOT 2545.

Do not include if bridge is being removed.

Remove all conduit and power conductors between the source of power and the switch.  Plug any holes in the junction box as directed by the Engineer.  If the conduit is stubbed out of the bridge, cap the conduit end after removal of the power conductors.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Remove the feed point identification plate.

### Remove Sign Type Special

The Contractor shall completely remove the entire street name sign structure, including concrete footings.

### Remove Sign Panel Type A

The Contractor shall remove the sign panel and post clips.

### Remove Sign Panel Type D

The Contractor shall remove the sign panel and mounting hardware.  The bridge mounted sign structure is to remain in place for attaching a new sign panel.

### Remove Sign Panel Type OH

For Sign OH XX-XX, the Contractor shall remove the left sign panel and panel mounting posts.

Use when existing panels are flat sheet aluminum.

For each of the Type OH sign panels being removed from Sign(s) OH XX-XX, the Contractor shall remove the sign panel and sign bracket assembly(ies) attaching each sign panel to the sign support.

### Salvage Sign Type A

For Sign(s) A-XX, the Contractor shall:

Salvage the in place sign panel and sign posts which will be installed under Install Sign Type A.

Remove the post clips.

Remove the friction fuse and mounting hardware from each sign post.

Remove the concrete footings.

For Sign(s) A-XX, the Contractor shall:

Unbolt the sign post from each base plate and remove the base plate bolts, nuts, and washers.

Remove the friction fuse and mounting hardware from each sign post.

Remove the lower sign post section (from the base plate to the post cut) of each sign post.

Remove the post clips and salvage the sign panel as well as the sign post sections behind the sign panel (from the post cut to the top of the sign panel).  The sign panel and sign post sections will be installed under Install Sign Type A.

### Salvage Sign Type C and Type D

The Contractor shall inform the Engineer or his representative of any damaged in place sign panel prior to salvaging so a deduction will not be made.

When salvaging a sign panel, the Contractor shall remove the nuts, bolts and washers in such a manner so as not to damage the sign panel, and dispose of the sign structure and nuts, bolts and washers.  If the Contractor damages any sign panel, he shall dispose of the damaged, and fabricate a new sign panel in accordance with MnDOT 2564.2F and these Special Provisions, at no cost to the Department or City of __, __ County.

The salvaged sign panels, which will be installed under Install Sign Type __, should be stored in a manner to prevent damage to the aluminum sign panels and the sign sheeting materials.  The Contractor shall NOT:

Store sign panels lying on the ground

Allow reflective surfaces to come in contact with dirt, water and grass

Store sign panels covered with plastic or a tarp

### Salvage Sign Type EA and EO

For Sign(s) EO-XX, the Contractor shall salvage the in place sign panel and dispose of the post clips and flanged channel posts.  The sign panel will be installed under Install Sign Type __.

The salvaged sign panels should be stored in a manner to prevent damage to the aluminum sign panels and the sign sheeting materials.  The Contractor shall NOT:

Store sign panels lying on the ground

Allow reflective surfaces to come in contact with dirt, water and grass

Store sign panels covered with plastic or a tarp

### Salvage Sign Type OH

For Sign(s) OH XX-XX, the Contractor shall:

Disconnect the power conductors at the source of power (as indicated in the plan) and at the switch.  Resplice and waterproof any power cable connections at the source of power required to maintain roadway lighting in accordance with MnDOT 2545.

Disconnect and remove the conductors between the sign lighting fixtures and the switch and between the switch and the sign base.  Remove fuse connectors where encountered in the sign base.

Disconnect and remove the conduit between the sign lighting fixtures and the switch (if not attached to the sign post) and between the switch and the conduit stub on the sign post.

Remove the switch.  If the switch is mounted on the sign post, cap the opening in the sign post to the satisfaction of the Engineer.

Dismantle the sign lighting fixtures and ballasts (if encountered) from the sign structure.

Completely remove the walkway, including the walkway supports and the feed point identification plate.

Repair cut and/or damaged galvanized surfaces in accordance with ASTM A780.

Materials used for repair shall be applied according to manufacturer’s instructions.

After removal of the walkway from the sign truss:

Furnish high strength bolts, nuts and washers meeting the requirements of MnDOT 3391.2B and galvanized in accordance with MnDOT 3392 for each open hole in the sign truss.

Install the high strength, bolts, nuts and washers in the open holes in the sign truss and tighten in accordance with MnDOT 2402.3G2c.

Pull the power cable out of the conduit in the footing prior to removal of the concrete footing.  Tape the ends of the conductors.

Salvage the sign truss (including sign panels and panel mounting posts) and sign post from the concrete footing and store outside the clear zone on the job site as directed by the Engineer.  The sign truss and sign post shall each be placed on sufficient cribbing (railroad ties, etc.) to keep them straight and clear of the ground.  The sign truss and post will be installed under Install Sign Type OH.

Completely remove the concrete footing.  Bury the end of the power cable and abandon in place as directed by the Engineer.

Use for bridge mounted signs.

For Sign(s) OH XX-XX, the Contractor shall:

Salvage the bridge mounted sign structures and sign panels.

Remove bolts, nuts and washers connecting girder webs in a manner to prevent damage to the in place girder webs.

### Salvage Sign Type Special

The Contractor shall inform the Engineer or his representative of any damaged in place sign prior to salvaging so a deduction will not be made.

When salvaging each sign, the Contractor shall salvage the sign post with the bracket assembly and street name plates attached.  If the Contractor damages any sign post, bracket assembly and/or street name plates, he shall dispose of the damaged, and fabricate a new sign post, bracket assembly and/or street name plate marker in accordance with MnDOT 2564.2F and these Special Provisions, at no cost to the Department or City of __, __ County.

The salvaged signs, which will be installed under Install Sign Type Special, should be stored in a manner to prevent damage to the sign post, bracket assembly and street name plates.  The Contractor shall NOT:

Store sign panels lying on the ground

Allow reflective surfaces to come in contact with dirt, water and grass

Store sign panels covered with plastic or a tarp

### Salvage Sign Panel Type A

For Sign(s) A-XX, the Contractor shall salvage the in place sign panel and remove the post clips.

.

### Salvage Sign Panel Type OH

For Sign(s) OH XX-XX, the Contractor shall salvage the in place sign panel and remove the post clips. The Type EO sign panel shall remain attached to the Type OH sign panel, which will be installed under Install Sign Panel Type OH.

The salvaged sign panel(s) should be stored in a manner to prevent damage to the aluminum sign panels and the sign sheeting materials.  The Contractor shall NOT:

Store sign panels lying on the ground

Allow reflective surfaces to come in contact with dirt, water and grass

Store sign panels covered with plastic or a tarp

### Salvage Sign Panel Type C

The Contractor shall inform the Engineer or his representative of any damaged in place sign panel prior to salvaging so a deduction will not be made.

When salvaging a sign panel, the Contractor shall remove the nuts, bolts and washers in such a manner so as not to damage the sign panel.  If the Contractor damages the sign panel, he shall dispose of the damaged, and fabricate a new sign panel in accordance with MnDOT 2564.2F and these Special Provisions, at no cost to the Department or City of __, __ County .

The salvaged sign panel, which will be installed under Install Sign Panel Type C, should be stored in a manner to prevent damage to the aluminum sign panels and the sign sheeting materials.  The Contractor shall NOT:

Store sign panels lying on the ground

Allow reflective surfaces to come in contact with dirt, water and grass

Store sign panels covered with plastic or a tarp

### Salvage Sign Panel Type Special

Use if mounting above Type C sign on U-channel post—not permitted in Metro

The Contractor shall inform the Engineer or his representative of any damaged in place sign panel prior to salvaging so a deduction will not be made.

When salvaging street name sign panels, the Contractor shall salvage the street name plates (note if there are two plates mounted back to back) and bracket assembly with the sign panels remaining attached to the bracket assembly, and the mounting hardware attaching the bracket assembly to the flanged channel sign post(s).  If the Contractor damages any bracket assembly and/or street name plates, he shall dispose of the damaged, and fabricate a new bracket assembly and/or street name plates, in accordance with City of __,__County specifications, MnDOT 2564.2F and these Special Provisions, at no cost to the Department or City of __, __ County.

The salvaged sign panel, including bracket assembly and mounting hardware, which will be installed under Install Sign Panel Type Special, should be stored in a manner to prevent damage to the aluminum sign panels and the sign sheeting materials.  The Contractor shall NOT:

Store sign panels lying on the ground

Allow reflective surfaces to come in contact with dirt, water and grass

Store sign panels covered with plastic or a tarp

### Salvage Delineator and Marker

The Contractor shall inform the Engineer or his representative of any damaged in place marker or delineator prior to salvaging so a deduction will not be made.

When salvaging a delineator or marker, the Contractor shall remove the nuts, bolts and washers in such a manner so as not to damage the delineator or marker, and dispose of the sign structure and nuts, bolts and washers.  If the Contractor damages any delineator or marker, he shall dispose of the damaged, and fabricate a new delineator or marker in accordance with MnDOT 2564.2F and these Special Provisions, at no cost to the Department or City of __, __ County.

Salvaged delineators and markers, which will be installed under Install Delineator and Install Marker, should be stored in a manner to prevent damage to the aluminum sign panels and the sign sheeting materials.  The Contractor shall NOT:

Store sign panels lying on the ground

Allow reflective surfaces to come in contact with dirt, water and grass

Store sign panels covered with plastic or a tarp

### Miscellaneous

The Contractor shall deliver all mercury vapor lamps and all ballasts NOT marked “NON-PCB” (unmarked ballasts are assumed to be PCB ballasts) to one of the following companies:

Veolia ES Technical Solutions, L.L.C. at 2960 Yorkton Boulevard, Little Canada

(952-736-3335)

Green Lights Recycling, Inc. at 10040 Davenport Avenue N.E., Blaine

(763-785-0456, ext. 19)

Any costs incurred by either company for proper disposal of lamps and pcb ballasts shall be paid by the Contractor at his sole cost and expense.

The Contractor shall provide written documentation to the Engineer that lamps and PCB ballasts were delivered to one of these companies.

The lighting fixture housings and all other components shall be disposed of in accordance with MnDOT 2104.

## METHOD OF MEASUREMENT AND BASIS OF PAYMENT

Payment for Item No. 2104.501 (REMOVE SIGN WALKWAY) at the contract price per LINEAR FOOT shall be compensation in full for all work, material and costs involved in performing the work as specified above and in the Plan.

Payment for Item No. 2104.509 (REMOVE GROUT – SIGN FOOTING), (REMOVE SIGN LIGHTING SYSTEM) (REMOVE SIGN TYPE __), and (REMOVE SIGN PANEL TYPE __) at the contract price per EACH shall be compensation in full for all work, material and costs involved in performing the work as specified above and in the Plan.

Payment for Item No. 2104.523 (SALVAGE SIGN TYPE __), (SALVAGE SIGN PANEL TYPE __) and (SALVAGE DELINEATOR) and (SALVAGE MARKER) at the contract price per EACH shall be compensation in full for all work, material and costs involved in performing the work as specified above and in the Plan.

# (2564) TRAFFIC SIGNS AND DEVICES

Traffic signs shall be constructed in accordance with the provisions of MnDOT 2564, except as modified below:

## MATERIALS

Sign Face and Sign Legend Materials

All signs, markers and delineators, including the numerals on M1-5A Route Markers and M1-5B Type Overlays, shall be fabricated with either Sign Sheeting for Rigid Permanent Signs, Delineators and Markers (Type IX or Type XI), except as specified below:

Sign face material for certain warning signs specified in MnDOT 2564.2F3 shall be fluorescent yellow green reflective sheeting.

Sign face material for all other warning signs, yellow markers, yellow delineators, E11-X1 sign panels Type Overlay and the yellow background on Type A and Type OH sign panels shall be fluorescent yellow reflective sheeting, except for the following:

Sign face material for yellow Delineators Type Cylinder Style shall be flexible ASTM Type III MD retroreflective sheeting.

Sign face material for sign panels with brown background shall be white reflective sheeting.  Sign legend material for these sign panels shall be brown electronic cuttable (EC) film in accordance with MnDOT 2564.2F4.  Before applying the brown EC film to the sign face material, the Contractor shall:

Cut and weed out all legend and border (specified on the sign panel detail[s]) in the Plan) from the brown EC film – do NOT cut and weed out the brown EC film that will be covered by Sign Panels Type Overlay (route markers, etc.).

Apply the weeded brown EC film to the sign face material (white reflective sheeting).

Apply the brown EC film “corner” pieces that are located outside the border that fill in the square corners of the sign panel.

Sign face material for the M1-5A Route Marker and the M1-5B Type Overlay shall be as specified in MnDOT 2564.2F3.

Sign face material for X4-2 Hazard Markers shall be non-reflectorized black.

Sign face material for white Delineators Type Cylinder Style shall be ASTM Type V or ASTM Type VII MD reflective sheeting.

Sign legend material for colors other than black may be screened in accordance with

MnDOT 3352.2A5c.

Sign legend material for black legend shall be in accordance with MnDOT 3352.2A5c or MnDOT 3352.2A5d.

Sign legend material for Hazard Marker X4-2 shall be fluorescent yellow reflective sheeting.

The retroreflective sheeting types and qualified products used for rigid permanent signs, markers and delineators can be found at: http://www.dot.state.mn.us/products/index.html.

For signs comprised of extruded panels:

Sign face material shall be direct applied to each individual extruded aluminum panel.  Application procedures shall be performed in accordance with the sign sheeting manufacturer’s recommendations.

All extruded aluminum panels comprising a complete sign panel shall be bolted together prior to application of sign legend material.  Sign legend material shall be applied and cut on all extruded aluminum panel joints in accordance with the sign sheeting manufacturer’s recommendations.

Splicing of reflective sheeting shall not be permitted on signs or panels with dimensions up to and including 48 inches in height or width unless the reflective sheeting specified does not come in this width, then the widest width material shall be used.  All sheeting joints shall be vertical, and the fabricator shall use the least number of seams possible. Roller applied or reverse screened sheeting may be butt-jointed with joint gap not to exceed 1⁄32 inch. Color matching of adjacent sheets of reflective sheeting comprising a sign shall be accomplished without a noticeable difference in color. No borders shall be spliced other than the splice of the tangent border to the corner radius.  EC film shall never be spliced except on multicolored signs, such as those requiring both green and yellow backgrounds on separate parts of the panel.

## CONSTRUCTION REQUIREMENTS

The provisions of MnDOT 2564.3H are hereby modified and supplemented as follows:

Fabrication stickers shall be provided and affixed to the sign panels by the sign fabricator.

The sign fabricator may indicate the month and year of sign panel fabrication by either of the methods described.

The sign fabricator shall provide a full size mockup of the sticker as described.

The Contractor shall install Department furnished warning stickers on new Type A, C and D sign panels in accordance with MnDOT 2564.3H.  The Contractor shall give 30 days advance notice to Jeff Streeter at 651-366-5191 (Metro)-(for other districts, use maintenance area sign supervisor) prior to picking up the Department furnished warning stickers.

For Type A signs and sign panel overlays, the Contractor shall affix a warning sticker to the backside of each Type A sign panel in the lower right corner (when facing the back of the sign panel) in accordance with MnDOT 2564.3H.

Use for projects containing LOGO signs.

The Contractor shall notify Dave DeSutter, General Manager, Minnesota Logos, at 952-895-8079 to have him arrange to install the Logo signs designated in the Plans as “INSTALLED BY OTHERS”.

Use for projects involving roundabouts to field spot accuracy of sign location and orientation.

Signs in and within 50’ of the roundabout will be spotted by MnDOT personnel.  The Contractor shall give 14 Calendar Days advance notice to Josie Tayse (MnDOT) at 651-234-7810 (Metro) prior to installing the signs.

Use for sign replacement projects.

MnDOT 2564.3U – ADD the following at the end of this section:

For all other signs being replaced under this contract, the Contractor shall schedule the work so that a new Type C or Type D sign (that is replacing a sign being removed) is installed the same work day that the in place sign is removed.

Replace Paragraphs 6 and 7 of 2564.3E with the following:

The threads of anchor rods and nuts shall be lubricated with an anti-seize material. All leveling nuts and top nuts shall be brought to a snug-tight condition against the post base plate. Nuts shall be determined to be snug-tight when no shifting of top or bottom washers occurs when they are struck with an inspection hammer. After all nuts are snug-tight, the top nuts shall be additionally tightened 1/12th turn (one-half of a flat). Contractor shall mar anchor rod threads at top of top nuts after tightening is complete, per MnDOT Specification 2402.3H.

For sign panel overlays on Type A, OH, EA, and EO sign panels detailed in the Plan:

The sign face material shall be Sign Sheeting for Rigid Permanent Signs, Delineators

and Markers (Type IX or Type XI), except the sign face material for the yellow background on Type A and Type OH sign panels and E11-X1 sign panels Type Overlay shall be fluorescent yellow reflective sheeting.

The sign legend material shall be Sign Sheeting for Rigid Permanent Signs, Delineators and Markers (Type IX or Type XI), except where black legend is specified the sign legend material shall be in accordance with MnDOT 3352.2A5c or 3352.2A5d.

The Contractor shall furnish and install new post clips and torque in accordance with MnDOT 2564.3L, Paragraph 2.

The Contractor shall remove inplace sign panel overlays.

EA and EO signs shall be attached to Type A and OH signs with new flanged channel posts and new post clips.

For Sign Panel Overlays Type OH indicated in Chart XX of the Plan and Sign Panels Type Overlay to be attached to these panels that are to have graffiti film applied, the Contractor shall:

Apply the graffiti film to each sign panel overlay panel in the shop in accordance with the manufacturer’s specifications; the overlay film cannot be applied to the sign panel overlay panels after they are riveted to the extruded sign panel.  The overlay film also must be applied to the Sign Panels Type Overlay prior to riveting them to the panels

For salvaged Type EA and EO sign panels, the Contractor shall:

Furnish and install extruded panel sections as detailed in the Plan.

Attach to Type A and Type OH signs with new flanged channel posts and new post clips as detailed in the Plan.

For Sign Panel(s) OH XX-XX, the Contractor shall remove extruded panel sections as detailed in the Plan.

For Sign Panel(s) OH XX-XX, the Contractor shall:

Add/remove extruded panel sections as detailed in the Plan.  The panel sections do not require sign face material as they will be overlayed.

Extend /shorten the in place panel mounting posts as detailed in the Plan.

Furnish and install new post clips and torque in accordance with MnDOT 2564.3H.

For Sign(s) A-XX, the Contractor shall attach salvaged sign panel A-YY on the new sign posts with new post clips and torque in accordance with MnDOT 2564.3L, Paragraph 2.

Use when installing a new post and panel on existing concrete footings

For Sign(s) A-XX, the Contractor shall:

Furnish and install new sign posts, complete with new keeper plates, new friction fuses, high strength bolts, heavy hex nuts, and washers to attach new sign posts to the stub posts in the in-place concrete footings in accordance with Sheet No. Y of the Plan.

Use when modifying a sign panel and utilizing existing posts

For Sign(s) A-XX, the Contractor shall:

Remove existing extruded panel sections.

Furnish and install extruded panel sections as specified in the Plan.  New extruded panel sections do not require a sign face applied since they will be overlaid under Sign Panel Overlay Type A.

Install the sign panel, with new post clips, on the existing sign posts.

Use when modifying a sign panel and modifying existing posts

For Sign(s) A-XX, the Contractor shall:

Furnish and install extruded panel sections as specified in the Plan.  New extruded panel sections do not require a sign face applied since they will be overlaid under Sign Panel Overlay Type A.

Install the sign panel, with new post clips, on the modified sign posts as detailed in the plan.

For Sign Panel(s) A-XX, the Contractor shall remove extruded panel sections as detailed in the Plan.

For Sign Panel(s) OH XX-XX, the Contractor shall saw each sign panel as detailed in the Plan.

Use when installing flat sheet aluminum panels on sign supports

For each of the new Type OH sign panels being furnished and installed on Sign Support(s) OH XX-XX, the Contractor shall:

Furnish and attach new sign bracket assembly(ies) to the sign support.

Attach each new panel to the new sign bracket assembly(ies) with new mounting hardware as detailed in the Plan.

For Sign(s) OH XX-XX, the Contractor shall install the salvaged sign on the new concrete footing as detailed in the Plan.

For each Install Sign Panel Type OH in Chart XX of the Plan, the Contractor shall furnish and install new post clips.

For each new single or set of reference post marker(s), the Contractor shall:

Furnish and install a new 3 lb/ft. flanged channel post at the location to be determined in the field by the Engineer and at the offset detailed in the Plan.

Attach the new marker or set of markers to the sign post with stainless steel bolts, nuts and washers and nylon washers at the mounting height detailed in the Plan.  If two markers are mounted back to back, install a nylon washer against the face of each marker to prevent damage to the sheeting materials.

For each new reference post marker attached to a new Type C or Type D sign structure, the Contractor shall attach the marker, at the mounting height designated in the Plan, to the left sign post with new mounting hardware detailed in the Plan.

For each salvaged delineator or marker, the Contractor shall:

Furnish and install a new 3 lb/ft. flanged channel post at the location to be determined in the field by the Engineer and at the offset detailed in the Plan.

Attach the salvaged delineator or marker to the flanged channel post with new mounting hardware at the mounting height detailed in the Plan.

For each new Overhead Sign Identification Plate for post mounted Type OH signs indicated in the Plan, the Contractor shall remove the existing plate (if present) and install the new plate on the sign post in accordance with MnDOT 2564.3P.

Use if no feed point ID plate.

For each new Overhead Sign Identification Plate for bridge mounted Type OH signs indicated in the Plan, the Contractor shall install the plate on a 2 lb./ft. post, in accordance with MnDOT 2564.3P.

Use if there is an existing feed point ID plate.

For each new Overhead Sign Identification Plate for bridge mounted Type OH signs indicated in the Plan, the Contractor shall install the plate on the same 2 lb./ft. post that the feed point identification plate is attached, in accordance with MnDOT 2564.3P.

The Contractor shall install each salvaged Sign Type Special, at the location to be determined in the field by the Engineer, at the same embedment depth as the salvaged sign.

Use if mounting street name sign above Type C sign on U-channel post.

For salvaged street name sign panels, the Contractor shall attach the salvaged bracket assembly (with the street name plates attached), with the salvaged mounting hardware, to the sign post(s) as directed by the Engineer.  The bottom of the bracket assembly shall be mounted a minimum of six inches above the Type C sign panel.

For each Sign Panel Type Special, the Contractor shall furnish and install street name plates, bracket assemblies and mounting hardware, and sign posts as detailed in the Plan.  The location of each sign post will be determined in the field by the Engineer.

## AS-BUILT SIGNING DATA

The Contractor shall submit to MnDOT an electronic data table that contains GPS coordinates and sign attribute data for all new signs and signs that change location.

The Contractor shall contact Rich Frick at 651-775-0316 or richard.frick@state.mn.us 15 working days prior to field collection to obtain tables containing attribute and plan spreadsheet data.  The Contractor shall provide the electronic data table in standard geospatial vector data format.  The format must be accepted by MnDOT prior to commencement of field work.

The GPS receiver and geospatial data collected shall:

Be accurate to less than one meter.

Use a minimum of 5 satellites

Use UTM15N or WGS84 coordinate systems.

One GPS coordinate shall be collected per sign support structure (post, beam, signal mast arm, signal post, guardrail end treatment, wall mount, cantilever, sign bridge, etc.)

If the support consists of a sign post(s), beam(s), or guardrail end treatment, coordinate shall be collected at the center of the support.

If the sign is located on a signal mast arm or signal post, the GPS coordinate shall be the signal post.

If the sign is located on a cantilever or sign bridge, the GPS coordinate shall be the support pole located exterior to the roadway.

The Contractor shall define the following support structure data:

Route – MnDOT route where the sign is located.

Travel Direction – Cardinal direction of the roadway

Type – Support Type

Material – The physical properties/description of which the support is made.

Mounting Kit - Means of attaching or mounting a sign panel or support.

The Contractor shall associate each sign panel to a support and define the following attributes:

Code – Specify MUTCD code or if the panel is a uniquely designed sign, use the panel number as specified on plan sheet (i.e. D-101 or OH100-001).

Size – width x height (inches).

Facing Direction – Direction the sign panel is facing.

Data that does not meet the above criteria may be rejected and the Contractor would be responsible to repeat the data collection.  No direct payment will be made for this work.

## POST AWARD DATA (if requested by the district)

SignCAD Millennium (Program Version 8.24) panel layout files for panel layouts shown in the plan are available electronically upon project award.  To request these files, please contact NAME at email.address@state.mn.us or PHONE NUMBER.

MnDOT believes the electronic data it will provide is accurate, but MnDOT provides no guarantee or warranty, express or implied, concerning the accuracy of the data and the Contractor shall not act in reliance on the data without verifying the data against the contract documents.  The documents originally provided with the Contract remain the basis of the Contract, and the electronic data that will be provided at the Request of the Contractor is provided only for the convenience of the Contractor.  Therefore, if use of this data causes an error, omission, unacceptable work, or work not in conformance with the contract documents, then any costs to the Contractor to make corrections as a result of this error will not be considered "extra work", and the Contractor will not be entitled to an adjustment of contract time.

## METHOD OF MEASUREMENT AND BASIS OF PAYMENT

Payment for Item No. 2564.522 (STRUCTURAL STEEL -- __) at the contract price per POUND shall be compensation in full for all work, materials and costs involved in performing the work as specified above and in the Plan.

Payment for Item No. 2564.531 (SIGN PANELS TYPE _) and 2564.535 (SIGN PANEL OVERLAY TYPE __) at the contract price per SQUARE FOOT shall be compensation in full for all work, materials and costs involved in performing the work as specified above and in the Plan.

Payment for Item No. 2564.536 (INSTALL SIGN PANEL TYPE _), Item No. 2564.537 (INSTALL SIGN TYPE __), and Item No. 2564.539 (OVERHEAD SIGN IDENTIFICATION PLATE) at the contract price per EACH shall be compensation in full for all work, materials and costs involved in performing the work as specified above and in the Plan.

Payment for Item No. 2564.550 DELINEATOR TYPE __), Item No. 2564.551 (REFERENCE POST MARKER), Item No. 2564.552 (HAZARD MARKER X4-2), Item No. 2564.553 (CLEARANCE MARKER X4-4), Item No. 2564.554 (SNOW PLOW MARKER X4-5), Item No. 2564.555 (END OF ROADWAY MARKER X4-11), and Item No. 2564.602 (INFILTRATION AREA MARKER X3-6A) at the contract price per EACH shall be compensation in full for all work, materials and costs involved in performing the work as specified above and in the plan.

Payment for Item No. 2564.602 (INSTALL DELINEATOR, (INSTALL MARKER) and (INSTALL SIGN TYPE SPECIAL) at the contract price per EACH shall be compensation in full for all work, materials and costs involved in performing the work as specified above and in the Plan.

# (3352) SIGNS, DELINEATORS AND MARKERS

## Fabrication of traffic signs, delineators, and markers consisting of sign panels shall conform to the provisions of MnDOT 3352, except as modified below.

## WARRANTY REQUIREMENTS

Ensure the reflective sheeting manufacturer provides the following warranties to the Department:

| Reflective Sheeting Manufacturer Warranties |  |
| --- | --- |
| Material | Warranty |
| Prismatic sign sheeting for permanent signs and white cylinder style delineators (non-fluorescent) | 12 years total. The first 7 years covers 100 percent full replacement of all material and labor costs associated with fabricating and installing the sign or device. The final 5 years covers 100 percent of sheeting replacement. |
| Fluorescent prismatic sign sheeting for permanent signs | 10 years total. The first 7 years covers 100 percent full replacement of all material and labor costs associated with fabricating and installing the sign or device. The final 3 years covers 100 percent of sheeting replacement. |
| Encapsulated lens sign sheeting for permanent signs and yellow cylinder style delineators | 10 years total. The first 7 years covers 100 percent full replacement of all material and labor costs associated with fabricating and installing the sign or device. The final 3 years covers 100 percent of sheeting replacement. |
| Encapsulated lens sign sheeting for traffic cones | Replacement of defective products. |
| Encapsulated lens sign sheeting for tubular markers | Replacement of defective products. |
| Prismatic sign sheeting for rigid Signs (work zone use) | 3 years covers 100 percent sheeting replacement. |
| Prismatic sign sheeting for reboundable plastic drums and weighted channelizers | Replacement of defective products. |
| Prismatic sign sheeting for work zone barricades | Prorated warranty for 3 years covers sheeting replacement. |
| Rollup Sign Sheeting (work zone use) | Prorated warranty for 3 years covers sheeting replacement. |

The Department will require the manufacturer’s warranty to cover the loss of retroreflectivity, loss of colorfastness, cracking, and other conditions inherent to the sheeting, including inks and overlay film that causes ineffectiveness in meeting the intended use.

If the sheeting color does not meet the color specifications of ASTM D 4956-09, Table 11 during the warranty period, the Department will consider the sheeting to have lost colorfastness.

Submit applicable warranties for retroreflective sheeting to the Department.  Ensure the Department is named the obligee on manufacturer warranties.  The Department may remove products from the MnDOT-Approved Product List if the Contractor does not honor the warranties.

Maintain retroreflectivity values during the warranty in accordance with the following:

| Minimum Retroreflectivity Values |  |  |  |
| --- | --- | --- | --- |
| Sign Sheeting Type | Warranty Period, years | Minimum Allowable Retroreflection | Total Daytime Luminance Factor Minimum |
| Type III | 1 – 7 | 80% | — |
| Type III | 8 – 10 | 70% | — |
| Type III MC | None | — | — |
| Type III MT | None | — | — |
| Type VI | 1 – 3 | 50% | 20% |
| Type VIII Cylinder Style Delineators | 1 – 7 | 80% | — |
| Type VIII Cylinder Style Delineators | 8 – 12 | 70% | — |
| Type VIII Fluorescent Orange | None | — | — |
| Type IX | 1 – 7 | 80% | — |
| Type IX | 8 – 12 | 70% | — |
| Type IX Fluorescent Yellow-Green | 1 – 10 | 70% | 60% |
| Type IX Fluorescent Yellow | 1 – 10 | 70% | 40% |
| Type XI FO Fluorescent Orange | 1 – 3 | 70% | 20% |
| Type XI | 1 – 7 | 80% | — |
| Type XI | 8 – 12 | 70% | — |
| Type XI Fluorescent Yellow Green | 1 – 10 | 70% | 60% |
| Type XI FO Fluorescent Yellow | 1 – 10 | 70% | 40% |
| Type XI FO Fluorescent Orange | 1 – 10 | 70% | 20% |
| Type XI B | 1 – 10 | 50% | — |
| Note: Percentages are based on minimum original values meeting the requirements of ASTM D 4956-09 |  |  |  |