DIVISION ST

Section	P

No.	Item

ST-1	(2104) REMOVING MISCELLANEOUS STRUCTURES	1

ST-2	(2564) TRAFFIC SIGNS AND DEVICES	12

ST-3	(3352) SIGNS, DELINEATORS AND MARKERS	20

ST-4	(3402) SQUARE TUBULAR SIGN POSTS	21

DELETE WHEN DONE: To Update Table of Contents Right Click on the index above and choose Update Field.  If given a choice choose “Update Entire Table”.

DELETE WHEN DONE: Statements highlighted in yellow are guidelines or instructional in nature.  Remove these notes before completing the spec.  When appearing at the top left of a provision, it pertains to the paragraph immediately beneath the note, as well as any indented items following it.

DELETE WHEN DONE: Words highlighted in green are fields that may need to be modified or removed before completion of the spec, such as contact information, city names, sign numbers, or charts within the plan.

For provisions containing the descriptor, “For Sign(s) [Type]-XX:”, this descriptor should only be used when the provision applies to specific sign numbers among that type in the plan. Insert as necessary.  Leave out the descriptor if the provision applies to all signs of that type in the plan.

I hereby certify that the Special Provisions for traffic sign construction (Division ST) contained in this proposal were prepared by me or under my direct supervision and that I am a duly licensed Professional Engineer under the laws of the State of Minnesota.

_________________________________________                            Engineer Name

Lic.  No.  XXXXX                    Date MM/DD/YY

age

No.

-ST

-ST

-ST

# DIVISION ST

# (2104) REMOVING MISCELLANEOUS STRUCTURES

## DESCRIPTION

The Contractor shall remove and salvage miscellaneous structures according to 2104, “Removing Pavement and Miscellaneous Structures” and these Special Provisions.

## MATERIALS

The Contractor shall use materials according to 2104, “Removing Miscellaneous Structures” and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions.

## CONSTRUCTION REQUIREMENTS

### Remove By Others

Use for projects that include LOGO signs.

Give at least 14 calendar days advance notice to the General Manager of Minnesota Logos to arrange removal of Logo signs designated in the Plan as “REMOVE BY OTHERS”.

Dave DeSutter

General Manager

Minnesota Logos

952-895-8079

ddesutter@interstatelogos.com

### Remove Grout - Sign Footing

Remove all in place grout between the base plate and the sign footing, taking appropriate measures not to damage the in-place anchor bolts, nuts, washers and conduit.

### Remove Sign Lighting System

Use for removing existing and installing new lighting on cantilever or sign bridge (safety cable and lighting specs are not included in masterspec document)

For Sign(s) OH XX-XX:

Disconnect the power conductors at the source of power (SOP) and the switch.  Tape both ends for reuse in the new sign lighting system.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Disconnect and remove the conductors between each sign lighting fixture, the switch, the sign base, and any fuse connectors in the sign base.

Disconnect and remove all conduit between the sign lighting fixtures, the switch, and the sign post’s conduit stub.

Remove the switch.  If the switch is mounted on the sign post, obtain the Engineer’s approval before capping the opening in the sign post.

Dismantle the sign lighting fixtures and any ballasts from the sign structure.

Remove the feed point identification plate.

Use for bridge mounted sign with conduit and SOP above

For Sign(s) OH XX-XX:

Disconnect the power conductors at the source of power (SOP) and the switch.  Tape both ends for reuse in the new sign lighting system.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Disconnect and remove conduit between the switch and the conduit stub in the bridge.  Cut the power conductors extending out of the conduit stub so that 12 inches of each conductor extends out of the stub.  Tape each power conductor, loop it, and insert back into the conduit stub.  Cap the conduit stub.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and any ballasts from the sign structure.

Remove the feed point identification plate.

Use for bridge mounted sign with junction box as SOP

For Sign(s) OH XX-XX:

Disconnect the power conductors at the source of power (SOP) and the sign base.  Tape both ends for reuse in the new sign lighting system.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Remove the power conductors between the source of power and the switch.

Disconnect and remove conduit between the switch and the conduit stub in the bridge.  Cap the conduit stub.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and ballasts (if encountered) from the sign structure.

Remove the feed point identification plate.

Use if pullbox or streetlight on mainline is the SOP (this is the old bridge mounted method)

For Sign(s) OH XX-XX:

Disconnect the power conductors at the source of power (SOP) and at the switch.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Cut the power cable at the condulet, which is on top of the 2 inch rigid steel conduit and attached to the bridge pier; ensure that 12 inches of cable extends out of the condulet.  Tape the cable end.  Remove the power cable and cable straps between the condulet and the switch.  Plug holes (from removed hangers in concrete) with epoxy grout.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and any ballasts from the sign structure.

Remove the feed point identification plate.

After removing the sign structure from the bridge,

Fill anchorage holes (within the concrete) with epoxy grout.

Plug holes (within the steel) with galvanized high strength steel bolts, washers and nuts.

### Remove Sign Panel Type A

Remove the sign panel and post clips.

### Remove Sign Panel Type D

Remove the sign panel and mounting hardware.

Use when the bridge-mounted sign structure is to be reused

Leave the bridge mounted sign structure in place for the later installation of a new sign panel.

### Remove Sign Panel Type OH

For Sign(s) OH XX-XX:

Remove the left sign panel and panel mounting posts.

Use when existing panels are flat sheet aluminum.

Remove the sign panel and sign bracket assembly(ies) which attach each sign panel to the sign support.

### Remove Sign Type A

Remove the concrete footings or H-pile footings completely.

### Remove Sign Type C and Type D

Use for mast arm-mounted signs

For each mast arm mounted sign, remove the bracketing and hardware which attaches each sign panel to the traffic signal mast arm.

Use for light fixture-mounted signs

For each sign mounted on a light fixture, remove the bracketing and hardware which attaches each sign panel to the light fixture.

Use for bridge rail-mounted signs

For each bridge rail mounted sign, remove the bridge rail mounted sign structure and mounting hardware.  Fill anchorage holes in concrete with epoxy grout.

Use for bridge-mounted signs

For each bridge mounted sign, remove the sign panel, mounting hardware, and bridge mounted sign structure.

After removal of the sign structure,

Fill anchorage holes in concrete with epoxy grout.

Plug holes in steel with galvanized high strength steel bolts, washers, and nuts.

Use for sign replacement projects.

Schedule the work so that replacement signs are installed on the same work day that the in-place signs are removed.

### Remove Sign Type EA and EO

Remove and dispose of the sign panel, post clips, and flanged channel posts.

### Remove Sign Type OH

Disconnect the power conductors at the source of power (SOP) and the sign base.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”  Shut off the circuit breaker(s) in service cabinets.

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Use when the heavy hex nuts are to be salvaged from the pedestal. (Metro)

Salvage as many heavy hex nuts as possible from the pedestal’s anchor rods.

Contact the Department for delivery instructions:

Pat O’Brien / Joe Podobinski

651-772-7537

Remove the concrete footing(s).  Bury the end of the power cable and abandon it in place as directed by the Engineer.

### Remove Sign Type OH (Bridge Mounted)

Disconnect the power conductors at the source of power (SOP) and at the switch.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Do not include if bridge is being removed.

Cut the power cable at the condulet, which is on top of the 2 inch rigid steel conduit and attached to the bridge pier; ensure that 12 inches of cable extends out of the condulet.  Tape the cable end.  Remove the power cable and cable straps between the condulet and the switch.  Plug holes (from removed hangers in concrete) with epoxy grout.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Remove the feed point identification plate.

Do not include if bridge is being removed or re-decked.

After removing the sign structure from the bridge,

Fill anchorage holes (within the concrete) with epoxy grout.

Plug holes (within the steel) with galvanized high strength steel bolts, washers and nuts.

Use for bridge redecking projects when fascia beams will remain inplace.

Ensure that the fascia beam webs are not damaged during the removal of the sign structure(s).

Use for bridge redecking projects when fascia beams will remain inplace and holes will not be reused.

Fill the holes in the fascia beams with epoxy grout.

Use for bridge redecking projects when fascia beams will remain inplace and holes will be reused.

Leave open the holes in the fascia beams; they will be reused to mount new structures.

Use if junction box is the SOP

Disconnect the power conductors at the source of power (SOP) and at the switch.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Do not include if bridge is being removed.

Remove all conduit and power conductors between the source of power and the switch.  Plug any holes in the junction box as directed by the Engineer.  If the conduit is stubbed out of the bridge, cap the conduit end after removal of the power conductors.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Remove the feed point identification plate.

### Remove Sign Type Special

Remove the entire street name sign structure, including any concrete footings.

### Remove Sign Walkway

Use for walkways on cantilevers or sign bridges without lighting systems.

For Sign(s) OH XX-XX:

Remove the walkway and walkway supports.

Repair cut/damaged galvanized surfaces according to ASTM A780.  Apply repair materials according to the manufacturer’s instructions.

After removal of the walkway from the sign truss:

Furnish high strength bolts, nuts, and washers according to 3391.2B, “Fasteners: High Strength Structural Steel Bolts” for each open hole in the sign truss.

Galvanize the high strength bolts, nuts, and washers according to 3392, “Galvanized Hardware.”

Install the high strength bolts, nuts, and washers in the open holes and tighten according to 2402.3G2c, “Steel Bridge Construction: Permanent Connections: Installation.”

Use for walkways for signs on bridges without lighting systems.

For Sign(s) OH XX-XX:

Cut each walkway support so that 2 inches of each support extend beyond the face of the sign panel.

Remove the walkway supports attached to the walkway.

Remove the feed point identification plate.

Repair cut/damaged galvanized surfaces according to ASTM A780.  Apply repair materials according to the manufacturer’s instructions.

Use for walkways with lighting systems on cantilevers or sign bridges.

For Sign(s) OH XX-XX:

Disconnect the power cable at the source of power (SOP) and the sign base.  Tape the ends of the power cable.

Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

If the SOP is a service cabinet,

Disconnect the power conductors from the circuit breaker(s).

Shut off the circuit breaker(s).

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Disconnect and remove the conductors between the sign lighting fixtures and the switch, as well as between the switch and the sign base.  Remove fuse connectors encountered in the sign base.

Disconnect and remove conduit between the switch and the conduit stub on the sign post.  Cap the conduit stub.

If the switch is not attached to the sign post,

Disconnect and remove the conduit between the sign lighting fixtures and the switch.

Remove the switch.

If the switch is attached to the sign post,

Cap the opening in the sign post to the satisfaction of the Engineer.

Dismantle the sign lighting fixtures and any ballasts from the sign structure.

Remove the walkway, walkway supports, and the feed point identification plate.

Repair cut/damaged galvanized surfaces according to ASTM A780.  Apply repair materials according to the manufacturer’s instructions.

After removal of the walkway from the sign truss:

Furnish high strength bolts, nuts, and washers according to 3391.2B, “Fasteners: High Strength Structural Steel Bolts” for each open hole in the sign truss.

Galvanize the high strength bolts, nuts, and washers according to 3392, “Galvanized Hardware.”

Install the high strength bolts, nuts, and washers in the open holes and tighten according to 2402.3G2c, “Steel Bridge Construction: Permanent Connections: Installation.”

### Salvage Delineator and Marker

Inform the Engineer of any damaged in-place delineators or markers prior to salvaging.

Remove and dispose of the sign structure, nuts, bolts and washers.

If the Contractor damages a delineator or marker,

Dispose of the damaged delineator or marker.

Fabricate a new delineator or marker according to 2564.2F, “Traffic Signs and Devices: Signs and Markers,” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the aluminum sign panels and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged delineators will be reinstalled under Item No. 2564.602 - Install Delineator.

Salvaged markers will be reinstalled under Item No. 2564.602 - Install Marker.

### Salvage Sign Panel Type A

Inform the Engineer of any damaged in-place Type A or Type EA sign panels prior to salvaging.

Salvage the in-place sign panel and remove the post clips.

Leave the Type EA sign panels attached to the Type A sign panel while salvaging; both signs shall be reinstalled together as one unit under Item No. 2564.536 - Install Sign Panel Type A.

### Salvage Sign Panel Type C

Inform the Engineer of any damaged in-place Type C sign panels prior to salvaging.

Remove and dispose of the nuts, bolts and washers.

If the Contractor damages a sign panel,

Dispose of the damaged sign panel.

Fabricate a new sign panel according to 2564.2F, “Traffic Signs and Devices: Signs and Markers,” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the aluminum sign panels and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged sign panels will be reinstalled under Item No. 2564.536 - Install Sign Panel Type C.

### Salvage Sign Panel Type OH

Inform the Engineer of any damaged in-place Type OH or EO panels prior to salvaging.

Salvage the in place sign panel and remove the post clips.

Leave the Type EO sign panels attached to the Type OH sign panel while salvaging; both signs shall be reinstalled together as one unit under Item No. 2564.536 - Install Sign Panel Type OH.

Prevent damage to the aluminum sign panels and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

### Salvage Sign Panel Type Special

Use if mounting above Type C sign on U-channel post (not permitted in MetroDistrict)

Inform the Engineer of any damaged in-place sign panels prior to salvaging.

(note if there are two plates mounted back to back)

Salvage the street name plates and bracket assembly.

Leave the sign panels attached to the bracket assembly.

Leave the mounting hardware attached to the flanged channel sign post(s).

If the Contractor damages a bracket assembly or street name plates,

Dispose of the damaged bracket assembly or damaged street name plate.

Fabricate new bracket assembly or street name plate according to City of __,__County specifications, 2564.2F, “Traffic Signs and Devices: Signs and Markers;” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the bracket assemblies, aluminum sign panels, and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged sign panels, bracket assemblies, and mounting hardware will be reinstalled under Item No. 2564.536 - Install Sign Panel Type Special.

### Salvage Sign Type A

Inform the Engineer of any damaged in-place Type A sign panels or posts prior to salvaging.

Use this provision if salvaging and later installing upon new footings.

For Sign(s) A-XX, the Contractor shall:

Salvage the in place sign panel and sign posts.

Remove the post clips.

Remove the friction fuse and mounting hardware from each sign post.

Remove the concrete footings.

The sign panel and sign posts will be reinstalled under Item No. 2564.537 - Install Sign Type A.

Use this provision if salvaging and later installing upon the same footings.

For Sign(s) A-XX, the Contractor shall:

Unbolt the sign post from each base plate and remove the base plate bolts, nuts, and washers.

Remove the friction fuse and mounting hardware from each sign post.

Remove the lower sign post section (from the base plate to the post cut) of each sign post.

Remove the post clips and salvage the sign panel as well as the sign post sections behind the sign panel (from the post cut to the top of the sign panel).

The sign panel and sign post sections will be reinstalled under Item No. 2564.537 - Install Sign Type A.

### Salvage Sign Type C and Type D

Inform the Engineer of any damaged in-place Type C or Type D sign panels prior to salvaging.

Remove and dispose of the sign structure, nuts, bolts and washers.

If the Contractor damages a sign panel,

Dispose of the damaged sign panel.

Fabricate a new sign panel according to 2564.2F, “Traffic Signs and Devices: Signs and Markers,” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the aluminum sign panels and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged Type C sign panels will be reinstalled under Item No. 2564.537 - Install Sign Type C.

Salvaged Type D sign panels will be reinstalled under Item No. 2564.537 - Install Sign Type D.

### Salvage Sign Type EA and EO

Use this provision only when modifying the panel height or repositioning the EA/EO sign on the A/OH panel.

Inform the Engineer of any damaged in-place EA/EO sign panels prior to salvaging.

Remove and dispose of the post clips and flanged channel posts.

Prevent damage to the bracket assemblies, aluminum sign panels, and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged Type EA signs will be reinstalled under Item No. 2564.537 - Install Sign Type EA.

Salvaged Type EO signs will be reinstalled under Item No. 2564.537 - Install Sign Type EO.

### Salvage Sign Type OH

If the OH sign has lighting to be removed as part of the salvage, insert the appropriate language from the 2104: Construction Requirements: Remove Sign Lighting System section of this document.

If the OH sign has walkway to be removed as part of the salvage, insert the appropriate language from the 2104: Construction Requirements: Remove Sign Walkway section of this document.

Pull the power cable out of the conduit in the footing prior to removal of the concrete footing.  Tape the ends of the conductors.

Salvage the sign truss (including sign panels and panel mounting posts) and sign post from the concrete footing and store outside the clear zone on the job site as directed by the Engineer.  The sign truss and sign post shall each be placed on sufficient cribbing (railroad ties, etc.) to keep them straight and clear of the ground.  The sign truss and post will be installed under Install Sign Type OH.

Remove the concrete footing.  Bury the end of the power cable and abandon in place as directed by the Engineer.

Use for bridge mounted signs.

Salvage the bridge mounted sign structures and sign panels.

Remove bolts, nuts, and the washers connecting to girder webs.  Avoid damaging the in-place girder webs.

### Salvage Sign Type Special

Inform the Engineer of any damaged in-place sign prior to salvaging.

Salvage the sign post with the bracket assembly and street name plates attached.

If the Contractor damages a sign post, bracket assembly, or street name plate,

Dispose of the damaged sign post, bracket assembly, or street name plate.

Fabricate a new sign post, bracket assembly, or street name plate according to City of __,__County specifications, 2564.2F, “Traffic Signs and Devices: Signs and Markers;” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the sign posts, bracket assemblies, aluminum sign panels, and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged signs will be reinstalled under Item No. 2564.537 - Install Sign Type Special.

### Disposal of Lighting Devices

Deliver all mercury vapor lamps and ballasts that are NOT marked “NON-PCB” to one of the following companies:

Veolia ES Technical Solutions, L.L.C

2960 Yorkton Boulevard

Little Canada, MN 55117

952-736-3335

Green Lights Recycling, Inc

10040 Davenport Street NE

Blaine, MN 55449-4423

763-785-0456

http://glrnow.com/contact-us/

Provide written documentation to the Engineer that lamps and PCB ballasts were delivered to a listed disposal company.

The Contractor shall pay any costs incurred by the disposal company for proper disposal of lamps and PCB ballasts.

Dispose of lighting fixture housings and all other components according to 2104, “Removing Pavement and Miscellaneous Structures.”

## METHOD OF MEASUREMENT & BASIS OF PAYMENT

The Engineer will measure each item according to the Contract and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions.

The Department will include all work described in the Contract and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions as part of the contract unit price per unit of measure.

The Department will pay for traffic signs and devices on the basis of the following schedule:

| Item No.: | Item: | Unit: |
| --- | --- | --- |
| 2104.501 | Remove Sign Walkway | Linear foot |
| 2104.509 | Remove Grout – Sign Footing | Each |
| 2104.509 | Remove Sign Lighting System | Each |
| 2104.509 | Remove Sign Panel Type _ | Each |
| 2104.509 | Remove Sign Type _ | Each |
| 2104.523 | Salvage Sign Panel Type _ | Each |
| 2104.523 | Salvage Sign Type _ | Each |
| 2104.523 | Salvage Delineator | Each |
| 2104.523 | Salvage Marker | Each |

The Department’s payment for each item shall be compensation in full for all work, material, and costs involved in performing the work specified in the Plan and these Special Provisions.

# (2564) TRAFFIC SIGNS AND DEVICES

## DESCRIPTION

The Contractor shall furnish and install traffic signs according to 2564, “Traffic Signs and Devices,” except as modified in these Special Provisions.

## MATERIALS

Fabricate all standard signs, markers, and delineators according to the standard sign drawings in the MnDOT Standard Signs Manual.

Fabricate all signs, markers, and delineators with material according to 3352, “Signs, Delineators, and Markers.”

Fabricate all rigid permanent signs, markers, and delineators with materials from the MnDOT Approved/Qualified Products List.

Fabricate all permanent signs, markers, and delineators with Sign Sheeting Type IX or Sign Sheeting Type XI, except if otherwise indicated in 2564, “Traffic Signs and Devices” or in the 2564, “Traffic Signs and Devices: Materials” section of these Special Provisions.

Provide Type V or Type VIII retroreflective sheeting material for white X4-13 Cylinder Style Delineators.

Provide Type III MT or Type V retroreflective sheeting material for yellow X4-13 Cylinder Style Delineators.

Provide all warning signs, yellow markers, yellow delineators, E11-X1 sign panels Type Overlay, and the yellow background on Type A and Type OH sign panels with fluorescent yellow reflective sheeting, except if indicated differently below:

Replace 2564.2F, “Traffic Signs and Devices: Signs and Markers” specifications regarding the fabrication of standard signs and plaques with fluorescent yellow-green sheeting, with the following language:

For W11-1, W11-2, W11-9, W11-15 series, W15-1, S1-1, S3-1, S3-2a, and S4-5 standard signs, provide reflective sheeting sign face material meeting the requirements of 3352.2.A.2.e, “Sign Sheeting Type IX,” for Type IX FL fluorescent yellow-green, or 3352.2.A.2.f, “Sign Sheeting Type XI,” for Type XI FL fluorescent yellow-green.

When warning sign plaques are mounted on the same structure as a warning sign, match the plaque color and sheeting type with the primary warning sign color and sheeting type.

When a S4-3P sign plaque is to be installed above a regulatory speed limit sign, fabricate the sign plaque with fluorescent yellow-green reflective sheeting.

Fabricate the R1-6 series, the R1-9, and the S5-1 standard signs with fluorescent yellow-green reflective sheeting on only the warning color parts of the sign, according to the design in the MnDOT Standard Signs Manual.

Sign face material for X4-2 Hazard Markers shall be non-reflectorized black.   Sign legend material for Hazard Marker X4-2 shall be fluorescent yellow reflective sheeting.

Provide black sign legend material according to 3352.2.A.5.c, “Signs, Delineators, and Markers: Requirements: Materials: Screen Processed Painted Legend” or 3352.2.A.5.d, “Signs, Delineators, and Markers: Requirements: Materials: Pigmented Plastic Film Legend.”

## CONSTRUCTION REQUIREMENTS

### As-Built Signing Data

Use this provision to collect sign location data for inventory management purposes.

Submit to the Department an electronic data table containing GPS coordinates and sign attribute data for all new signs, as well as signs that change location.

Contact the Department 15 working days prior to field collection to obtain tables containing the attribute and plan spreadsheet data to be collected.

Contact Name

Title

Phone Number

Email@state.mn.us

Provide the electronic data table in standard geospatial vector data format.  The format must be accepted by MnDOT prior to commencement of field work.

Collect the data in the 1996 adjustment to the UTM15N North American Datum (NAD83), unless otherwise specified.

Ensure that the GPS receiver uses a minimum of 5 satellites during data collection.

Ensure that the geospatial data collected is accurate to less than 1 meter.

Collect 1 GPS coordinate per sign support structure.

Sign support structures include but are not limited to the following types: flanged channel sign post, square tube sign post, I-beam, signal mast arm, signal post, guardrail end treatment, wall mounting, cantilever, sign bridge.

If the support consists of a guardrail end treatment or multiple sign posts or I-beams,

Collect the GPS coordinate at the center of the support.

If the sign is located on a signal mast arm or signal post,

Collect the GPS coordinate at the signal post.

If the sign is located on a cantilever or sign bridge,

Collect the GPS coordinate at the support pole located exterior to the roadway.

Use the following support structure data definitions:

Route – MnDOT route where the sign is located.

Travel Direction – Cardinal direction of the roadway

Type – Support Type

Material – The physical properties/description of which the support is made.

Mounting Kit - Means of attaching or mounting a sign panel or support.

Assign each sign panel to a support structure and use the following attribute definitions:

Code – Specify the MN MUTCD code of standard signs.  If the panel is a uniquely designed sign, use the panel number as specified on plan sheet (i.e. D-101 or OH100-001).

Size – width x height for rectangular and octagonal panels, diameter for circular panels, length of a side for triangular panels, and length of the bottom side for pentagonal panels (inches).

Facing Direction – Direction the sign panel is facing.

Sheeting Material – Manufacturer’s brand name (DG3, OmniCube)

If the submitted data does not meet the criteria of this specification,

The Department may reject the data and require the Contractor to repeat the data collection.  No direct payment will be made for this work.

### Fabrication & Warning Stickers

Use this provision if new sign panels are being installed (almost all projects)

The provisions of 2564.3H, “Traffic Signs and Devices: Construction Requirements: Sign Panels” are hereby modified as follows:

The Sign Fabricator shall provide and affix fabrication stickers to all furnished sign panels.

Install Department-provided warning stickers on new Type A, C, and D sign panels according to 2564.3H, “Traffic Signs and Devices: Construction Requirements: Sign Panels.”

Give 30 days advance notice to the Department prior to picking up the Department-provided warning stickers:

Jeff Streeter	(Metro)-(for other districts, use maintenance area sign supervisor)

651-366-5191

If the project involves Type A signs:

For Type A signs and sign panel overlays on Type A signs,

Affix the warning sticker to each Type A sign panel in the lower right corner of the back of the sign panel, directly above the fabrication sticker.

### Feed Point Identification Plate

Use this provision when installing new, lighted overhead signs

For each new lighted overhead sign being installed,

Provide a Feed Point Identification Plate according to 2545.3.J.4, “Electrical Lighting Systems: Construction Requirements: Feed Point Identification Plate.”

Fabricate the Feed Point Identification Plate with plate material according to 3352.2.A.1.a, “Signs, Delineators, and Markers: Requirements: Sheet Aluminum.”

Provide black sign legend material over a yellow retroreflective sheeting sign face material.

### Field Spotting of Signs

Use this provision when installing signs where location and orientation is critical (i.e. roundabouts, RCUTs, DDI)

Give the Engineer 14 calendar days advance notice prior to installing signs inside or within 50 feet of roundabouts.

The Engineer will contact the District Traffic Office, which will provide personnel to field spot the installation location and orientation of the signs:

Contact Name

Title

Phone Number

Email@state.mn.us

### Install Delineator and Marker

Use this provision when installing salvaged delineators and/or markers

For each salvaged delineator or marker being installed,

Furnish and install a new 3 lb/ft. flanged channel post at the location determined in the field by the Engineer and at the offset detailed in the Plan.

Attach the salvaged delineator or marker to the flanged channel post with new mounting hardware at the mounting height detailed in the Plan.

Install delineators under Item No. 2564.602 - Install Delineator.

Install markers under Item No. 2564.602 - Install Marker.

### Install Sign Panel Type A

For each Item No. 2564.536 – Install Sign Panel Type A in the Plan,

Furnish and install new post clips.  Torque the post clips according to Item (3) in 2564.3L, “Traffic Signs and Devices: Construction Requirements: Install Sign Panel Type.”

Use when installing a salvaged sign panel on new posts

For Sign Panel(s) A-XX,

Attach salvaged Sign Panel A-YY on the new sign posts designated in the Plan.

Use this provision when modifying a sign panel and reinstalling it upon the existing posts

For Sign Panel(s) A-XX,

Furnish and install OR Remove extruded panel sections to obtain the sign panel height specified in the Plan.

Leave the new extruded panel sections without sign face material; they will be overlaid under Item No. 2564.535 - Sign Panel Overlay Type A.

Install the sign panel upon the existing sign posts.

Use this provision when modifying a sign panel and modifying existing posts

For Sign Panel(s) A-XX,

Furnish and install OR Remove extruded panel sections to obtain the sign panel height as specified in the Plan.

Leave the new extruded panel sections without sign face material; they will be overlaid under Item No. 2564.535 - Sign Panel Overlay Type A.

Install the sign panel upon the modified sign posts.

### Install Sign Panel Type OH

For each Item No. 2564.536 – Install Sign Panel Type OH in the Plan,

Furnish and install new post clips.  Torque the post clips according to Item (3) in 2564.3L, “Traffic Signs and Devices: Construction Requirements: Install Sign Panel Type.”

Use this provision when modifying an overhead sign panel

For Sign Panel(s) OH XX-XX,

Furnish and install OR Remove extruded panel sections to obtain the sign panel height as specified in the Plan.

Use if furnishing and installing new sections

Leave the new extruded panel sections without sign face material; they will be overlaid under Item No. 2564.535 – Sign Panel Overlay Type OH.

Modify the in-place panel mounting posts as detailed in the Plan.

### Install Sign Panel Type Special

Use this provision when mounting salvaged street name signs above Type C signs on U-channel posts.

Attach the salvaged street name plate, attached bracket assembly, and salvaged mounting hardware to the sign post as directed by the Engineer.

Mount the bottom of the bracket assembly a minimum of six inches above the Type C sign panel.

### Install Sign Type A

Use this provision when installing a new panel and post on existing concrete footings

For Sign(s) A-XX,

Attach new sign posts to the stub posts on the in-place concrete footings in accordance with Sheet No. Y of the Plan.

### Install Sign Type C and D

Use this provision when installing Type C or Type D signs in areas of high winds such as Districts 1, 2, 4, 7, and 8

Install signs with 3/8” stainless steel bolts and zinc-plated nylon insert lock nuts on the L-Bracket assembly when a knee brace is used.

### Install Sign Type EA and EO

Use this provision when converting 24” EA/EO panels to 30”

For each salvaged Sign Type EA and Sign Type EO being installed,

Furnish and install new extruded panel sections as detailed in the Plan; they will be furnished and installed under Item No. 2564.531 – Sign Panels Type EA and Item No. 2564.531 – Sign Panels Type EO.

Install by attaching Type EA panels to Type A sign panels and Type EO panels to Type OH sign panels with new flanged channel posts and new post clips.  Install the panels under Item No. 2564.537– Install Sign Type EA and Item No. 2564.537 – Install Sign Type EO.

Torque the post clips according to Item (3) in 2564.3L, “Traffic Signs and Devices: Construction Requirements: Install Sign Panel Type.”

Overlay the Type EA and Type EO panels under Item No. 2564.535 – Sign Panel Overlay Type EA and Item No. 2564.535 – Sign Panel Overlay Type EO.

### Install Sign Type OH

Use this provision when installing a salvaged OH sign structure on a new footing

For Sign(s) OH XX-XX,

Install the salvaged sign on the new concrete footing detailed in the Plan.

### Install Sign Type Special

Install each salvaged Sign Type Special:

At the location to be determined in the field by the Engineer

At the same embedment depth as the salvaged sign.

### Overhead Sign Identification Plate

For each new Overhead Sign Identification Plate (X5-1) being installed,

Remove the existing plate (if present) and install the new plate on the sign post according to 2564.3P, “Traffic Signs and Devices: Construction Requirements: Overhead Sign Identification Plate.”

### Post-Award Data

Use this provision if requested by the District Traffic Office

SignCAD Millennium (Program Version 8.24) panel layout files for panel layouts shown in the plan are available electronically upon project award.  To request these files, please contact:

Contact Name

Title

Phone Number

Email@state.mn.us

MnDOT believes the electronic data it will provide is accurate, but MnDOT provides no guarantee or warranty, express or implied, concerning the accuracy of the data and the Contractor shall not act in reliance on the data without verifying the data against the contract documents.  The documents originally provided with the Contract remain the basis of the Contract, and the electronic data that will be provided at the Request of the Contractor is provided only for the convenience of the Contractor.  Therefore, if use of this data causes an error, omission, unacceptable work, or work not in conformance with the contract documents, then any costs to the Contractor to make corrections as a result of this error will not be considered "extra work", and the Contractor will not be entitled to an adjustment of contract time.

### Reference Location Signs

For each new D10-series Reference Location Sign,

If the Reference Location Sign is to be attached to a new Type C or Type D sign structure:

Attach the Reference Location Sign to the left sign post with new stainless steel bolts, nuts, washers, and nylon washers, at the mounting height designated in the Plan.

If the Reference Location Sign is to be installed on its own post:

Furnish and install a new 3 lb/ft. flanged channel post at the location determined in the field by the Engineer and at the offset detailed in the Plan.

Attach new Reference Location Signs to the sign post with new stainless steel bolts, nuts, washers, and nylon washers, at the mounting height detailed in the Plan.

If two Reference Location Signs are mounted back-to-back:

Install a nylon washer against the face of each Reference Location Sign to prevent damage to the sheeting materials.

### Safety Cable

Use this provision to add cable to overhead structures with walkway

Furnish and install brackets, aircraft cable, and all necessary hardware required to assemble and attach a safety cable, according to 2564.3.W, “Traffic Signs and Devices: Construction Requirements: Safety Cable.”

### Sign Panel Overlay Type _

Use this provision to remove existing sign panel overlays from the extruded panels

In addition to the work described in 2564.3K, “Traffic Signs and Devices: Construction Requirements: Sign Panel Overlay Type_,”

Remove in-place sign panel overlays before attaching the new overlays to the panel.

Use this provision to add graffiti film to sign panel overlay Type OH

For Sign Panel Overlays Type OH indicated in Chart XX of the Plan and for Sign Panels Type Overlay to be attached to these panels,

Apply the graffiti film to the panels in the shop and according to the manufacturer’s specifications.

Apply graffiti film to the entire Sign Panel Overlay Type OH panel.

Apply graffiti film to the Sign Panels Type Overlay.

Rivet the Sign Panels Type Overlay to the Sign Panel Overlay Type OH panel according to 2564.3H, “Traffic Signs and Devices: Construction Requirements: Sign Panels”, paragraph 1.

### Sign Panel Type C and D

Use this provision when installing Type C or D sign panels in areas of high winds such as Districts 1, 2, 4, 7, and 8

Install signs with 3/8” stainless steel bolts and zinc-plated nylon insert lock nuts on the L-Bracket assembly when a knee brace is used.

### Sign Panels Type OH

Use this provision  when installing flat sheet aluminum panels on OH sign supports

For Sign Panel(s) OH XX-XX,

Furnish and attach new sign bracket assemblies to the sign support.

Attach each new panel to the new sign bracket assemblies with new mounting hardware as detailed in the Plan.

### Sign Panels Type Overlay

Use this provision when including an organization’s official logo/pictograph and attaching them to the panel with rivets .

(pictograph = government agency; logo = private organization)

For Sign(s)/Sign Panel(s) A/D/OH-XX,

Use the official name of logo here for the logo/pictograph design on the sign panel.

If design requirements are publicly available:

Access the logo/pictograph's graphic design standards on the following website:

Insert website here

If design requirements are NOT publicly available:

Contact Company Name to obtain the logo/pictograph’s graphic design standards:

Contact Name

Company Name

Telephone and/or email

Fabricate the sign panel(s) with the logo/pictograph at the size and location shown on Sheet No(s). XX of the Plan.

Attach the logo/pictograph to the sign panel according to 2564.3H, “Traffic Signs and Devices: Construction Requirements: Sign Panels”, paragraph 1.

### Sign Panels Type Special

Use this provision if mounting new street name sign above Type C signs on U-channel posts.

Furnish and install street name plates, bracket assemblies, mounting hardware, and sign posts as detailed in the Plan.

The Engineer will determine the location of each sign post in the field.

### Sign Replacement Projects

Use this provision on sign replacement projects

The provisions of 2564.3U, “Traffic Signs and Devices: Construction Requirements: Scheduling of Work” are hereby supplemented as follows:

For signs not detailed in 2564.3U, “Traffic Signs and Devices: Construction Requirements: Scheduling of Work”:

Schedule the work so that replacement Type C and Type D signs are installed the same work day that the in-place signs are removed.

### Structural Steel

Use this provision when installing OH sign structures

The provisions of 2564.3E, “Traffic Signs and Devices: Construction Requirements: Structural Steel” are hereby supplemented as follows:

Lubricate the threads of anchor rods and nuts with an anti-seize material.

Tighten all leveling nuts and top nuts against the post base plate so that no shifting of top or bottom washers occurs when they are struck with an inspection hammer. After this tightness has been achieved, additionally tighten the top nuts another 1/12th turn (one-half of a flat).

Mar the anchor rod threads directly above the top nuts after tightening is completed.

## METHOD OF MEASUREMENT AND BASIS OF PAYMENT

The Engineer will measure each item according to the Contract and the 2564, “Traffic Signs and Devices: Construction Requirements” section of these Special Provisions.

The Department will include all work described in the Contract and the 2564, “Traffic Signs and Devices: Construction Requirements” section of these Special Provisions as part of the contract unit price per unit of measure.

The Department will pay for traffic signs and devices on the basis of the following schedule:

| Item No.: | Item: | Unit: |
| --- | --- | --- |
| 2013.603 | Safety Cable | Linear foot |
| 2564.522 | Structural Steel - __ | Pound |
| 2564.531 | Sign Panels Type _ | Square foot |
| 2564.535 | Sign Panel Overlay Type _ | Square foot |
| 2564.536 | Install Sign Panel Type _ | Each |
| 2564.537 | Install Sign Type _ | Each |
| 2564.539 | Overhead Sign Identification Plate | Each |
| 2564.550 | Delineator Type _ | Each |
| 2564.551 | Reference Post Marker | Each |
| 2564.552 | Hazard Marker X4-2 | Each |
| 2564.553 | Clearance Marker X4-4 | Each |
| 2564.554 | Snow Plow Marker X4-5 | Each |
| 2564.555 | End of Roadway Marker X4-11 | Each |
| 2564.602 | Infiltration Area Marker X3-6A | Each |
| 2564.602 | Install Delineator | Each |
| 2564.602 | Install Marker | Each |
| 2564.602 | Install Sign Type Special | Each |

The Department’s payment for each item shall be compensation in full for all work, material, and costs involved in performing the work specified in the Plan and these Special Provisions.

# (3352) SIGNS, DELINEATORS AND MARKERS

## SCOPE

The Contractor shall fabricate traffic signs, delineators, and markers consisting of sign panels 3352, “Signs, Delineators and Markers” and these Special Provisions.

## REQUIREMENTS

The Contractor shall use materials according to 3352, “Signs, Delineators and Markers” and these Special Provisions.

Splice retroreflective sheeting according to the following:

Do not splice retroreflective sheeting on panels with dimensions smaller than 48 inches in height and 48 inches in width.

If the retroreflective sheeting material is not available in this width, use the widest width material available for that type.

Splice the retroreflective sheeting so that all sheeting joints are vertical.

Use the least number of seams possible.

Butt-joint roller-applied and reverse-screened sheeting types so that joint gaps do not exceed 1/32 inch.

Match colors of adjacent sheets of retroreflective sheeting on sign panels so that there is no noticeable difference in color.

Do not splice vinyl cuttable film except on signs where the background changes color.

## SAMPLING AND TESTING — (BLANK)

# (3402) SQUARE TUBULAR SIGN POSTS

The provisions of MnDOT 3402 are as follows:

## SCOPE

Provide square steel tubular posts for signs.

## REQUIREMENTS

A	Material

Use steel posts meeting standard specification for hot rolled carbon sheet steel, structural quality, ASTM A 1011, Grade 50.  The average minimum yield strength after cold forming shall be a minimum of 50,000 psi.

B	Length

Use posts with lengths as shown on the plans or in the special provisions.  The Engineer will not allow the use of posts with lengths greater than ¼ in under the length shown on the plans or in the special provisions.

C	Weight

Use posts in accordance with the following, as shown on the plans:

| Size | U.S.S. Gauge | Weight (lbs/ft) |
| --- | --- | --- |
| 1 ½” x 1 ½” | 12 | 1.7 |
| 1 ¾” x 1 ¾” | 12 | 2.06 |
| 2” x 2” | 12 | 2.42 |
| 2 ¼” x 2 ¼” | 12 | 2.77 |
| 2 ½” x 2 ½” | 12 | 3.14 |
| 2 3⁄16” x 2 3⁄16” | 10 | 3.43 |
| 2 ½” x 2 ½” | 10 | 4.01 |

The Engineer will allow the use of posts weighing up to 5 percent under the weight shown on the plans or the special provisions.

D	Cross Section

Use square tube formed of 12 gauge or 10 gauge steel as shown in plans or special provisions, carefully rolled to size and welded directly in the corner by high frequency resistance welding and externally scarfed to agree with corner radii.

E	Hole Punching

Use posts with punched holes 7/16± 1/64 inches in diameter on 1 inch centers on all fours sides down the entire length of the post.  Holes shall be on centerline of each side and in true alignment and opposite of each other directly and diagonally.

F	Coating

Use posts hot dip galvanized steel in accordance with ASTM A 653, G90.  The corner weld shall be zinc coated after scarfing operation. The steel shall also be coated with a chromate conversion coating and a clear organic polymer topcoat. Both the interior and the exterior of the post shall be galvanized.  Modifications made to the post after the initial fabrication, such as additional welding or other alterations shall be galvanized.

G	Telescoping Properties

Use straight posts, with a smooth uniform finish free of bow, twist and other unsightly defects.  All holes and ends shall be free from burrs and ends shall be cut square.  Telescoping systems shall meet dimensional requirements and shall permit consecutive sizes of square tubes to telescope freely without necessity of matching any particular face to any other face.

H	Anchors

Use galvanized square tube anchors as shown in plans or special provisions.

## SAMPLING AND TESTING

Obtain a certified mill analysis from the supplier that states the chemical composition of each lot or heat of posts delivered.

The Engineer may take samples for testing from any of the provided posts.

During the inspection of any lot of posts, if the Engineer rejects more than 20 percent of the posts in the lot, the Engineer may reject the entire lot.