DIVISION ST

Section	P

No.	Item

ST-1	(2104) REMOVING MISCELLANEOUS STRUCTURES	1

ST-2	(2564) TRAFFIC SIGNS AND DEVICES	12

ST-3	(3352) SIGNS, DELINEATORS AND MARKERS	20

DELETE WHEN DONE: To Update Table of Contents Right Click on the index above and choose Update Field.  If given a choice choose “Update Entire Table”.

DELETE WHEN DONE: Statements highlighted in yellow are guidelines or instructional in nature.  Remove these notes before completing the spec.  When appearing at the top left of a provision, it pertains to the paragraph immediately beneath the note, as well as any indented items following it.

DELETE WHEN DONE: Words highlighted in green are fields that may need to be modified or removed before completion of the spec, such as contact information, city names, sign numbers, or charts on the Plans.

For provisions containing the descriptor, “For Sign(s) [Type]-XX:”, this descriptor should only be used when the provision applies to specific sign numbers among that type on the Plans.  Insert as necessary.  Leave out the descriptor if the provision applies to all signs of that type on the Plans.

I hereby certify that the Special Provisions for traffic sign construction (Division ST) contained in this proposal were prepared by me or under my direct supervision and that I am a duly licensed Professional Engineer under the laws of the State of Minnesota.

_________________________________________                            Engineer Name

Lic.  No.  XXXXX                    Date MM/DD/YY

age

No.

-ST

-ST

-ST

# DIVISION ST

# (2104) REMOVING MISCELLANEOUS STRUCTURES

## DESCRIPTION

The Contractor shall remove and salvage miscellaneous structures according to 2104, “Removing Pavement and Miscellaneous Structures” and these Special Provisions.

## MATERIALS

The Contractor shall use materials according to 2104, “Removing Miscellaneous Structures” and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions.

## CONSTRUCTION REQUIREMENTS

### Remove By Others

Use for projects that include LOGO signs.

Give at least 14 calendar days advance notice to the General Manager of Minnesota Logos to arrange removal of Logo signs designated on the Plans as “REMOVE BY OTHERS”.

Dave DeSutter

General Manager

Minnesota Logos

952-895-8079

ddesutter@interstatelogos.com

### Remove Sign Panel Type D

Remove the sign panel and mounting hardware.

Use when the bridge-mounted sign structure is to be reused

Leave the bridge mounted sign structure in place for the later installation of a new sign panel.

### Remove Sign Type C and Type D

Use for mast arm-mounted signs

For each mast arm mounted sign, remove the bracketing and hardware which attaches each sign panel to the traffic signal mast arm.

Use for light fixture-mounted signs

For each sign mounted on a light fixture, remove the bracketing and hardware which attaches each sign panel to the light fixture.

Use for bridge rail-mounted signs

For each bridge rail mounted sign, remove the bridge rail mounted sign structure and mounting hardware.  Fill anchorage holes in concrete with epoxy grout.

Use for bridge-mounted signs

For each bridge mounted sign, remove the sign panel, mounting hardware, and bridge mounted sign structure.

After removal of the sign structure,

Fill anchorage holes in concrete with epoxy grout.

Plug holes in steel with galvanized high strength steel bolts, washers, and nuts.

Use for sign replacement projects.

Schedule the work so that replacement signs are installed on the same work day that the in-place signs are removed.

### Remove Sign Type Special

Remove the entire street name sign structure, including any concrete footings.

### Salvage Delineator and Marker

Inform the Engineer of any damaged in-place delineators or markers prior to salvaging.

Remove and dispose of the sign structure, nuts, bolts and washers.

If the Contractor damages a delineator or marker,

Dispose of the damaged delineator or marker.

Fabricate a new delineator or marker according to 2564.2F, “Traffic Signs and Devices: Signs and Markers,” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the aluminum sign panels and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged delineators will be reinstalled under Item No. 2564.602 - Install Delineator.

Salvaged markers will be reinstalled under Item No. 2564.602 - Install Marker.

### Salvage Sign Panel Type C

Inform the Engineer of any damaged in-place Type C sign panels prior to salvaging.

Remove and dispose of the nuts, bolts and washers.

If the Contractor damages a sign panel,

Dispose of the damaged sign panel.

Fabricate a new sign panel according to 2564.2F, “Traffic Signs and Devices: Signs and Markers,” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the aluminum sign panels and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged sign panels will be reinstalled under Item No. 2564.536 - Install Sign Panel Type C.

### Salvage Sign Panel Type Special

Use if mounting above Type C sign on U-channel post (not permitted in MetroDistrict)

Inform the Engineer of any damaged in-place sign panels prior to salvaging.

(note if there are two plates mounted back to back)

Salvage the street name plates and bracket assembly.

Leave the sign panels attached to the bracket assembly.

Leave the mounting hardware attached to the flanged channel sign post(s).

If the Contractor damages a bracket assembly or street name plates,

Dispose of the damaged bracket assembly or damaged street name plate.

Fabricate new bracket assembly or street name plate according to City of __,__County specifications, 2564.2F, “Traffic Signs and Devices: Signs and Markers;” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the bracket assemblies, aluminum sign panels, and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged sign panels, bracket assemblies, and mounting hardware will be reinstalled under Item No. 2564.536 - Install Sign Panel Type Special.

### Salvage Sign Type C and Type D

Inform the Engineer of any damaged in-place Type C or Type D sign panels prior to salvaging.

Remove and dispose of the sign structure, nuts, bolts and washers.

If the Contractor damages a sign panel,

Dispose of the damaged sign panel.

Fabricate a new sign panel according to 2564.2F, “Traffic Signs and Devices: Signs and Markers,” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the aluminum sign panels and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged Type C sign panels will be reinstalled under Item No. 2564.537 - Install Sign Type C.

Salvaged Type D sign panels will be reinstalled under Item No. 2564.537 - Install Sign Type D.

### Salvage Sign Type Special

Inform the Engineer of any damaged in-place sign prior to salvaging.

Salvage the sign post with the bracket assembly and street name plates attached.

If the Contractor damages a sign post, bracket assembly, or street name plate,

Dispose of the damaged sign post, bracket assembly, or street name plate.

Fabricate a new sign post, bracket assembly, or street name plate according to City of __,__County specifications, 2564.2F, “Traffic Signs and Devices: Signs and Markers;” and these Special Provisions, at no cost to the Department or City of __, __ County.

Prevent damage to the sign posts, bracket assemblies, aluminum sign panels, and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Salvaged signs will be reinstalled under Item No. 2564.537 - Install Sign Type Special.

## METHOD OF MEASUREMENT & BASIS OF PAYMENT

The Engineer will measure each item according to the Contract and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions.

The Department will include all work described in the Contract and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions as part of the contract unit price per unit of measure.

The Department will pay for traffic signs and devices on the basis of the following schedule:

Include only pay items that contain work specified in the special provisions.

| Item No.: | Item: | Unit: |
| --- | --- | --- |
| 2104.509 | Remove Sign Panel Type _ | Each |
| 2104.509 | Remove Sign Type _ | Each |
| 2104.523 | Salvage Sign Panel Type _ | Each |
| 2104.523 | Salvage Sign Type _ | Each |
| 2104.523 | Salvage Delineator | Each |
| 2104.523 | Salvage Marker | Each |

The Department’s payment for each item shall be compensation in full for all work, material, and costs involved in performing the work specified on the Plans and these Special Provisions.

# (2564) TRAFFIC SIGNS AND DEVICES

## DESCRIPTION

The Contractor shall furnish and install traffic signs in accordance with 2564, “Traffic Signs and Devices,” except as modified in these Special Provisions.

## MATERIALS

METRO DISTRICT ONLY

Fabricate all signs, markers, and delineators with material in accordance with 3352, “Signs, Delineators, and Markers” except as modified in these Special Provisions.

Fabricate all rigid permanent signs, markers, and delineators with materials from the MnDOT Approved/Qualified Products List.

Provide sign face material meeting the performance requirements of 3352.2.A.2.b, “Sign Sheeting Type IV” for rigid permanent signs, markers, and delineators described in Table 2564.2-ST1 and on the Plans:

| Table 2564.2-ST1: MnDOT Metro District Sign Sheeting Type IV Requirements |  |  |
| --- | --- | --- |
| Category | Item | Condition(s) |
| (1) | Sign Panels Type C | All Sign Panels Type C with the exception of: - R1-1 STOP, R1-2 YIELD, R1-3P ALL WAY, R5-1 DO NOT ENTER signs, - W-Series (warning) signs, - S-Series (school) signs, - Mast arm-mounted signs, and - Bridge-mounted signs. Provide sign face material meeting the requirements of 3352.2.A.2.f, “Sign Sheeting Type XI for the exceptions listed in Category (1). |
| (2) | Sign Panels Type D | Ground-mounted signs |
| (3) | Sign Panels Type Overlay | Type Overlays which are attached to signs described in Categories (1) and (2) |
| (4) | Delineator Type X3-1 | All |
| (5) | Delineator Type X4-6 | White background color |
| (6) | Delineator Type X4-8 | All |
| (7) | Infiltration Area Marker X3-6a | All |

Provide non-fluorescent reflective sheeting for the yellow background color portions for:

- Overhead-mounted Type D sign panels,

  - Sign Panel Type Overlays attached to overhead-mounted Type C and Type D sign panels, and

- X3-6A Infiltration Area Markers.

## CONSTRUCTION REQUIREMENTS

The provisions of 2564.3, “Construction Requirements, General” are modified and supplemented as follows:

The following replaces the fourth paragraph of 2564.3A:

Sign locations and sign structure posts lengths indicated on the Plans are approximate.  Locate and stake final sign and delineator locations and obtain approval of locations by the Engineer.   Determine the final post lengths for Type C signs, Type D signs and delineators in accordance with offsets, mounting heights and clearances detailed on the Plans and field verification of the proposed or inplace slopes.

### As-Built Signing Data

DO NOT INCLUDE provisions in DIV ST to collect sign data for inventory management purposes.  Use the Pay Item 2011.601 AS BUILT and follow the directions within DIV S to cover sign inventory management.

### Fabrication & Warning Stickers

Use this provision if new sign panels or sign panel overlays are being installed for Type A, Type C and Type D signs (almost all projects)

Install Department-provided warning stickers on new Type C and D sign panels according to 2564.3H.2, “Traffic Signs and Devices: Construction Requirements: Sign Panels: Fabrication and Warning Stickers.”

Give 30 days advance notice to the Department prior to picking up the Department-provided warning stickers:

Jeff Streeter	(Metro)-(for other districts, use maintenance area sign supervisor)

651-366-5191

### Field Spotting of Signs

Use this provision when installing signs where location and orientation is critical (i.e. roundabouts, RCUTs, DDI)

Give the Engineer 14 calendar days advance notice prior to installing signs inside or within 50 feet of roundabouts.

The Engineer will contact the District Traffic Office, which will provide personnel to field spot the installation location and orientation of the signs:

Contact Name

Title

Phone Number

Email@state.mn.us

### Infiltration Area Marker

Furnish and install a new 3 lb/ft. flanged channel post at the location approved by the Engineer.  Attach the furnished sign panel to the flanged channel post with new mounting hardware at the mounting height detailed on the Plans.

### Install Delineator and Marker

Use this provision when installing salvaged delineators and/or markers

For each salvaged delineator or marker being installed,

Furnish and install a new 3 lb/ft. flanged channel post at the location approved by the Engineer.

Attach the salvaged delineator or marker to the flanged channel post with new mounting hardware at the mounting height detailed on the Plans.

Install delineators under Item No. 2564.602 - Install Delineator.

Install markers under Item No. 2564.602 - Install Marker.

### Install Sign Panel Type Special

Use this provision when mounting salvaged street name signs above Type C signs on U-channel posts.

Attach the salvaged street name plate, attached bracket assembly, and salvaged mounting hardware to the sign post as directed by the Engineer.

Mount the bottom of the bracket assembly a minimum of six inches above the Type C sign panel.

### Install Sign Type C and D

Use this provision when installing Type C or Type D signs in areas of high winds such as Districts 1, 2, 4, 7, and 8

Install signs with 3/8” stainless steel bolts and zinc-plated nylon insert lock nuts on the L-Bracket assembly when a knee brace is used.

Use this provision when installing Type C or Type D signs in areas of high winds in District 8

Install signs with zinc 9/16” head nylon insert lock nuts placed on all structural hardware.  Use driven stub posts of at least 8 feet long and embedded a minimum of 54 inches.  Use flanged channel posts with a 3/8 in diameter holes punched on 3 inch centers for all stringers and lateral braces.

### Install Sign Type Special

Install each salvaged Sign Type Special:

At the location approved by the Engineer

At the same embedment depth as the salvaged sign.

### Post-Award Data

Use this provision if requested by the District Traffic Office

SignCAD Millennium (Program Version 8.61) panel layout files for panel layouts shown on the Plans are available electronically upon project award.  To request these files, please contact:

Contact Name

Title

Phone Number

Email@state.mn.us

MnDOT believes the electronic data it will provide is accurate, but MnDOT provides no guarantee or warranty, express or implied, concerning the accuracy of the data and the Contractor shall not act in reliance on the data without verifying the data against the contract documents.  The documents originally provided with the Contract remain the basis of the Contract, and the electronic data that will be provided at the Request of the Contractor is provided only for the convenience of the Contractor.  Therefore, if use of this data causes an error, omission, unacceptable work, or work not in conformance with the contract documents, then any costs to the Contractor to make corrections as a result of this error will not be considered "extra work", and the Contractor will not be entitled to an adjustment of contract time.

### Sign Panel Overlay Type _

For Districts 1, 2, 3, 4, 6, 7, 8 use the following provisions:

The provisions of 2564.3K, “Construction Requirements: Sign Panel Overlay Type_” are modified as follows:

Use retroreflective sheeting sign face material in accordance with 3352.2.A.2.f, “Sign Sheeting Type XI,” except for the following:

(1)	For a yellow background on sign panel overlays, use retroreflective sheeting as specified by 3352.2.A.2.f, “Sign Sheeting Type XI,” FL fluorescent yellow, and

(2)	For a brown background, use white retroreflective sheeting as specified by 3352.2.A.2.f, “Sign Sheeting Type XI.”

Use sign legend material as specified by 3352.2.A.2.f, “Sign Sheeting Type XI” for the sign face material on each sign panel, except for the following:

(1)	Use brown (1179), electronic Cuttable (EC) film produced by the same manufacturer that fabricates the retroreflective sheeting specified by 3352.2.A.2.f, “Sign Sheeting Type XI,” for sign legend material on sign panel overlays with brown sheeting; and

(2)	If brown EC film for sign panel overlays requires splicing, make splices vertical and butt spliced and spaced so splices do not occur through letters or arrows.

For METRO Districts use the following provisions:

The provisions of 2564.3K, “Construction Requirements: Sign Panel Overlay Type_” are modified as follows:

Use retroreflective sheeting sign face material in accordance with 3352.2.A.2.f, “Sign Sheeting Type XI,” except for the following:

(1)	For a yellow background on sign panel overlays, use retroreflective sheeting as specified by 3352.2.A.2.f, “Sign Sheeting Type XI,” non-fluorescent yellow, and

(2)	For a brown background, use white retroreflective sheeting as specified by 3352.2.A.2.f, “Sign Sheeting Type XI.”

Use sign legend material as specified by 3352.2.A.2.f, “Sign Sheeting Type XI” for the sign face material on each sign panel, except for the following:

(1)	Use brown (1179), electronic Cuttable (EC) film produced by the same manufacturer that fabricates the retroreflective sheeting specified by 3352.2.A.2.f, “Sign Sheeting Type XI,” for sign legend material on sign panel overlays with brown sheeting; and

(2)	If brown EC film for sign panel overlays requires splicing, make splices vertical and butt spliced and spaced so splices do not occur through letters or arrows.

### Sign Panels Type C and D

Use this provision when installing Type C or D sign panels in areas of high winds such as Districts 1, 2, 4, 7, and 8

Install signs with 3/8” stainless steel bolts and zinc-plated nylon insert lock nuts on the L-Bracket assembly when a knee brace is used.

### Sign Panels Type Overlay

Use this provision when including an organization’s official logo/pictograph and attaching them to the panel with rivets .

(pictograph = government agency; logo = private organization)

For Sign(s)/Sign Panel(s) A/D/OH-XX,

Use the official name of logo here for the logo/pictograph design on the sign panel.

If design requirements are publicly available:

Access the logo/pictograph's graphic design standards on the following website:

Insert website here

If design requirements are NOT publicly available:

Contact Company Name to obtain the logo/pictograph’s graphic design standards:

Contact Name

Company Name

Telephone and/or email

Fabricate the sign panel(s) with the logo/pictograph at the size and location shown on Sheet No(s). XX on the Plans.

Attach the logo/pictograph to the sign panel according to 2564.3H, “Traffic Signs and Devices: Construction Requirements: Sign Panels”, paragraph 1.

### Sign Panels Type Special

Use this provision if mounting new street name sign above Type C signs on U-channel posts.

Furnish and install street name plates, bracket assemblies, mounting hardware, and sign posts as detailed on the Plans.

The Engineer will approve the location of each sign post in the field.

### Sign Replacement Projects

Use this provision on sign replacement projects

The provisions of 2564.3U, “Traffic Signs and Devices: Construction Requirements: Scheduling of Work” are supplemented as follows:

For signs not detailed in 2564.3U, “Traffic Signs and Devices: Construction Requirements: Scheduling of Work”:

Schedule the work so that replacement Type C and Type D signs are installed the same work day that the in-place signs are removed.

## METHOD OF MEASUREMENT AND BASIS OF PAYMENT

The Engineer will measure each item according to the Contract and the 2564, “Traffic Signs and Devices: Construction Requirements” section of these Special Provisions.

The Department will include all work described in the Contract and the 2564, “Traffic Signs and Devices: Construction Requirements” section of these Special Provisions as part of the contract unit price per unit of measure.

The contract square foot [square meter] prices for Sign Panels Type EA and Type EO includes the cost of providing and installing the flanged channel or S4x7.7 panel mounting posts.

The Department will pay for traffic signs and devices on the basis of the following schedule:

Include only pay items that contain work specified in the special provisions.  For METRO projects, also include pay items which will have sign panels fabricated with sign sheeting Type IV.

| Item No.: | Item: | Unit: |
| --- | --- | --- |
| 2564.531 | Sign Panels Type _ | Square foot |
| 2564.535 | Sign Panel Overlay Type _ | Square foot |
| 2564.536 | Install Sign Panel Type _ | Each |
| 2564.537 | Install Sign Type _ | Each |
| 2564.550 | Delineator Type _ | Each |
| 2564.551 | Reference Location Sign | Each |
| 2564.552 | Object Marker Type_ | Each |
| 2564.553 | Bridge Number Marker X4-12A | Each |
| 2564.602 | Infiltration Area Marker X3-6A | Each |
| 2564.602 | Install Delineator | Each |
| 2564.602 | Install Marker | Each |
| 2564.602 | Install Sign Type Special | Each |

The Department’s payment for each item shall be compensation in full for all work, material, and costs involved in performing the work specified on the Plans and these Special Provisions.

# (3352) SIGNS, DELINEATORS AND MARKERS

## SCOPE

The Contractor shall fabricate traffic signs, delineators, and markers consisting of sign panels 3352, “Signs, Delineators and Markers” and these Special Provisions.

## REQUIREMENTS

The Contractor shall use materials according to 3352, “Signs, Delineators and Markers” and these Special Provisions.

METRO DISTRICT ONLY:

    - Sign Sheeting Type IV

The provisions of 3352.2.A.2.b, “Signs, Delineators, and Markers: Requirements: Sign Sheeting Type IV” are modified as follows:

Provide Sign Sheeting Type IV for highway signing, markers, and delineators. Provide white and fluorescent orange Sign Sheeting Type IV for reboundable plastic drums and weighted channelizers. Additionally, test to ensure the impact resistance of the material at 32 °F [0 °C].

The provisions of 3352.2.A.3, “Signs, Delineators, and Markers: Requirements: Warranty Requirements” are supplemented as follows:

| Table 3352-3 Minimum Retroreflectivity Values |  |  |  |
| --- | --- | --- | --- |
| Sign Sheeting Type | Warranty Period, years | Minimum Allowable Retroreflection | Total Daytime Luminance Factor Minimum |
| Type IV Signs, Delineators, and Markers | 1 - 7 | 80% | --- |
| Type IV Signs, Delineators, and Markers | 8 - 10 | 70% | --- |

## SAMPLING AND TESTING — (BLANK)