DIVISION ST

Section	P

No.	Item

ST-1	(2104) REMOVING MISCELLANEOUS STRUCTURES	1

ST-2	(2564) TRAFFIC SIGNS AND DEVICES	10

ST-3	(3352) SIGNS, DELINEATORS, AND MARKERS	31

ST-4	(3402) SQUARE TUBULAR SIGN POSTS	32

DELETE WHEN DONE: To Update Table of Contents Right Click on the index above and choose Update Field.  If given a choice choose “Update Entire Table”.

DELETE WHEN DONE: Statements highlighted in yellow are guidelines or instructional in nature.  Remove these notes before completing the spec.  When appearing at the top left of a provision, it pertains to the paragraph immediately beneath the note, as well as any indented items following it.

DELETE WHEN DONE: Words highlighted in green are fields that may need to be modified or removed before completion of the spec, such as contact information, city names, sign numbers, or charts on the Plans.

For provisions containing the descriptor, “For Sign(s) [Type]-XX:”, this descriptor should only be used when the provision applies to specific sign numbers among that type on the Plans.  Insert as necessary.  Leave out the descriptor if the provision applies to all signs of that type on the Plans.

I hereby certify that the Special Provisions for traffic sign construction (Division ST) contained in this proposal were prepared by me or under my direct supervision and that I am a duly licensed Professional Engineer under the laws of the State of Minnesota.

_________________________________________                            Engineer Name

Lic.  No.  XXXXX                    Date MM/DD/YY

age

No.

-ST

-ST

-ST

-ST

# DIVISION ST

# (2104) REMOVING MISCELLANEOUS STRUCTURES

## DESCRIPTION

The Contractor shall remove and salvage miscellaneous structures according to 2104, “Removing Pavement and Miscellaneous Structures” and these Special Provisions.

## MATERIALS

The Contractor shall use materials according to 2104, “Removing Miscellaneous Structures” and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions.

## CONSTRUCTION REQUIREMENTS

### Remove By Others

Use for projects that include LOGO signs.

Give at least 14 calendar days advance notice to the General Manager of Minnesota Logos to arrange removal of Logo signs designated on the Plans as “REMOVE BY OTHERS”.

Dave DeSutter

General Manager

Minnesota Logos

952-895-8079

ddesutter@interstatelogos.com

### Remove Grout - Sign Footing

Remove all in place grout between the base plate and the sign footing, taking appropriate measures not to damage the in-place anchor bolts, nuts, washers and conduit.

### Remove Delineator/Marker

Remove the delineator/marker panel, mounting hardware, and post.

### Remove Delineator/Marker Panel

Remove the delineator/marker panel and mounting hardware.  Leave the sign structure in place.

### Remove Sign

For each sign, remove the sign panels, mounting hardware, and sign structure.

Use for concrete rail mounted signs

For each concrete rail mounted sign, remove the sign panels, mounting hardware, and concrete rail mounted sign structure.  Fill anchorage holes in concrete with epoxy grout.

Use for bridge-mounted signs

For each bridge mounted sign, remove the sign panel, mounting hardware, and bridge mounted sign structure.

After removal of the sign structure,

Fill anchorage holes in concrete with epoxy grout.

Plug holes in steel with galvanized high strength steel bolts, washers, and nuts.

Use for sign replacement projects.

Schedule the work so that replacement signs are installed on the same work day that the in-place signs are removed.

### Remove Sign Type Special

Add to the list all equipment included with the sign

Remove sign panels, mounting hardware, sign structure, including any concrete footings.

### Remove Overhead Sign

For each overhead sign, remove the extruded sign panels, mounting hardware, overhead sign structure, and remove concrete footing to five feet below ground surface.  Disconnect the power conductors at the source of power (SOP) and the sign base.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to Section 2545, “Electric Lighting Systems.”  Shut off the circuit breaker(s) in service cabinets.  Bury the end of the power cable and abandon it in place as directed by the Engineer.

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Use when the heavy hex nuts are to be salvaged from the pedestal. (Metro)

Salvage as many heavy hex nuts as possible from the pedestal’s anchor rods.

Contact the Department for delivery instructions:

Pat O’Brien / Joe Podobinski

651-366-5820

### Remove Overhead Bridge Mounted Sign

For each overhead bridge mounted sign, remove the extruded sign panels, mounting hardware, and overhead bridge mounted sign structure.  Disconnect the power conductors at the source of power (SOP) and at the switch.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Do not include if bridge is being removed.

Cut the power cable at the conduit, which is on top of the 2 inch rigid steel conduit and attached to the bridge pier; ensure that 12 inches of cable extends out of the conduit.  Tape the cable end.  Remove the power cable and cable straps between the conduit and the switch.  Plug holes (from removed hangers in concrete) with epoxy grout.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Remove the feed point identification plate.

Do not include if bridge is being removed or re-decked.

After removing the sign structure from the bridge,

Fill anchorage holes (within the concrete) with epoxy grout.

Plug holes (within the steel) with galvanized high strength steel bolts, washers and nuts.

Use for bridge redecking projects when fascia beams will remain inplace.

Ensure that the fascia beam webs are not damaged during the removal of the sign structure(s).

Use for bridge redecking projects when fascia beams will remain inplace and holes will not be reused.

Fill the holes in the fascia beams with epoxy grout.

Use for bridge redecking projects when fascia beams will remain inplace and holes will be reused.

Leave open the holes in the fascia beams; they will be reused to mount new structures.

Use if junction box is the SOP

Disconnect the power conductors at the source of power (SOP) and at the switch.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Do not include if bridge is being removed.

Remove all conduit and power conductors between the source of power and the switch.  Plug any holes in the junction box as directed by the Engineer.  If the conduit is stubbed out of the bridge, cap the conduit end after removal of the power conductors.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Remove the feed point identification plate.

### Remove Monotube Sign

For each monotube sign, remove the sign panels, mounting hardware, monotube sign structure, and remove concrete footing to five feet below ground surface.

### Remove I-Beam Sign

For each I-Beam sign, remove the extruded sign panel, mounting hardware, posts, and concrete footings or H-pile footings completely.

### Remove Sign Lighting System

Use for removing existing lighting on cantilever or sign bridge (safety cable and lighting specs are not included in masterspec document)

For Sign(s) OH XX-XX:

Disconnect the power conductors at the source of power (SOP) and the switch.  Tape both ends.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Disconnect and remove the conductors between each sign lighting fixture, the switch, the sign base, and any fuse connectors in the sign base.

Disconnect and remove all conduit between the sign lighting fixtures, the switch, and the sign post’s conduit stub.

Remove the switch.  If the switch is mounted on the sign post, obtain the Engineer’s approval before capping the opening in the sign post.

Dismantle the sign lighting fixtures and any ballasts from the sign structure.

Remove the feed point identification plate.

Dispose of sign lighting system according to 2104, “Removing Pavement and Miscellaneous Structures.”

Use for bridge mounted sign with conduit and SOP above

For Sign(s) OH XX-XX:

Disconnect the power conductors at the source of power (SOP) and the switch.  Tape both ends.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Disconnect and remove conduit between the switch and the conduit stub in the bridge.  Cut the power conductors extending out of the conduit stub so that 12 inches of each conductor extends out of the stub.  Tape each power conductor, loop it, and insert back into the conduit stub.  Cap the conduit stub.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and any ballasts from the sign structure.

Remove the feed point identification plate.

Dispose of sign lighting system according to 2104, “Removing Pavement and Miscellaneous Structures.”

Use for bridge mounted sign with junction box as SOP

For Sign(s) OH XX-XX:

Disconnect the power conductors at the source of power (SOP) and the sign base.  Tape both ends.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Remove the power conductors between the source of power and the switch.

Disconnect and remove conduit between the switch and the conduit stub in the bridge.  Cap the conduit stub.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and ballasts (if encountered) from the sign structure.

Remove the feed point identification plate.

Dispose of sign lighting system according to 2104, “Removing Pavement and Miscellaneous Structures.”

Use if pullbox or streetlight on mainline is the SOP (this is the old bridge mounted method)

For Sign(s) OH XX-XX:

Disconnect the power conductors at the source of power (SOP) and at the switch.  Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Cut the power cable at the conduit, which is on top of the 2 inch rigid steel conduit and attached to the bridge pier; ensure that 12 inches of cable extends out of the conduit.  Tape the cable end.  Remove the power cable and cable straps between the conduit and the switch.  Plug holes (from removed hangers in concrete) with epoxy grout.

Disconnect and remove the conductors between each sign lighting fixture and the switch.

Disconnect and remove all conduit between the sign lighting fixtures and the switch.

Remove the switch.

Dismantle the sign lighting fixtures and any ballasts from the sign structure.

Remove the feed point identification plate.

Dispose of sign lighting system according to 2104, “Removing Pavement and Miscellaneous Structures.”

After removing the sign structure from the bridge,

Fill anchorage holes (within the concrete) with epoxy grout.

Plug holes (within the steel) with galvanized high strength steel bolts, washers and nuts.

### Remove Extruded Sign Panel

Remove the extruded sign panel, mounting hardware, and any panel mounting posts.  Leave the sign structure in place.

### Remove Sign Panel

Remove the sign panel and mounting hardware.  Leave the sign structure in place.

### Remove Sign Panel Type Special

Add to the list all equipment included with the sign panel

Remove sign panels and mounting hardware.  Leave the sign structure in place.

### Contractor Responsibility for Salvaged Sign Panels

To be included for any salvage pay items.

Contractor to inform the Engineer of any damaged or missing in-place sign panels prior to salvaging.

If a sign panel is damaged, lost, or stolen after being salvaged, a new sign panel shall be fabricated according to 2564.2F “Traffic Signs and Devices: Signs and Markers,” and these Special Provisions, at no cost to the Department, City of ____ or ____ County.

To deter theft, store sign panels in a secure area.

Contractor to prevent damage to the bracket assemblies, sign panels, and the sign sheeting materials at all times, including during storage.  Methods to prevent damage during storage include but are not limited to:

Store sign panels so they are not laying on the ground.

Store sign panels so that the reflective surfaces do not come in contact with dirt, water, or grass.

Store sign panels so they are not covered with a tarp or plastic.

### Salvage Delineator/Marker

Salvage the sign structure, mounting hardware, and delineator/marker.

Salvaged delineator/marker to be stored for reinstallation.

### Salvage Sign

Use if existing post will be salvaged.

Remove and dispose of the in-ground base for the sign structure.

Use if existing post will not be salvaged.

Remove and dispose of the sign structure and mounting hardware

The salvaged sign to be stored for reinstallation.

### Salvage Sign Type Special

Use if salvaging a street name sign.

Salvage the sign post (if post is MASH-16 crashworthy) and the bracket assembly with street name plates attached.

Remove the sign post if not MASH-16 crashworthy.

If the Contractor damages a sign post, bracket assembly, or street name plate,

Dispose of the damaged sign post, bracket assembly, or street name plate.

Fabricate a new sign post, bracket assembly, or street name plate according to City of __ or __County specifications, 2564.2F, “Traffic Signs and Devices: Signs and Markers;” and these Special Provisions, at no cost to the Department, City of __, or __ County.

Salvaged sign special to be stored for reinstallation.

### Salvage Extruded Sign Panel

Salvage the extruded sign panel and panel mounting posts.

Remove the post clips.

Salvaged extruded sign panel and panel mounting posts to be stored for reinstallation.

### Salvage Sign Panel

Salvage the sign panel.

Remove mounting hardware.

Salvaged sign panel to be stored for reinstallation.

### Salvage Sign Panel Type Special

Use if salvaging street name sign panels.

Salvage the bracket assembly with street name plates attached.

If the Contractor damages a bracket assembly or street name plates,

Dispose of the damaged bracket assembly or damaged street name plate.

Fabricate new bracket assembly or street name plate according to City of __ or __County specifications, 2564.2F, “Traffic Signs and Devices: Signs and Markers;” and these Special Provisions, at no cost to the Department, City of __ or __ County.

Salvaged sign panel special to be stored for reinstallation.

### Salvage Overhead Sign

If the OH sign has lighting to be removed as part of the salvage, insert the appropriate language from the 2104: Construction Requirements: Remove Sign Lighting System section of this document.

If the OH sign has walkway to be removed as part of the salvage, insert the appropriate language from the 2104: Construction Requirements: Remove Sign Walkway section of this document.

Pull the power cable out of the conduit in the footing prior to removal of the concrete footing.  Tape the ends of the conductors.

Salvage the sign truss (including sign panels and panel mounting posts) and sign post from the concrete footing and store outside the clear zone on the job site as directed by the Engineer.  The sign truss and sign post shall each be placed on sufficient cribbing (railroad ties, etc.) to keep them straight and clear of the ground.  The salvaged sign truss and post to be stored for reinstallation.

Remove the concrete footing to five feet below ground surface.  Bury the end of the power cable and abandon in place as directed by the Engineer.

### Salvage Overhead Bridge Mounted Sign

If the OH sign has lighting to be removed as part of the salvage, insert the appropriate language from the 2104: Construction Requirements: Remove Sign Lighting System section of this document.

If the OH sign has walkway to be removed as part of the salvage, insert the appropriate language from the 2104: Construction Requirements: Remove Sign Walkway section of this document.

Salvage the bridge mounted sign structure and sign panels.  The salvaged sign structure and sign panels to be stored for reinstallation.

Remove bolts, nuts, and the washers connecting to girder webs.  Avoid damaging the in-place girder webs.

### Salvage Monotube Sign

Salvage the sign panels, stringers, and monotube sign structure from the concrete footing.  Store outside the clear zone on the job site as directed by the Engineer.  The monotube sign structure shall be placed on sufficient cribbing (railroad ties, etc.) to keep them straight and clear of the ground.  The salvaged monotube sign to be stored for reinstallation.

Remove sign mounting hardware.

Remove the concrete footing to five feet below ground surface.

### Salvage I-Beam Sign

Use this provision if salvaging and later installing upon new footings.

For Sign(s) A-XX, the Contractor shall:

Salvage the in place extruded sign panel and sign posts.

Remove the post clips.

Remove the friction fuse and mounting hardware from each sign post.

Remove the concrete or H pile footings.

The salvaged sign to be stored for reinstallation.

Use this provision if salvaging and later installing upon the same footings.

For Sign(s) A-XX, the Contractor shall:

Salvage the in place extruded sign panel and post section above the friction fuses.

Remove the post clips.

Remove the friction fuse and mounting hardware from each sign post.

Remove the lower sign post section (from the base plate to the friction fuse) of each sign post.

The salvaged sign to be stored for reinstallation.

### Remove Sign Walkway

Use for walkways on cantilevers or sign bridges without lighting systems.

For Sign(s) OH XX-XX:

Remove the walkway and walkway supports.

Repair cut/damaged galvanized surfaces according to ASTM A780.  Apply repair materials according to the manufacturer’s instructions.

After removal of the walkway from the sign truss:

Furnish high strength bolts, nuts, and washers according to 3391.2B, “Fasteners: High Strength Structural Steel Bolts” for each open hole in the sign truss.

Galvanize the high strength bolts, nuts, and washers according to 3392, “Galvanized Hardware.”

Install the high strength bolts, nuts, and washers in the open holes and tighten according to 2402.3G2c, “Steel Bridge Construction: Permanent Connections: Installation.”

Use for walkways for signs on bridges without lighting systems.

For Sign(s) OH XX-XX:

Cut each walkway support so that 2 inches of each support extend beyond the face of the sign panel.

Remove the walkway supports attached to the walkway.

Remove the feed point identification plate.

Repair cut/damaged galvanized surfaces according to ASTM A780.  Apply repair materials according to the manufacturer’s instructions.

Use for walkways with lighting systems on cantilevers or sign bridges.

For Sign(s) OH XX-XX:

Disconnect the power cable at the source of power (SOP) and the sign base.  Tape the ends of the power cable.

Resplice and waterproof any SOP power cable connections required to maintain roadway lighting according to 2545, “Electric Lighting Systems.”

If the SOP is a service cabinet,

Disconnect the power conductors from the circuit breaker(s).

Shut off the circuit breaker(s).

Contact the Department for final acceptance of the source of power disconnection:

John Pedersen

Metro Traffic Lighting Operations

651-234-7849

Or

Mike Posch

Greater MN

320-248-3179

Disconnect and remove the conductors between the sign lighting fixtures and the switch, as well as between the switch and the sign base.  Remove fuse connectors encountered in the sign base.

Disconnect and remove conduit between the switch and the conduit stub on the sign post.  Cap the conduit stub.

If the switch is not attached to the sign post,

Disconnect and remove the conduit between the sign lighting fixtures and the switch.

Remove the switch.

If the switch is attached to the sign post,

Cap the opening in the sign post to the satisfaction of the Engineer.

Dismantle the sign lighting fixtures and any ballasts from the sign structure.

Remove the walkway, walkway supports, and the feed point identification plate.

Repair cut/damaged galvanized surfaces according to ASTM A780.  Apply repair materials according to the manufacturer’s instructions.

After removal of the walkway from the sign truss:

Furnish high strength bolts, nuts, and washers according to 3391.2B, “Fasteners: High Strength Structural Steel Bolts” for each open hole in the sign truss.

Galvanize the high strength bolts, nuts, and washers according to 3392, “Galvanized Hardware.”

Install the high strength bolts, nuts, and washers in the open holes and tighten according to 2402.3G2c, “Steel Bridge Construction: Permanent Connections: Installation.”

## METHOD OF MEASUREMENT & BASIS OF PAYMENT

The Engineer will measure each item according to the Contract and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions.

The Department will include all work described in the Contract and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions as part of the contract unit price per unit of measure.

The Department will pay for traffic signs and devices on the basis of the following schedule:

Include only pay items that contain work specified in the special provisions.

| Item No.: | Item: | Unit: |
| --- | --- | --- |
| 2104.502 | Remove Grout – Sign Footing | Each |
| 2104.502 | Remove Delineator/Marker | Each |
| 2104.502 | Remove Delineator/Marker Panel | Each |
| 2104.502 | Remove Sign | Each |
| 2104.502 | Remove Sign Type Special | Each |
| 2104.502 | Remove Overhead Sign | Each |
| 2104.502 | Remove Overhead Bridge Mounted Sign | Each |
| 2104.502 | Remove Monotube Sign | Each |
| 2104.502 | Remove I-Beam Sign | Each |
| 2104.502 | Remove Sign Lighting System | Each |
| 2104.502 | Remove Extruded Sign Panel | Each |
| 2104.502 | Remove Sign Panel | Each |
| 2104.502 | Remove Sign Panel Type Special | Each |
| 2104.502 | Salvage Delineator/Marker | Each |
| 2104.502 | Salvage Sign | Each |
| 2104.502 | Salvage Sign Type Special | Each |
| 2104.502 | Salvage Extruded Sign Panel | Each |
| 2104.502 | Salvage Sign Panel | Each |
| 2104.502 | Salvage Sign Panel Type Special | Each |
| 2104.502 | Salvage Overhead Sign | Each |
| 2104.502 | Salvage Overhead Bridge Mounted Sign | Each |
| 2104.502 | Salvage Monotube Sign | Each |
| 2104.502 | Salvaged I-Beam Sign | Each |
| 2104.503 | Remove Sign Walkway | Linear foot |

The Department’s payment for each item shall be compensation in full for all work, material, and costs involved in performing the work specified on the Plans and these Special Provisions.

# (2564) TRAFFIC SIGNS AND DEVICES

## DESCRIPTION

The Contractor shall furnish and install traffic signs in accordance with 2564, “Traffic Signs and Devices,” except as modified in these Special Provisions.

## CONSTRUCTION REQUIREMENTS

The provisions of 2564.3A, “Traffic Signs and Devices: Construction Requirements: General” are modified and supplemented as follows:

The following replaces the fourth paragraph of 2564.3A:

Sign locations and sign structure posts lengths indicated on the Plans are approximate.  Locate and stake final sign and delineator locations and obtain approval of locations by the Engineer.   Determine the final post lengths for signs and delineators in accordance with offsets, mounting heights and clearances detailed on the Plans and field verification of the proposed or inplace slopes.  Determine the final post lengths for I-Beam and Overhead signs in accordance with the offsets, mounting heights and clearances detailed on the Plans and by field verification of the proposed or inplace slopes.   Provide shop drawings for I-Beam and Overhead signs in accordance with 2564.3, “Construction Requirements, Structural Steel.”

### As-Built Signing Data

DO NOT INCLUDE provisions in DIV ST to collect sign data for inventory management purposes.  Use the Pay Item 2011.601 AS BUILT and follow the directions within DIV S to cover sign inventory management.

### Warning Stickers

Use this provision if new sign panels or sign panel overlays are being installed for Type A, Type C and Type D signs (almost all projects) You will need to do a PIF, the template is at https://www.dot.state.mn.us/pre-letting/prov/public-interest.html

Install Department-provided warning stickers on new sign panels according to 2564.3H.2, “Traffic Signs and Devices: Construction Requirements: Sign Panels: Fabrication and Warning Stickers.”

Give 30 days advance notice to the Department prior to picking up the Department-provided warning stickers:

Chris Dochniak	(Metro)-(for other districts, use maintenance area sign supervisor)

651-755-0316

James Smidt	(for D6 use)

507-286-7618

If the project involves I-Beam signs:

For I-Beam signs and overlays on I-Beam signs,

Affix the warning sticker to each I-Beam extruded panel in the lower right corner of the back of the extruded panel, directly above the fabrication sticker.

### Field Spotting of Signs

Use this provision when installing signs where location and orientation is critical (i.e. roundabouts, RCUTs, DDI)

Give the Engineer 14 calendar days advance notice prior to installing signs inside or within 50 feet of roundabouts.

The Engineer will contact the District Traffic Office, which will provide personnel to field spot the installation location and orientation of the signs:

Contact Name

Title

Phone Number

Email@state.mn.us

### Post-Award Data

Use this provision if requested by the District Traffic Office

SignCAD panel layout files for panel layouts shown on the Plans are available electronically upon project award.  To request these files, please contact:

Contact Name

Title

Phone Number

Email@state.mn.us

### MnDOT believes the electronic data it will provide is accurate, but MnDOT provides no guarantee or warranty, express or implied, concerning the accuracy of the data and the Contractor shall not act in reliance on the data without verifying the data against the contract documents.  The documents originally provided with the Contract remain the basis of the Contract, and the electronic data that will be provided at the Request of the Contractor is provided only for the convenience of the Contractor.  Therefore, if use of this data causes an error, omission, unacceptable work, or work not in conformance with the contract documents, then any costs to the Contractor to make corrections as a result of this error will not be considered "extra work", and the Contractor will not be entitled to an adjustment of contract time.

### Sign Replacement Projects

Use this provision on sign replacement projects

The provisions of 2564.3V, “Traffic Signs and Devices: Construction Requirements: Scheduling of Work” are supplemented as follows:

For signs not detailed in 2564.3V, “Traffic Signs and Devices: Construction Requirements: Scheduling of Work”:

Schedule the work so that replacement signs are installed the same work day that the in-place signs are removed.

### New Overhead, I-Beam and Monotube Sign Posts

Use this provision for projects with new overhead signs, I-beam signs, and monotube signs.

Replace paragraph 4 in section 2564.3A with:

Sign locations and post lengths indicated in the contract are approximate. The Engineer will make the final determination of sign locations and the required post lengths for I-Beam, Overhead, and Monotube Signs in the field prior to starting fabrication of the posts. The Contractor shall determine the required post lengths for all other signs.

### New Posts Installed

Use this provision in all of Districts 1 and 2 and in all counties north of and including for District 3 (Kanabec, Mille Lacs, Morrison, Todd) for District 4 (Douglas, Grant, Traverse).

Base post lengths for ground mounted signs shall be 60 inches instead of 48 inches.

### E-ZPASS

The E-ZPASS logo color is Pantone #259.

To obtain the E-ZPASS logo graphical image for printing contact:

Joe Bruer

Joseph.bruer@state.mn.us

### Post-Installed Adhesive Anchorages

Use this provision when installing signs to concrete structures

This section is applicable for any post-installed adhesive anchorage used to facilitate installation of signs to in-place concrete structures.

Except when part of a proprietary anchorage assembly, ensure threaded rods and bolts meet the requirements of 3385, "Anchor Rods," and 3391," Fasteners," respectively.

- Post-Installed Adhesive Anchorages

Adhesive anchorage installers must hold current ACI Adhesive Anchor Installer Certification credentials.  Installers are required to check depth, diameter and condition of the drilled hole, clean the hole, and install the anchorage per the Manufacturer’s Printed Installation Instructions (MPII).  Record the name(s) of all certified installers on the RECORD OF CONTRACTOR/INSTALLER ACI CERTIFICATION form available on the MnDOT Bridges and Structures website under "Bridge Construction; Construction forms and tools".

Furnish only one of the systems listed on the Department’s "Approved/Qualified Products List for Bridge Products, Concrete Adhesive Anchorages for Structural Applications".  Verify that the adhesive has an uncracked characteristic bond strength as specified in the plan.  Install all anchors as specified by the MPII.  Install in sound concrete to a depth equal to the minimum depth specified in the plan or as specified by the manufacturer, whichever is greater.

Meet the following conditions prior to installation and testing:

- Concrete is greater than 14 days old;

- Concrete surface is free of water prior to drilling;

- The hole is dry, as defined below; and

- Any additional requirements listed in the Manufacturer’s Printed Installation Instructions.

A dry hole is defined as:  a hole with no water present within the hole.  If the hole is filled with water, partially filled with water, or water entered the hole during drilling, blow out the water using compressed air and allow a minimum of 24 hours dry-out time before cleaning the hole and installing the anchorage.

It is essential that the adhesive material completely fill the hole in the concrete for proper anchorage performance.  Ensure that the hole is completely filled to the top of the concrete surface in which the anchorage is installed.  Do not permit the adhesive to overtop the concrete surface in a way that will interfere with the placement of the elements.

- Testing of Post-Installed Anchorages

Perform all testing by an independent third party testing agency.  Testing agent must have current ACI Adhesive Anchor Inspector Certification credentials.

Verify the anchor strength and installation procedures by proof testing anchorages in accordance with this specification.  Perform all testing in accordance with ASTM E488, Standard Test Methods for Strength of Anchors in Concrete Elements.  Set up the tension testing device such that no portion of the device bears on the concrete surface within a distance equal to one and a half times the anchorage embedment depth.  Test anchorages to not less than the required proof load as provided in the plan (if no anchor proof load is provided in the plan, contact the Engineer).  Failure criteria of an anchorage test are defined in ASTM E488.

Ensure that nothing interferes with the testing apparatus during the proof test.  Do not perform any caulk prior to testing.

Verify the anchor strength and installation procedure by demonstrating the anchorage system at the first site of field installation.  One passing demonstration is required to be able to move to the remaining production anchorage installations.  Include a proof test in each demonstration installation.  Failure of a proof test will require a modification of installation procedures or use of a different anchorage system and an additional demonstration of the modified or substituted system.   Demonstration anchorages may be used as production anchorages.  The Contractor assumes all liability for repairs that may need to be performed as a result of a failed test.  Record all demonstration results on the PRE-PRODUCTION ANCHORAGES QUALIFICATION TEST REPORT available on the www.dot.state.mn.us/bridge/construction.html under "Construction forms and tools," and furnish the original of the completed form to the Engineer.

Notify the Engineer immediately after any failure.  Provide a non-conformance anchorage replacement plan, to be accepted by the Engineer.  Once accepted by the Engineer:

- Remove all anchorages that fail the field test without damage to the surrounding concrete;

- Redrill holes to remove adhesive bonding material;

- Install replacement anchorages in accordance with the MPII; and,

- Test anchors using the method listed above.

Perform replacement of failed anchorages to the satisfaction of the Engineer and at no cost to the Department.

Payment for all costs associated with furnishing, testing, and installing the anchorages are included with the sign installation.

### Modify Post

Do not splice I-Beam sign post sections below the friction fuse.

Extend I-Beam sign posts or panel mounting posts by bolt splicing as required by the contract and in accordance with 2471, “Structural Metals.”

The Engineer will measure modify post as a complete unit, including adding post length as shown in the contract.

### Install Extruded Sign Panel

Install a salvaged extruded sign panel as required by the contract.

Use new post clips to attach extruded panel to posts.

Torque each post clip to between 12 foot•lb and 14 foot•lb when attaching extruded sign panel to posts.

For overhead mounted extruded sign panels, install new panel mounting posts and bracket assemblies as shown on the plans.

Use this provision when modifying an I-Beam extruded panel

For Extruded Sign Panel(s) A-XX,

Remove existing overlay.

Furnish and install OR Remove extruded panel sections to obtain the extruded panel height specified on the Plans.

Use this provision when modifying an overhead extruded panel

For Extruded Sign Panel(s) OH XX-XX,

Remove existing overlay.

Furnish and install OR Remove extruded panel sections to obtain the extruded panel height specified on the Plans.

Modify the in-place panel mounting posts as detailed on the Plans.

The Engineer will measure install extruded sign panel as a complete unit, including salvaged extruded sign panel, modifications to the extruded sign panel, modifications to panel mounting posts, and new mounting hardware.

### Install Sign Panel

Install a salvaged or Department provided sign panel with new mounting hardware to sign structure.

Sign panel punching codes, stringer details, and panel joint details have been updated to correspond with new square tube post standards and crashworthy requirements.

Field Punch salvaged sign panels as needed based on the new Punching Codes and Stringer Detail file found on MnDOT’s Standard Signs and Markings Manual (2020) website:

http://www.dot.state.mn.us/trafficeng/publ/signsmanual/index.html

Aluminum stringers shall be extruded aluminum made with 6061-T6 Alloy.  The stringer shape shall be a channel with the legs folded back upon themselves to form an enlarged area for mounting to the post.  The stringer shall be two inches wide with a milled surface that the sign will be mounted upon.  The stringer shall have 3/8 inch holes provided at one inch intervals on center.  The legs of the stringer shall be 7/8 inch long to the folded portion.  The nominal thickness of the stringer shall be 1/8 inch.

The Engineer will measure install sign panel as a complete unit, including new mounting hardware and stringers used to install salvaged or Department provided sign panel.

### Install Sign Panel Type Special

Use if installing salvaged street name bracket assembly.

Install the salvaged bracket assembly with street name plates attached to new square tube sign structure using new mounting hardware as directed by the Engineer.

The Engineer will measure install sign panel Special as a complete unit, including salvaged bracket assembly with street name plates attached, new square tube posts, and new mounting hardware.

### Extend Walkway Support

Extend each walkway support on existing overhead sign structures as required by the contract. Galvanize support extensions and repair galvanized areas marred by cutting or welding in accordance with 2471, “Structural Metals.”

The Engineer will measure extend walkway support as a complete unit, including adding walkway support extensions as shown in the contract.

### Friction Fuse

Use if only installing a friction fuse and not a complete structure.

Provide and install a friction fuse on each existing I-Beam sign post as required by the contract and the following:

(1)	Remove the in-place friction fuse, consisting of a friction fuse plate and a hinge plate, and in-place mounting hardware, and dispose of these items in accordance with 2104, “Removing Pavement and Miscellaneous Structures”

(2)	Provide and install a new friction fuse consisting of a friction fuse plate and a hinge plate, mounting hardware as required by the contract

(3)	Repair damaged galvanized surfaces in accordance with 2471.3L.1, “Galvanizing”

The Engineer will measure friction fuse as a complete unit, including removing inplace friction fuse and installing new friction plate, hinge plate, and hardware.

### Keeper Plate

Use if only installing a keeper plate and not a complete structure.

Provide and install a keeper plate on each existing I-Beam sign post as required by the contract.

Verify sign post dimensions in the field. Immediately before installing keeper plates, clean the base connection plate to remove grit, dirt, and deleterious material.

Reinstall the post in accordance with base connection bolting procedures shown in the contract.

Replace missing, damaged, or rusty hardware.

The Engineer will measure keeper plate as a complete unit, including removing inplace keeper plate and installing new keeper plate.

### Delineator Type Special

Use this provision if using Linear Delineation Panels (Waffle Sheeting) – Very Rare.

This work shall consist of furnishing and installing linear delineation panels upon median barriers and guardrail in accordance with the provisions of MnDOT 2564, the details shown in the Plans, and the following:

Provide linear delineation panels of sizes and colors identified in Tabulation X on Sheet No. XX of the Plan.

Only use linear delineation panels listed on MnDOT’s Approved/Qualified Products List under “Signing: Delineation Devices.”

Remove any inplace linear delineation panels that conflict with the installation of new linear delineation panels shown in the Plan.

Modify the following as necessary to identify mounting requirements and/or panel spacing (if not identified in Plan).

For concrete barrier mounted linear delineation panels:

Place each of the linear delineation panels along the concrete barrier, spaced (edge-to-edge) at the distance apart specified in the Plans.

Vertically space the linear delineation panels 2” below the top edge of concrete barrier to the top edge of the panel.

Prior to installing the linear delineation panels, prepare and clean the concrete surface per

the manufacturer’s detail. Attach the linear delineation panels to concrete barrier surface

with anchor bolts AND adhesive caulk per the manufacturer’s detail.

For guardrail mounted linear delineation panels:

Place each of the linear delineation system panels along the inside vertical surface of the guardrail

trough, spaced (center-to-center) at the distance apart specified in the Plans.  Center each panel horizontally between guardrail post bolt heads.

The Contractor may adjust the spacing of panels if the specified panel distance and placement conflicts with guardrail post bolt heads.  If spacing is adjusted, the Contractor shall ensure approximately uniform spacing between the panels.

Prior to installing the linear delineation panels, prepare and clean the guardrail surface per

the manufacturer’s detail. Attach the linear delineation system panels to the guardrail surface with

adhesive tape per the manufacturer’s detail.

### Overhead Sign Structure Repair

Repair an existing overhead sign structure in accordance with the 2123, “Equipment Rental,” and the following:

Contact the Department's Structural Metals Inspection Unit (DSMIU) to schedule inspection of sign structures. Perform the inspection before removing the sign structure from storage, or after salvaging the structure.

Correct the following conditions, if encountered, as determined by the DSMIU:

(1)	Rusted or missing nuts, bolts or washers

(2)	Defective shop and field splices on main chord angles

(3)	Missing welds

(4)	Cracking welds or structural elements

(5)	Section loss on post base plate

(6)	Flame gouges on base plate or at bolt holes

(7)	Cracks around post handhole

(8)	Zinc coating loss or deterioration

(9)	Rusting

Perform repairs in accordance with 2471, “Structural Metals.”

Verify that each locking pin fits completely into the locking pin hole in the handrail hinge, while the handrail is in the raised position. For locking pins that do not fit, drill out the handrail hinge to make a proper fit. Repair damage to galvanized surfaces in accordance with 2471.3L.1, “Galvanizing.”

After completing the repairs and before installation, obtain reinspection by the DSMIU.

The Engineer will measure overhead sign structure repair by the number of hours required to complete the repair, including use and operation of equipment, travel time inside the project limits, and work and materials involved. Overhead sign structure repair is exempt from 1903, “Compensation for Altered Quantities.”

### Sign Panels Type Special

Use for new street sign panels.

Fabricate the street sign panels in accordance with 3352, “Signs.”

Package, deliver, store, and install street sign panels in accordance with 1607, “Handling Materials,” 3352, “Signs,” and the retroreflective sheeting manufacturer's recommendations.

Install on inplace post and attach new street sign panels with new brackets and mounting hardware.

The Engineer will measure sign panels type special by square foot based on the nominal dimensions of the street sign panels. Sign panels are considered rectangular for the purpose of measurement except that the Engineer will measure triangular shaped sign panels as the actual area of the triangle. The Engineer will not make deductions for rounded corners.  Sign type special includes new street sign panels, new post, new brackets, and new mounting hardware.

### Install Sign Collar

For signs installed in concrete as indicated in the plans, they shall be installed using sign collars supplied by the City.  The Contractor shall contact City of Saint Paul Traffic Operations, 651- 266-9778, to obtain sign collars. Sign posts installed in sign collars shall be Schedule 40 aluminum round pipe with an inside diameter of two inches (2"), an outside diameter of two and three eighths inches (2.375"), and a length of ten feet (10').

The Engineer will measure install sign collar as a complete unit, including installing City furnished sign collar.

### Sign Collar

For signs installed in concrete as indicated in the plans, they shall be installed using sign collars.  The Contractor shall contact City of Saint Paul Traffic Operations, 651- 266-9778, to obtain information on the sign collars. Sign posts installed in sign collars shall be Schedule 40 aluminum round pipe with an inside diameter of two inches (2"), an outside diameter of two and three eighths inches (2.375"), and a length of ten feet (10').

The Engineer will measure sign collar as a complete unit, including new sign collar.

### Install Sign

Provide and install new in ground base for the sign structure.  Install new riser post if post was removed.  Install salvaged post to new base.  Install salvaged or Department provided sign panel(s) with new stringers and mounting hardware to riser post.

Sign panel punching codes, stringer details, and panel joint details have been updated to correspond with new square tube post standards and crashworthy requirements.

Field Punch salvaged sign panels as needed based on the new Punching Codes and Stringer Detail file found on MnDOT’s Standard Signs and Markings Manual (2020) website:

http://www.dot.state.mn.us/trafficeng/publ/signsmanual/index.html

The Engineer will measure install sign as a complete unit, including new or salvaged post(s), new stringers, and new mounting hardware used to install salvaged or Department provided sign panel(s).

### Install Delineator/Marker

Install new base post.  Install salvaged riser post with attached delineator/marker (if square tube) or new riser post and attach salvaged or Department provided delineator/marker with new mounting hardware.

The Engineer will measure install delineator/marker as a complete unit, including new base post, salvaged riser post or new riser post, and salvaged or Department provided delineator/marker attached with new mounting hardware.

### Foundation Special OH

For the Foundation Special OH work on Signs designated for repair in the Plans:

Remove the top 6” of concrete surrounding the anchor rods. Clean all debris and grease at the top concrete surface and anchor rods after removal. No separate payment will be made for removal of grout and concrete.

Installation of Coupling Nuts and Threaded Anchor Rods:

The surfaces of the anchor rods shall be free of concrete, grease or other debris. Saw-cut the corroded part of inplace anchor rods and, if necessary, tap-thread the remaining part of the anchor rods with the length that meets the coupling nut manufacturer’s minimum required length.

Clean the anchor rods and concrete of any debris and oil due to the thread-tapping.

Thread coupling nut over the anchor rod to have equal amounts of threading of the existing and new anchor rods inside the coupling. This above mentioned work shall be inspected by a qualified and certified third party inspector from independent testing agencies that are accepted by the Engineer. Submit the inspection report to MnDOT for review and filing.

The top of all extended anchor rods shall be at the elevations as shown in the Plan.

Install the PVC conduit extension to project 1” above the foundation design elevation.

Provide a template for anchor rod placement and keep the template in place until the concrete has set.

Apply anti-seize compound to all threads with a brush. Use an approved bridge grease found on MnDOT’s Approved/Qualified Products list under Bridge Products.

Construct footing extensions in accordance with 2401, “Concrete Bridge Construction.”

Apply bonding grout to the in-place concrete immediately before placing the new concrete against it. Use an approved epoxy bonding agent found on MnDOT’s Approved/Qualified Products list under Concrete products – Epoxies (non-bridge applications).

The Engineer will measure foundation special OH as a complete unit, including removal of concrete, cutting and threading anchor rods, new anchor rod extensions, and new concrete.

### Repair Sign Type OH

Describe repairs to be made.

The Engineer will measure repair sign type OH as a complete unit, including all repairs as described.

### Reposition Sign Panel

For Sign Panel(s) OH XX-XX,

Inform the Engineer of any damaged to in-place OH sign panels prior to repositioning.

Reposition the panel mounting posts add detail here as necessary; vertically, horizontally, etc. on the truss in accordance with the include elements present in the plan here; overhead details, cross-section(s), etc. in the Plans.

Prevent damage to the sign panels and the sign sheeting materials at all times, including during storage.

Methods to prevent damage during storage include but are not limited to:

Store sign panels so that they are NOT lying on the ground.

Store sign panels so that reflective surfaces do NOT come in contact with dirt, water, or grass.

Store sign panels so that they are NOT covered with plastic or a tarp.

Include the following if desired for the project

Schedule the work so that the sign panel is repositioned on the same work day.

### Install I-Beam Sign

Use this provision if installing salvaged sign on new footings.

For Sign(s) A-XX, the Contractor shall:

Install new footings.

Attach the salvaged posts with new keeper plates and friction fuses to the new footings.

Attach the salvaged extruded panel to the posts with new post clips.

Torque each post clip to between 12 foot•lb and 14 foot•lb when attaching extruded panels to posts.

Installations shall be in accordance with the contract and details shown on the plans. Repair damaged galvanized surfaces in accordance with 2471, “Structural Metals.”

The Engineer will measure install I-Beam sign as a complete unit, including new footings, salvaged posts, new friction fuse, salvaged or Department provided extruded panel, and new mounting hardware.

Use this provision if installing salvaged sign on the same footings.

For Sign(s) A-XX, the Contractor shall:

Attach new lower post section to the existing footings with new keeper plates.

Attach salvaged upper post section to lower post section with new friction fuse.

Attach the salvaged extruded panel to the posts with new post clips.

Torque each post clip to between 12 foot•lb and 14 foot•lb when attaching extruded panels to posts.

Installations shall be in accordance with the contract and details shown on the plans. Repair damaged galvanized surfaces in accordance with 2471, “Structural Metals.”

The Engineer will measure install I-Beam sign as a complete unit, including new lower posts, salvaged upper posts, new friction fuse, salvaged or Department provided extruded panel, and new mounting hardware.

### Install Monotube Sign

Install salvaged monotube structure on new concrete footings in accordance with the torque requirements specified under Structural Steel and as required by the contract.

Install salvaged sign panels and stringers to the monotube structure with new mounting hardware.

The Engineer will measure install monotube sign as a complete unit, including installing salvaged structure, sign panels and stringers with new mounting hardware.

### Install Overhead Sign

Use this provision when installing a salvaged overhead sign structure on a new footing

For Sign(s) OH XX-XX,

Install salvaged overhead sign structure on new concrete footings. Obtain Engineer’s approval of the wrench used for tightening the anchor rod nuts and provide proof of calibration performed within the last 12 months from an accredited calibration service before installing the post. Tighten anchor rods in accordance with the MnDOT Anchor Rod Tightening Handbook- New Installation Procedures. Complete a MnDOT Anchor Rod Tightening Form and submit the completed form to the Engineer.

Install salvaged extruded panels and panel mounting posts with new post clips as required by the contract.

Torque each post clip to between 12 foot •lb and 14 foot•lb when attaching extruded panels to panel mounting posts.

If present, remove old and install new safety chains and components of safety chain snap assemblies, safety rail locking pins, and chains welded to the locking pin heads (one chain/locking pin).

Dispose of removed items in accordance with 2104, “Removing Pavement and Miscellaneous Structures.”

Verify that each locking pin fits in the locking pin hole in the handrail hinge while the handrail is in the raised position.

Drill out the handrail hinge to make a proper fit for locking pins that do not fit.

Repair damaged galvanized surfaces in accordance with 2471.3L.1, “Galvanizing.”

The Engineer will measure install overhead sign as a complete unit, including installing salvaged structure, extruded panels, and panel mounting posts with new post clips, new safety chains, components of safety chain snap assemblies, safety rail locking pins, and chains welded to the locking pin heads.

### Install Overhead Bridge Mounted Sign

Install salvaged extruded panels with new post clips on new bridge mount sign structures as required by the contract.

Torque each post clip to between 12 foot •lb and 14 foot•lb when attaching extruded panels to posts.

If present, remove old and install new safety chains and components of safety chain snap assemblies, safety rail locking pins, and chains welded to the locking pin heads (one chain/locking pin).

Dispose of removed items in accordance with 2104, “Removing Pavement and Miscellaneous Structures.”

Verify that each locking pin fits in the locking pin hole in the handrail hinge while the handrail is in the raised position.

Drill out the handrail hinge to make a proper fit for locking pins that do not fit. Repair damaged galvanized surfaces in accordance with 2471.3L.1, “Galvanizing.”

The Engineer will measure install overhead bridge mounted sign as a complete unit, including new bridge mount structure, installing salvaged extruded panels using new post clips, new safety chains, components of safety chain snap assemblies, safety rail locking pins, and chains welded to the locking pin heads.

### Install Sign Type Special

Use if installing salvaged street name sign.

Install new base post.  Install salvaged riser post (if square tube) or new riser post.  Install the salvaged bracket assembly with street name plates attached.

The Engineer will measure install sign type special as a complete unit, including new base post, new riser post or salvaged riser post, and salvaged bracket with street name plates attached.

### Delineator/Marker

Provide delineator/marker panel as required by the contract.

Provide and install the post and attach the delineator/marker panel with new mounting hardware required by the contract.

Affix a Department-provided warning sticker to the backside of each marker panel directly above the fabrication sticker. Warning stickers are available at the Department's Transportation District Office specified in the Contract. Provide thirty calendar days advance notice before picking up the stickers.

The Engineer will measure delineator/marker as a complete unit, including new post, new mounting hardware, new delineator/marker panel, and warning sticker.

### Delineator/Marker Panel

Provide delineator/marker panel as required by the contract.

Attach the delineator/marker panel with new mounting hardware required by the contract.

Affix a Department-provided warning sticker to the backside of each marker panel directly above the fabrication sticker. Warning stickers are available at the Department's Transportation District Office specified in the Contract. Provide thirty calendar days advance notice before picking up the stickers.

The Engineer will measure delineator/marker panel as a complete unit, including new delineator/marker panel and new mounting hardware.

### Median Barrier Footing (Shaft)

Coordinate placement of the project’s foundation report on the MnDOT Foundations Reports website with Rich Lamb at (651) 366-5595 or rich.lamb@state.mn.us

Obtain and review the soils information and foundation recommendation(s) found within the S.P. XXXX-XXX foundations report located on the MnDOT Foundations Unit’s Foundations Reports website:

http://www.dot.state.mn.us/materials/borings.html

- Excavate for footing and construct footing as shown in the contract. After footing has cured, backfill to proposed ground elevation. Replace any removed pavement and dispose of excess excavation.

Construct footings as required by the contract and in accordance with 2401.3, “Concrete Bridge Construction: Construction Requirements.”  The Department will only pay for the planned design quantity for each footing at the contract unit price.

- If the shaft embedment depth cannot be achieved, the Contractor shall coordinate with MnDOT Foundations Unit and the Engineer to determine a suitable alternative footing.  Concrete footing will be paid in accordance with 1402.2: Differing Site Conditions.

- The Contractor may use undisturbed earth to form concrete placement if the soil is stable enough to allow concrete placement as determined by the Engineer and the Contractor takes precautions to prevent contamination of the concrete.

- Provide non-metallic conduit and fittings to connect to the non-metallic conduit in adjacent median barrier as detailed in the contract.

- Mark both ends of anchor rods in the anchor rod assembly in accordance with ASTM F1554 supplementary requirement S3.  The markings required are die stamped grade identification and color.  The color shall be:

Blue for Grade 36

Yellow for Grade 55

Red for Grade 105.

- Provide a surface finish and color that matches the adjacent median barrier.

- Install sign posts on concrete footings after the concrete has cured for a minimum of seven days.

The Engineer will measure median barrier footing (shaft) by the cubic yard of concrete based on the dimensions required by the contract, including footing construction, metal reinforcement, anchorage assemblies, conduit, excavation, and backfill.

### Median Barrier footing (Spread)

Coordinate placement of the project’s foundation report on the MnDOT Foundations Reports website with Rich Lamb at (651) 366-5595 or rich.lamb@state.mn.us

Obtain and review the soils information and foundation recommendation(s) found within the S.P. XXXX-XXX foundations report located on the MnDOT Foundations Unit’s Foundations Reports website:

http://www.dot.state.mn.us/materials/borings.html

- Excavate for footing and construct footing as shown in the contract. After footing has cured, backfill to proposed ground elevation. Replace any removed pavement and dispose of excess excavation.

Construct footings as required by the contract and in accordance with 2401.3, “Concrete Bridge Construction: Construction Requirements.”

- The Contractor may use undisturbed earth to form concrete placement if the soil is stable enough to allow concrete placement as determined by the Engineer and the Contractor takes precautions to prevent contamination of the concrete.

- Provide non-metallic conduit and fittings to connect to the non-metallic conduit in adjacent median barrier as detailed in the contract.

- Mark both ends of anchor rods in the anchor rod assembly in accordance with ASTM F1554 supplementary requirement S3.  The markings required are die stamped grade identification and color.  The color shall be:

Blue for Grade 36

Yellow for Grade 55

Red for Grade 105.

- Provide a surface finish and color that matches the adjacent median barrier.

- Install sign posts on concrete footings after the concrete has cured for a minimum of seven days.

The Engineer will measure median barrier footing (spread) by the cubic yard of concrete based on the dimensions required by the contract, including footing construction, metal reinforcement, anchorage assemblies, conduit, excavation, and backfill.

### Concrete Footing (I-Beam)

Coordinate placement of the project’s foundation report on the MnDOT Foundations Reports website with Rich Lamb at (651) 366-5595 or rich.lamb@state.mn.us

Obtain and review the soils information and foundation recommendation(s) found within the S.P. XXXX-XXX foundations report located on the MnDOT Foundations Unit’s Foundations Reports website:

http://www.dot.state.mn.us/materials/borings.html

- Excavate for footing and construct footing as shown in the contract. After footing has cured, backfill to proposed ground elevation. Replace any removed pavement and dispose of excess excavation.

Construct footings as required by the contract and in accordance with 2401.3, “Concrete Bridge Construction: Construction Requirements.”  The Department will only pay for the planned design quantity for each footing at the contract unit price.

- If the shaft embedment depth cannot be achieved, the Contractor shall coordinate with MnDOT Foundations Unit and the Engineer to determine a suitable alternative footing.  Concrete footing will be paid in accordance with 1402.2: Differing Site Conditions.

- The Contractor may use undisturbed earth to form concrete placement if the soil is stable enough to allow concrete placement as determined by the Engineer and the Contractor takes precautions to prevent contamination of the concrete.

- Provide a rubbed-surface finish on exposed concrete surfaces.

- Install sign posts on concrete footings after the concrete has cured for a minimum of seven days.

The Engineer will measure concrete footing (I-Beam) by the cubic yard of concrete based on the dimensions required by the contract, including footing construction, metal reinforcement, anchorage assemblies, excavation, and backfill.

### Concrete Footing (Overhead Shaft)

Coordinate placement of the project’s foundation report on the MnDOT Foundations Reports website with Rich Lamb at (651) 366-5595 or rich.lamb@state.mn.us

Obtain and review the soils information and foundation recommendation(s) found within the S.P. XXXX-XXX foundations report located on the MnDOT Foundations Unit’s Foundations Reports website:

http://www.dot.state.mn.us/materials/borings.html

- Excavate for footing and construct footing as shown in the contract. After footing has cured, backfill to proposed ground elevation. Replace any removed pavement and dispose of excess excavation.

Construct footings as required by the contract and in accordance with 2401.3, “Concrete Bridge Construction: Construction Requirements.”  The Department will only pay for the planned design quantity for each footing at the contract unit price.

- If the shaft embedment depth cannot be achieved, the Contractor shall coordinate with MnDOT Foundations Unit and the Engineer to determine a suitable alternative footing.  Concrete footing will be paid in accordance with 1402.2: Differing Site Conditions.

The Contractor may submit an alternate design for approval by the Engineer with the understanding that, if approved, the Department will not adjust the contract quantities or unit prices. If the Engineer approves the alternate design, the Contractor may proceed.

Provide design details for alternate drilled shaft footing designs in accordance with any one of the following:

(1)	Use a constant diameter shaft at least 6 inches greater than the diagonal dimension of the column base plate. Use the planned longitudinal reinforcement bars without bending. Use either spiral reinforcement or tie bars spaced at 6 inch centers for the full length of the shaft.

(2)	Install a horizontal construction joint at the bottom of the tapered section of the shaft about 6 feet below the top. Before placing new concrete above the construction joint, coat the surface of the in-place concrete with a Department-approved bonding agent. Lap vertical reinforcement bars a length equal to 40 times the diameter of the reinforcement bar used.

- The Contractor may use undisturbed earth to form concrete placement if the soil is stable enough to allow concrete placement as determined by the Engineer and the Contractor takes precautions to prevent contamination of the concrete.

- Mark both ends of anchor rods in the anchor rod assembly in accordance with ASTM F1554 supplementary requirement S3.  The markings required are die stamped grade identification and color.  The color shall be:

Blue for Grade 36

Yellow for Grade 55

Red for Grade 105.

- Provide a rubbed-surface finish on exposed concrete surfaces.

- Install sign posts on concrete footings after the concrete has cured for a minimum of seven days.

The Engineer will measure concrete footing (overhead shaft) by the cubic yard of concrete based on the dimensions required by the contract, including footing construction, metal reinforcement, anchorage assemblies, excavation, and backfill.

### Concrete Footing (Overhead Spread)

Coordinate placement of the project’s foundation report on the MnDOT Foundations Reports website with Rich Lamb at (651) 366-5595 or rich.lamb@state.mn.us

Obtain and review the soils information and foundation recommendation(s) found within the S.P. XXXX-XXX foundations report located on the MnDOT Foundations Unit’s Foundations Reports website:

http://www.dot.state.mn.us/materials/borings.html

- Excavate for footing and construct footing as shown in the contract. After footing has cured, backfill to proposed ground elevation. Replace any removed pavement and dispose of excess excavation.

Construct footings as required by the contract and in accordance with 2401.3, “Concrete Bridge Construction: Construction Requirements.”

- The Contractor may use undisturbed earth to form concrete placement if the soil is stable enough to allow concrete placement as determined by the Engineer and the Contractor takes precautions to prevent contamination of the concrete.

- Mark both ends of anchor rods in the anchor rod assembly in accordance with ASTM F1554 supplementary requirement S3.  The markings required are die stamped grade identification and color.  The color shall be:

Blue for Grade 36

Yellow for Grade 55

Red for Grade 105.

- Provide a rubbed-surface finish on exposed concrete surfaces.

- Install sign posts on concrete footings after the concrete has cured for a minimum of seven days.

The Engineer will measure concrete footing (overhead spread) by the cubic yard of concrete based on the dimensions required by the contract, including footing construction, metal reinforcement, anchorage assemblies, excavation, and backfill.

### Concrete Footing (Monotube)

Coordinate placement of the project’s foundation report on the MnDOT Foundations Reports website with Rich Lamb at (651) 366-5595 or rich.lamb@state.mn.us

Obtain and review the soils information and foundation recommendation(s) found within the S.P. XXXX-XXX foundations report located on the MnDOT Foundations Unit’s Foundations Reports website:

http://www.dot.state.mn.us/materials/borings.html

- Excavate for footing and construct footing as shown in the contract. After footing has cured, backfill to proposed ground elevation. Replace any removed pavement and dispose of excess excavation.

Construct footings as required by the contract and in accordance with 2401.3, “Concrete Bridge Construction: Construction Requirements.”  The Department will only pay for the planned design quantity for each footing at the contract unit price.

- If the shaft embedment depth cannot be achieved, the Contractor shall coordinate with MnDOT Foundations Unit and the Engineer to determine a suitable alternative footing.  Concrete footing will be paid in accordance with 1402.2: Differing Site Conditions.

- The Contractor may use undisturbed earth to form concrete placement if the soil is stable enough to allow concrete placement as determined by the Engineer and the Contractor takes precautions to prevent contamination of the concrete.

- Mark both ends of anchor rods in the anchor rod assembly in accordance with ASTM F1554 supplementary requirement S3.  The markings required are die stamped grade identification and color.  The color shall be:

Blue for Grade 36

Yellow for Grade 55

Red for Grade 105.

- Provide a rubbed-surface finish on exposed concrete surfaces.

- Install sign posts on concrete footings after the concrete has cured for a minimum of seven days.

The Engineer will measure concrete footing (monotube) by the cubic yard of concrete based on the dimensions required by the contract, including footing construction, metal reinforcement, anchorage assemblies, excavation, and backfill.

### Structural Steel – I-Beam Sign

The provisions of 2564.3E, “Traffic Signs and Devices: Construction Requirements: Structural Steel” are replaced as follows:

Manufacture and fabricate structural steel in accordance with 2471, “Structural Metals,” and the additional requirements and limitations specified in this subsection (2564.3.E).

Provide shop drawings for I-beam supported sign structures in accordance with 2471.3.B, “Shop Detail Drawings.”

Unless otherwise required by the contract, provide and install galvanized structural steel footing posts for I-beam supported signs in accordance with 2452, “Piling,” 2471, “Structural Metals,” and the following:

(1)	Construct footings as required by the contract;

(2)	Install footing posts to the minimum embedment depth specified in the Plans.  If the minimum embedment cannot be achieved, contact the Engineer for alternate foundation details.

(3)	Repair damage to galvanized surfaces in accordance with 2471, “Structural Metals,” before back filling.

The Engineer will measure structural steel – I-beam sign separately by the computed weight of structural steel incorporated in accordance with the measurement provisions of 2402.4A, “Weight.”  The Engineer will not measure bolts, nuts, rivets, washers, and shims used in the fabrication and erection of signs, and will not apply the provisions of 2402.4A, “Weight,” that provide a percent increase in weight.

The Engineer will measure structural steel – I-beam sign by the pound, including providing and erecting posts, keeper plate, friction fuse, and H-piles. The Engineer will base the computed weight on the quantity tables in the contract.

### Structural Steel – Overhead Sign Post

The provisions of 2564.3E, “Traffic Signs and Devices: Construction Requirements: Structural Steel” are replaced as follows:

Manufacture and fabricate structural steel in accordance with 2471, “Structural Metals,” and the additional requirements and limitations specified in this subsection (2564.3.E).

Provide shop drawings for overhead sign structures in accordance with 2471.3.B, “Shop Detail Drawings.”

Assemble the posts in the shop before galvanizing.  Check posts for straightness, alignment and dimensions and correct any variations.  Correct warpage from galvanizing before installing structural steel. Correct warpage that will interfere with fit-up from hot-dip galvanizing before installing structural steel.

Drill or mechanically cut overhead sign post base plate anchor rod holes.

Obtain Engineer’s approval of the wrench used fro tightening the anchor rod nuts and provide proof of calibration performed within the last 12 months from an accredited calibration service before installing the post.

Install posts for overhead signs and tighten anchor rods in accordance with the MnDOT Anchor Rod Tightening Handbook- New Installation.

Complete a MnDOT Anchor Rod Tightening Form for each overhead sign structure. Submit the completed forms to the Engineer.

The Engineer will measure structural steel – overhead sign post separately by the computed weight of structural steel incorporated in accordance with the measurement provisions of 2402.4A, “Weight.”  The Engineer will not measure bolts, nuts, rivets, washers, and shims used in the fabrication and erection of signs, and will not apply the provisions of 2402.4A, “Weight,” that provide a percent increase in weight.

The Engineer will measure structural steel – overhead sign post by the pound, including providing and erecting posts, truss connection plates, baseplate, and overhead sign identification plate. The Engineer will base the computed weight on the quantity tables in the contract.

### Structural Steel – Overhead Sign Truss

The provisions of 2564.3E, “Traffic Signs and Devices: Construction Requirements: Structural Steel” are replaced as follows:

Manufacture and fabricate structural steel in accordance with 2471, “Structural Metals,” and the additional requirements and limitations specified in this subsection (2564.3.E).

Provide shop drawings for overhead sign structures in accordance with 2471.3.B, “Shop Detail Drawings.”

Assemble the truss sections in the shop before galvanizing.  Check truss sections for straightness, alignment and dimensions and correct any variations.  Correct warpage from galvanizing before installing structural steel. Correct warpage that will interfere with fit-up from hot-dip galvanizing before installing structural steel.

Ensure main chord angles for overhead sign structures that are at least ½ in thick, meet a Charpy V-notch impact strength requirement of 15 ft•lb at 40 °F.

The Engineer will measure structural steel – overhead sign truss separately by the computed weight of structural steel incorporated in accordance with the measurement provisions of 2402.4A, “Weight.”  The Engineer will not measure bolts, nuts, rivets, washers, and shims used in the fabrication and erection of signs, and will not apply the provisions of 2402.4A, “Weight,” that provide a percent increase in weight.

The Engineer will measure structural steel – overhead sign truss by the pound, including providing and erecting truss members. The Engineer will base the computed weight on the quantity tables in the contract.

### Structural Steel – Overhead Walkway

The provisions of 2564.3E, “Traffic Signs and Devices: Construction Requirements: Structural Steel” are replaced as follows:

Manufacture and fabricate structural steel in accordance with 2471, “Structural Metals,” and the additional requirements and limitations specified in this subsection (2564.3.E).

Provide shop drawings for overhead sign structures in accordance with 2471.3.B, “Shop Detail Drawings.”

The Engineer will measure structural steel – overhead walkway separately by the computed weight of structural steel incorporated in accordance with the measurement provisions of 2402.4A, “Weight.”  The Engineer will not measure bolts, nuts, rivets, washers, and shims used in the fabrication and erection of signs, and will not apply the provisions of 2402.4A, “Weight,” that provide a percent increase in weight.

The Engineer will measure structural steel – overhead walkway by the pound, including providing and erecting walkway supports and grating. The Engineer will base the computed weight on the quantity tables in the contract.

### Structural Steel – Monotube Sign

The provisions of 2564.3E, “Traffic Signs and Devices: Construction Requirements: Structural Steel” are replaced as follows:

Manufacture and fabricate structural steel in accordance with 2471, “Structural Metals,” and the additional requirements and limitations specified in this subsection (2564.3.E).

Provide shop drawings for monotube sign structures in accordance with 2471.3.B, “Shop Detail Drawings.”

Assemble the posts and beams in the shop before galvanizing.  Check posts and beams for straightness, alignment and dimensions and correct any variations.  Correct warpage from galvanizing before installing structural steel. Correct warpage that will interfere with fit-up from hot-dip galvanizing before installing structural steel.

Drill or mechanically cut monotube sign post base plate anchor rod holes.

Obtain Engineer’s approval of the wrench used fro tightening the anchor rod nuts and provide proof of calibration performed within the last 12 months from an accredited calibration service before installing the post.

Install posts for overhead signs and tighten anchor rods in accordance with the MnDOT Anchor Rod Tightening Handbook- New Installation.

Complete a MnDOT Anchor Rod Tightening Form for each overhead sign structure. Submit the completed forms to the Engineer.

The Engineer will measure structural steel – monotube sign separately by the computed weight of structural steel incorporated in accordance with the measurement provisions of 2402.4A, “Weight.”  The Engineer will not measure bolts, nuts, rivets, washers, and shims used in the fabrication and erection of signs, and will not apply the provisions of 2402.4A, “Weight,” that provide a percent increase in weight.

The Engineer will measure structural steel – monotube sign by the pound, including providing and erecting posts, post-beam connection plates, baseplate, and overhead sign identification plate. The Engineer will base the computed weight on the quantity tables in the contract.

### Structural Steel – Bridge Mounted Truss

The provisions of 2564.3E, “Traffic Signs and Devices: Construction Requirements: Structural Steel” are replaced as follows:

Manufacture and fabricate structural steel in accordance with 2471, “Structural Metals,” and the additional requirements and limitations specified in this subsection (2564.3.E).

Provide shop drawings for bridge mounted truss structures in accordance with 2471.3.B, “Shop Detail Drawings.”

Assemble the truss sections in the shop before galvanizing.  Check truss sections for straightness, alignment and dimensions and correct any variations.  Correct warpage from galvanizing before installing structural steel. Correct warpage that will interfere with fit-up from hot-dip galvanizing before installing structural steel.

Ensure main chord angles for bridge mounted truss structures that are at least ½ in thick, meet a Charpy V-notch impact strength requirement of 15 ft•lb at 40 °F.

The Engineer will measure structural steel – bridge mounted truss separately by the computed weight of structural steel incorporated in accordance with the measurement provisions of 2402.4A, “Weight.”  The Engineer will not measure bolts, nuts, rivets, washers, and shims used in the fabrication and erection of signs, and will not apply the provisions of 2402.4A, “Weight,” that provide a percent increase in weight.

The Engineer will measure structural steel – bridge mounted truss by the pound, including providing and erecting truss members, and fixture mounting channels. The Engineer will base the computed weight on the quantity tables in the contract.

### Structural Steel – Panel Mounting Post

The provisions of 2564.3E, “Traffic Signs and Devices: Construction Requirements: Structural Steel” are replaced as follows:

Manufacture and fabricate structural steel in accordance with 2471, “Structural Metals,” and the additional requirements and limitations specified in this subsection (2564.3.E).

Check panel mounting post for straightness, alignment and dimensions and correct any variations.  Correct warpage from galvanizing before installing structural steel.

The Engineer will measure structural steel – panel mounting post separately by the computed weight of structural steel incorporated in accordance with the measurement provisions of 2402.4A, “Weight.”  The Engineer will not measure bolts, nuts, rivets, washers, and shims used in the fabrication and erection of signs, and will not apply the provisions of 2402.4A, “Weight,” that provide a percent increase in weight.

The Engineer will measure structural steel – panel mounting post by the pound, including providing and erecting panel mounting posts. The Engineer will base the computed weight on the quantity tables in the contract.

### Sign

Fabricate the sign panels in accordance with 3352, “Signs.”

Package, deliver, store, and install sign panels in accordance with 1607, “Handling Materials,” 3352, “Signs,” and the retroreflective sheeting manufacturer's recommendations.

Install new post in accordance with the plans and attach new sign panels with new stringers as needed and mounting hardware.

Affix a Department-provided warning sticker to the backside of each sign panel directly above the fabrication sticker. Warning stickers are available at the Department's Transportation District Office specified in the Contract. Give the Transportation District’s contact person thirty calendar days advance notice before picking up the stickers.

The Engineer will measure sign by square foot based on the nominal dimensions of the sign panels. Sign panels are considered rectangular for the purpose of measurement except that the Engineer will measure triangular shaped sign panels as the actual area of the triangle. The Engineer will not make deductions for rounded corners.  Sign includes new sign panels, new posts, new stringers, new mounting hardware, and installing Department provided warning sticker.

### Sign Panel

Fabricate the sign panel in accordance with 3352, “Signs.”

Package, deliver, store, and install sign panels in accordance with 1607, “Handling Materials,” 3352, “Signs,” and the retroreflective sheeting manufacturer's recommendations.

Attach new panel to support structure using new stringers as needed and mounting hardware.

Affix a Department-provided warning sticker to the backside of each sign panel directly above the fabrication sticker. Warning stickers are available at the Department's Transportation District Office specified in the Contract. Give the Transportation District’s contact person thirty calendar days advance notice before picking up the stickers.

The Engineer will measure sign panel by square foot based on the nominal dimensions of the sign panel. Sign panel is considered rectangular for the purpose of measurement except that the Engineer will measure a triangular shaped sign panel as the actual area of the triangle. The Engineer will not make deductions for rounded corners.  Sign panel includes new sign panel, new stringers, new mounting hardware, and installing Department provided warning sticker.

### Sign Panel Overlay

Fabricate the sign panels in accordance with 3352, “Signs.”

Package, deliver, store, and install sign panels in accordance with 1607, “Handling Materials,” 3352, “Signs,” and the retroreflective sheeting manufacturer's recommendations.

Remove any inplace sign panel overlays and dispose of them in accordance with 2104, “Removing Pavement and Miscellaneous Structures.”

Attach the sign panel overlay to the extruded aluminum panels or sign panels with 3/16 inch stainless steel rivets.

Tightly butt the sign panel overlays vertically and rivet to the extruded panels on 12 inch vertical and horizontal centers. Rivet the edges and corners of each sign panel overlay. Do not place rivets within 1 inch of the extruded panel joints. Sign panel overlays must be flat after attaching to extruded panels.

If extruded panel is detached from posts to perform the overlay work, reattach extruded panels to sign posts or panel mounting posts with new post clips and torque each post clip between 12 foot•lb and 14 foot•lb.

Use this provision to add graffiti film to sign panel overlay

For Sign Panel Overlays indicated in tabulations of the plans,

Apply the graffiti film to the entire sign panel overlay in the shop and according to the manufacturer’s specifications.

The Engineer will measure sign panel overlay by the square foot based on the nominal dimensions of the sign panel. Signs are considered rectangular for the purpose of measurement except that the Engineer will measure triangular shaped sign panels as the actual area of the triangle. The Engineer will not make deductions for rounded corners. Sign panel overlay includes removing any existing sign panel overlays, new mounting hardware, and providing and attaching new sign panel overlay.

The one area where I modified the provisions was for the overlay section.  Again, thought it looked good.  However for historical reasons I thought it might be prudent to add in some of the current language from our specbook provisions for Sign Panel Overlay Type _, particularly in relation to the sheeting material, to supply new post clips for overlays of existing extruded panels, and from DIV ST to remove inplace overlays:

### Extruded Panel

Fabricate the extruded panel in accordance with 3352, “Signs.”

Attach extruded panels to sign posts or panel mounting posts with new post clips and torque each post clip between 12 foot•lb and 14 foot•lb.

The Engineer will measure extruded panel by square foot, including new extruded aluminum panels bolted together and attached to the sign structure with new mounting hardware.

### Sign Type Special

Use for new street signs.

Fabricate the street sign panels in accordance with 3352, “Signs.”

Package, deliver, store, and install street sign panels in accordance with 1607, “Handling Materials,” 3352, “Signs,” and the retroreflective sheeting manufacturer's recommendations.

Install new post in accordance with the plans and attach new street sign panels with new brackets and mounting hardware.

The Engineer will measure sign type special by square foot based on the nominal dimensions of the street sign panels. Sign panels are considered rectangular for the purpose of measurement except that the Engineer will measure triangular shaped sign panels as the actual area of the triangle. The Engineer will not make deductions for rounded corners.  Sign type special includes new street sign panels, new post, new brackets, and new mounting hardware.

## METHOD OF MEASUREMENT AND BASIS OF PAYMENT

The Engineer will measure each item according to the Contract and the 2564, “Traffic Signs and Devices: Construction Requirements” section of these Special Provisions.

The Department will include all work described in the Contract and the 2564, “Traffic Signs and Devices: Construction Requirements” section of these Special Provisions as part of the contract unit price per unit of measure.

The Department will pay for traffic signs and devices on the basis of the following schedule:

Include only pay items that contain work specified in these special provisions.

| Item No.: | Item: | Unit: |
| --- | --- | --- |
| 2564.502 | Modify Post | Each |
| 2564.502 | Install Extruded Sign Panel | Each |
| 2564.502 | Install Sign Panel | Each |
| 2564.502 | Install Sign Panel Type Special | Each |
| 2564.502 | Extend Walkway Support | Each |
| 2564.502 | Friction Fuse | Each |
| 2564.502 | Keeper Plate | Each |
| 2564.502 | Delineator Type Special | Each |
| 2564.510 | Overhead Sign Structure Repair | Hour |
| 2564.518 | Sign Panels Type Special | Square Foot |
| 2564.602 | Install Sign Collar | Each |
| 2564.602 | Sign Collar | Each |
| 2564.602 | Install Sign | Each |
| 2564.602 | Install Delineator/Marker | Each |
| 2564.602 | Foundation Special OH | Each |
| 2564.602 | Repair Sign Type OH | Each |
| 2564.602 | Reposition Sign Panel | Each |
| 2564.602 | Install I-Beam Sign | Each |
| 2564.602 | Install Monotube Sign | Each |
| 2564.602 | Install Overhead Sign | Each |
| 2564.602 | Install Overhead Bridge Mounted Sign | Each |
| 2564.602 | Install Sign Type Special | Each |
| 2564.602 | Delineator/Marker | Each |
| 2564.602 | Delineator/Marker Panel | Each |
| 2564.607 | Median Barrier Footing (Shaft) | Cubic Yard |
| 2564.607 | Median Barrier Footing (Spread) | Cubic Yard |
| 2564.607 | Concrete Footing (I-Beam) | Cubic Yard |
| 2564.607 | Concrete Footing (Overhead Shaft) | Cubic Yard |
| 2564.607 | Concrete Footing (Overhead Spread) | Cubic Yard |
| 2564.607 | Concrete Footing (Monotube) | Cubic Yard |
| 2564.608 | Structural Steel – I-Beam Sign | Pound |
| 2564.608 | Structural Steel – Overhead Sign Post | Pound |
| 2564.608 | Structural Steel – Overhead Sign Truss | Pound |
| 2564.608 | Structural Steel – Overhead Walkway | Pound |
| 2564.608 | Structural Steel – Monotube Sign | Pound |
| 2564.608 | Structural Steel – Bridge Mounted Truss | Pound |
| 2564.608 | Structural Steel – Panel Mounting Post | Pound |
| 2564.618 | Sign | Square Foot |
| 2564.618 | Sign Panel | Square Foot |
| 2564.618 | Sign Panel Overlay | Square Foot |
| 2564.618 | Extruded Panel | Square Foot |
| 2564.618 | Sign Type Special | Square Foot |

The Department’s payment for each item shall be compensation in full for all work, material, and costs involved in performing the work specified on the Plans and these Special Provisions.

# (3352) SIGNS, DELINEATORS, AND MARKERS

## DESCRIPTION

The Contractor shall furnish signs, delineators, and markers in accordance with 3352, “Signs, Delineators, and Markers,” except as modified in these Special Provisions.

### Aluminum Stringers

Add under section 3352.2:

A.7  Aluminum Stringers

Use extruded aluminum alloy 6061-T6 with mill finished surface for stringers used to mount sign panels to square tube posts.

### Stainless Steel Clamp

Add under section3352.2:

A.8  Stainless Steel Clamps

To clamp stringers to square tube posts use 11 gauge Type 304, #2B finished stainless steel with 3/8"-16 x 2" carriage bolt & serrated flange nut.

### Stainless Steel bolts

Add under section 3352.2:

A.6.c  Stainless Steel Bolts

Use stainless steel bolts as specified in 3391.2E, “Fasteners: Requirements: Stainless Steel Bolts,” with zinc coated steel nylon insert lock nuts. When used to attach sign panels place a stainless steel washer and nylon washer on the sign sheeting surface.

### Galvanized Steel Screw Anchor Bolts

Add under section 3352.2:

A.6.d  Galvanized Steel Screw Anchor Bolts

Use galvanized screw anchor bolts as specified in plans. Galvanize screw ancho bolts in accordance with 3392, “Galvanized Hardware.”

# (3402) SQUARE TUBULAR SIGN POSTS

## DESCRIPTION

The Contractor shall furnish square tubular sign posts in accordance with 3402, “Square Tubular Sign Posts,” except as modified in these Special Provisions.

### Riser post for Square-Tube Three-Wall Sign Base

1 ¾ inch by 1 ¾ inch square tube posts shall be 14 gauge (1.71 lbs/ft).