G&B-408

09/08/2021

SFDR/CIR/CCPR Mix Design Form

| SFDR/CIR/CCPR Table of Design Parameters |  |  |
| --- | --- | --- |
| Parameter | Attribute | Required |
| Method of bituminous delivery | Foaming or emulsion | Always |
| Type & grade of Emulsion or PG grade of asphalt binder Note: for Foamed Asphalt Minimum Expansion Ratio: 8 Minimum Half-Life: 6 Seconds | Place emulsion type (for example 3151.2.I.1 Soft Base Engineered Emulsion) or PG grade (for foaming) here | Always |
| Recommended Foaming Temperature | Optimum Foaming temperature | Always, when using foamed asphalt |
| Additional Additives | Specify and add rate | Sometimes |
| Target amount of Bituminous Material for Mixture | Place percentage amount here * Note this is for bidding purposes, rate is adjusted in the field based upon conditions | Always |
| Amount and type of Add-Rock | Provide aggregate meeting specification 3127 FA-2 ½ or FA-3 at X rate (pounds/sy) | Sometimes |
| Amount and type of mineral stabilizing agent | Designate cement or lime and rate (pounds/sy) here | Sometimes |
| Stabilizing Depth | Inches of Stabilizing | Always |
| Estimated Optimum moisture Content | To the tenth of a %, x.x% | Always |
| Note: Report % Passing used for mix designs. Report sieves 1.5”, 1.25”, 1”, ¾”, 3/8”, #4, #10 & #30. |  |  |

Designer’s Note: fill in Table and provide in Contract documents.