2021 Initial Idea Development

2021-2022 NRRA Research and MnROAD Construction Development

(Updated Form – March 23, 2021)

Initial Proposal is for NRRA Executive Team to Approve for further development (keep to two pages)

| Research Title: | Alternative Dowel Bars |
| --- | --- |
| NRRA Team(s): | Rigid |
| Type of Effort: | Research |
| Developed By: | John Donahue and Brett Trautman-MoDOT, Matt Offenburg-OwensCorning, Jake Hiller- MichTech, Tom Burnham-MnDOT |
| Email: | tom.burnham@state.mn.us |
| Phone: |  |

| Research Funding Estimate: | $150,000 |
| --- | --- |
| Research Years Expected: | 3 |
| Beneficial Partnerships: | Potential dowel bar donations from manufacturers/suppliers |

| Number of Test Sections: | Six 180 foot sections, plus installation in other new concrete test sections |
| --- | --- |
| Instrumentation Effort: | High: Joint opening, dowel bar and concrete strain, VW, TC |
| MnROAD Monitoring: | Frequent FWD testing, seasonal IRI, distress surveys, load testing, faultmeter, MIRA |

Research Objectives:

Concrete pavements will only remain sustainable if their components can survive for long periods. Until recently, standard epoxy coated dowel bars often became notably distressed in freeze/thaw climates in as little as 15 years. To address this durability issue, several alternative types of dowel bars have been introduced to the market in past decade. Agencies and manufacturers have struggled to find cost effective ways to approve such dowel bars. A recent ACPA study is wrapping up soon that developed new guidelines for dowel bar acceptance tests for alternative dowel bars (AASHTO M254 & T253). Those guidelines were developed via laboratory testing of a matrix of dowel bars types and configurations. This NRRA research project proposes to construct new or equip existing MnROAD test sections with alternative dowel bar types and configurations that could be used for field validation of the ACPA laboratory testing and modeling results. This would then allow for refinement of the guidelines toward adoption of new AASHTO standards (followed by many NRRA member states). Consideration would be given (but not limited) to innovative ideas like: - Replicate at least a portion of the ACPA lab testing matrix to provide field validation of results. - Using larger diameter dowel bars to reduce bearing stresses from both traffic and curl/warping forces. - Study the behavior of retrofit dowel bars, where installation of sensors could be highly controlled compared to installation before slipform paving. - Use full-depth saw-cut joints to eliminate aggregate interlock (thus accelerating potential distresses). - Extensive FWD load transfer testing to fully understand the behavior of standard and alternative dowels.

Pavement Test Cells Needed:

A minimum of six 180 foot long dedicated test sections, along with additional configurations incorporated into other new NRRA proposed test sections (i.e. for new material studies).

NRRA Sustainability/Resiliency and or Intelligent Construction:

Components that extend the service life of concrete pavements, like corrosion resistant dowel bars, clearly meet the criteria of sustainability. If an alternative dowel bar material can be manufactured using a method that generates less greenhouse emissions, that too contributes to sustainability.

Cross-cutting Opportunities:

There is a strong possibility of incorporating alternative dowel bars in concrete pavement test sections related to other NRRA studies (assuming they do not confound the objective of a particular study).

Implementation Plan:

Findings from this study could be influential in providing comfort to agencies when determining whether to accept alternative dowel bar types onto their approved products lists. It could also provide further support for adoption of new AASHTO standards for dowel bars. Implementation would involve dissemination of the information in the form of a tech brief and final report posted on the NRRA Team webpage(s), as well as presentations at NRRA and other pavement related conferences.