DIVISION ST

Section	P

No.	Item

ST-1	(2104) REMOVING MISCELLANEOUS STRUCTURES	1

ST-2	(2564) TRAFFIC SIGNS AND DEVICES	3

ST-3	(3352) SIGNS, DELINEATORS, AND MARKERS	10

ST-4	(3402) SQUARE TUBULAR SIGN POSTS	11

DELETE WHEN DONE: To Update Table of Contents Right Click on the index above and choose Update Field.  If given a choice choose “Update Entire Table”.

DELETE WHEN DONE: Statements highlighted in yellow are guidelines or instructional in nature.  Remove these notes before completing the spec.  When appearing at the top left of a provision, it pertains to the paragraph immediately beneath the note, as well as any indented items following it.

DELETE WHEN DONE: Words highlighted in green are fields that may need to be modified or removed before completion of the spec, such as contact information, city names, sign numbers, or charts on the Plans.

For provisions containing the descriptor, “For Sign(s) [Type]-XX:”, this descriptor should only be used when the provision applies to specific sign numbers among that type on the Plans.  Insert as necessary.  Leave out the descriptor if the provision applies to all signs of that type on the Plans.

I hereby certify that the Special Provisions for traffic sign construction (Division ST) contained in this proposal were prepared by me or under my direct supervision and that I am a duly licensed Professional Engineer under the laws of the State of Minnesota.

_________________________________________                            Engineer Name

Lic.  No.  XXXXX                    Date MM/DD/YY

age

No.

-ST

-ST

-ST

-ST

# DIVISION ST

# (2104) REMOVING MISCELLANEOUS STRUCTURES

## DESCRIPTION

The Contractor shall remove and salvage miscellaneous structures according to 2104, “Removing Pavement and Miscellaneous Structures” and these Special Provisions.

## MATERIALS

The Contractor shall use materials according to 2104, “Removing Miscellaneous Structures” and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions.

## CONSTRUCTION REQUIREMENTS

### Remove By Others

Use for projects that include LOGO signs.

Give at least 14 calendar days advance notice to the General Manager of Minnesota Logos to arrange removal of Logo signs designated on the Plans as “REMOVE BY OTHERS”.

Dave DeSutter

General Manager

Minnesota Logos

952-895-8079

ddesutter@interstatelogos.com

### Remove Sign Panel Type C and Type D

Remove the sign panel and mounting hardware.

Use when the bridge-mounted sign structure is to be reused

Leave the bridge mounted sign structure in place for the later installation of a new sign panel.

### Remove Sign Type C and Type D

Use for mast arm-mounted signs

For each mast arm mounted sign, remove the bracketing and hardware which attaches each sign panel to the traffic signal mast arm.

Use for light fixture-mounted signs

For each sign mounted on a light fixture, remove the bracketing and hardware which attaches each sign panel to the light fixture.

Use for bridge rail-mounted signs

For each bridge rail mounted sign, remove the bridge rail mounted sign structure and mounting hardware.  Fill anchorage holes in concrete with epoxy grout.

Use for bridge-mounted signs

For each bridge mounted sign, remove the sign panel, mounting hardware, and bridge mounted sign structure.

After removal of the sign structure,

Fill anchorage holes in concrete with epoxy grout.

Plug holes in steel with galvanized high strength steel bolts, washers, and nuts.

Use for sign replacement projects.

Schedule the work so that replacement signs are installed on the same work day that the in-place signs are removed.

### Remove Sign Type Special

Remove the entire street name sign structure, including any concrete footings.

### Contractor Responsibility for Salvaged Sign Panels

To be included for any salvage pay items.

Contractor to inform the Engineer of any damaged or missing in-place sign panels prior to salvaging.

If a sign panel is damaged, lost, or stolen after being salvaged, a new sign panel shall be fabricated according to 2564.2F “Traffic Signs and Devices: Signs and Markers,” and these Special Provisions, at no cost to the Department or City of ____, ____ County.

To deter theft, store sign panels in a secure area.

Contractor to prevent damage to the bracket assemblies, sign panels, and the sign sheeting materials at all times, including during storage.  Methods to prevent damage during storage include but are not limited to:

Store sign panels so they are not laying on the ground.

Store sign panels so that the reflective surfaces do not come in contact with dirt, water, or grass.

Store sign panels so they are not covered with a tarp or plastic.

### Salvage Sign Panel Type C

Remove and dispose of the nuts, bolts and washers.

Salvaged sign panels will be reinstalled under Item No. 2564.502 - Install Sign Panel Type C.

### Salvage Sign Panel Type Special

(note if there are two plates mounted back to back)

Salvage the street name plates and bracket assembly.

Leave the sign panels attached to the bracket assembly.

Leave the mounting hardware attached to the flanged channel sign post(s).

If the Contractor damages a bracket assembly or street name plates,

Dispose of the damaged bracket assembly or damaged street name plate.

Fabricate new bracket assembly or street name plate according to City of __,__County specifications, 2564.2F, “Traffic Signs and Devices: Signs and Markers;” and these Special Provisions, at no cost to the Department or City of __, __ County.

Salvaged sign panels, bracket assemblies, and mounting hardware will be reinstalled under Item No. 2564.502 - Install Sign Panel Type Special.

### Salvage Sign Type C and Type D

Remove and dispose of the sign structure, nuts, bolts and washers.

Salvaged Type C sign panels will be reinstalled under Item No. 2564.502 - Install Sign Type C.

Salvaged Type D sign panels will be reinstalled under Item No. 2564.502 - Install Sign Type D.

### Salvage Sign Type Special

Salvage the sign post with the bracket assembly and street name plates attached.

If the Contractor damages a sign post, bracket assembly, or street name plate,

Dispose of the damaged sign post, bracket assembly, or street name plate.

Fabricate a new sign post, bracket assembly, or street name plate according to City of __,__County specifications, 2564.2F, “Traffic Signs and Devices: Signs and Markers;” and these Special Provisions, at no cost to the Department or City of __, __ County.

Salvaged signs will be reinstalled under Item No. 2564.602 - Install Sign Type Special.

### Salvage Delineator and Marker

Remove and dispose of the sign structure, nuts, bolts and washers.

Salvaged delineators will be reinstalled under Item No. 2564.502 - Install Delineator.

Salvaged markers will be reinstalled under Item No. 2564.502 - Install Marker.

## METHOD OF MEASUREMENT & BASIS OF PAYMENT

The Engineer will measure each item according to the Contract and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions.

The Department will include all work described in the Contract and the 2104, “Removing Miscellaneous Structures: Construction Requirements” section of these Special Provisions as part of the contract unit price per unit of measure.

The Department will pay for traffic signs and devices on the basis of the following schedule:

Include only pay items that contain work specified in the special provisions.

| Item No.: | Item: | Unit: |
| --- | --- | --- |
| 2104.502 | Remove Sign Panel Type _ | Each |
| 2104.502 | Remove Sign Type _ | Each |
| 2104.502 | Salvage Sign Panel Type _ | Each |
| 2104.502 | Salvage Sign Type _ | Each |
| 2104.502 | Salvage Delineator | Each |
| 2104.502 | Salvage Marker | Each |

The Department’s payment for each item shall be compensation in full for all work, material, and costs involved in performing the work specified on the Plans and these Special Provisions.

# (2564) TRAFFIC SIGNS AND DEVICES

## DESCRIPTION

The Contractor shall furnish and install traffic signs in accordance with 2564, “Traffic Signs and Devices,” except as modified in these Special Provisions.

## CONSTRUCTION REQUIREMENTS

The provisions of 2564.3A, “Construction Requirements: General” are modified and supplemented as follows:

The following replaces the second paragraph of 2564.3A:

Sign locations and sign structure posts lengths indicated in the Contract are approximate.  Locate and stake final sign and delineator/marker locations.  Obtain approval of locations by the Engineer.  Determine the final post lengths for signs and delineator/markers in accordance with the offsets, mounting heights, and clearances detailed in the Contract and field verification of the proposed or in-place ground slopes.  Obtain approval of the final required post lengths for I-Beam, Monotube, and Overhead Signs prior to starting fabrication of the posts.  Provide shop drawings for I-Beam, Monotube, and Overhead signs in accordance with 2471.3C, “Shop Drawings.”

The provisions of 2564.3A.1, “Sign Fabrication” are modified and supplemented as follows:

The following replaces the second paragraph of 2564.3A.1:

Use sheet aluminum for Sign Panels.

### As-Built Signing Data

DO NOT INCLUDE provisions in DIV ST to collect sign data for inventory management purposes.  Use the Pay Item 2011.601 AS BUILT and follow the directions within DIV S to cover sign inventory management.

### Warning Stickers

Use this provision if new sign panels or sign panel overlays are being installed for Type A, Type C and Type D signs (almost all projects)

Install Department-provided warning stickers on new sign panels according to 2564.3, “Construction Requirements.”

Give 30 days advance notice to the Department prior to picking up the Department-provided warning stickers:

Chris Dochniak	(Metro)-(for other districts, use maintenance area sign supervisor)

651-755-0316

James Smidt	(for D6 use)

507-286-7618

If the project involves Type A signs:

For Type A signs and sign panel overlays on Type A signs,

Affix the warning sticker to each Type A sign panel in the lower right corner of the back of the sign panel, directly above the fabrication sticker.

### Field Spotting of Signs

Use this provision when installing signs where location and orientation is critical (i.e. roundabouts, RCUTs, DDI)

Give the Engineer 14 calendar days advance notice prior to installing signs inside or within 50 feet of roundabouts.

The Engineer will contact the District Traffic Office, which will provide personnel to field spot the installation location and orientation of the signs:

Contact Name

Title

Phone Number

Email@state.mn.us

### Post-Award Data

Use this provision if requested by the District Traffic Office

SignCAD panel layout files for panel layouts shown on the Plans are available electronically upon project award.  To request these files, please contact:

Contact Name

Title

Phone Number

Email@state.mn.us

### MnDOT believes the electronic data it will provide is accurate, but MnDOT provides no guarantee or warranty, express or implied, concerning the accuracy of the data and the Contractor shall not act in reliance on the data without verifying the data against the contract documents.  The documents originally provided with the Contract remain the basis of the Contract, and the electronic data that will be provided at the Request of the Contractor is provided only for the convenience of the Contractor.  Therefore, if use of this data causes an error, omission, unacceptable work, or work not in conformance with the contract documents, then any costs to the Contractor to make corrections as a result of this error will not be considered "extra work", and the Contractor will not be entitled to an adjustment of contract time.

### Sign Replacement Projects

Use this provision on sign replacement projects

The provisions of 2564.3A.3, “Scheduling of Work” are supplemented as follows:

For signs not detailed in 2564.3A.3, “Scheduling of Work”:

Schedule the work so that replacement signs are installed the same workday that the in-place signs are removed.

### New Posts Installed

Use this provision in all of Districts 1 and 2 and in all counties north of and including for District 3 (Kanabec, Mille Lacs, Morrison, Todd) for District 4 (Douglas, Grant, Traverse).

Install base posts to a minimum depth of 60 inches.

### Install Sign Panel Type C and Type D

Sign panel punching codes, stringer details, and panel joint details have been updated to correspond with new square tube post standards and crashworthy requirements.

Field Punch salvaged sign panels as needed based on the new Punching Codes and Stringer Detail file found on MnDOT’s Standard Signs and Markings Manual (2020) website:

http://www.dot.state.mn.us/trafficeng/publ/signsmanual/index.html

Aluminum stringers shall be extruded aluminum made with 6061-T6 Alloy.  The stringer shape shall be a channel with the legs folded back upon themselves to form an enlarged area for mounting to the post.  The stringer shall be two inches wide with a milled surface that the sign will be mounted upon.  The stringer shall have 3/8 inch holes provided at one inch intervals on center.  The legs of the stringer shall be 7/8 inch long to the folded portion.  The nominal thickness of the stringer shall be 1/8 inch.

### Install Sign Panel Type Special

Attach the salvaged street name plate, attached bracket assembly, and new mounting hardware to the square tube sign post as directed by the Engineer.

### Install Sign Type C and D

Sign panel punching codes, stringer details, and panel joint details have been updated to correspond with new square tube post standards and crashworthy requirements.

Field Punch salvaged sign panels as needed based on the new Punching Codes and Stringer Detail file found on MnDOT’s Standard Signs and Markings Manual (2020) website:

http://www.dot.state.mn.us/trafficeng/publ/signsmanual/index.html

Aluminum stringers shall be extruded aluminum made with 6061-T6 Alloy.  The stringer shape shall be a channel with the legs folded back upon themselves to form an enlarged area for mounting to the post.  The stringer shall be two inches wide with a milled surface that the sign will be mounted upon.  The stringer shall have 3/8 inch holes provided at one inch intervals on center.  The legs of the stringer shall be 7/8 inch long to the folded portion.  The nominal thickness of the stringer shall be 1/8 inch.

### Delineator Type Special (Linear Delineation Panels)

Use this provision if using Linear Delineation Panels (Waffle Sheeting) – Very Rare.

This work shall consist of furnishing and installing linear delineation panels upon median barriers and guardrail in accordance with the provisions of MnDOT 2564, the details shown in the Plans, and the following:

Provide linear delineation panels of sizes and colors identified in Tabulation X on Sheet No. XX of the Plan.

Only use linear delineation panels listed on MnDOT’s Approved/Qualified Products List under “Signing: Delineation Devices.”

Remove any inplace linear delineation panels that conflict with the installation of new linear delineation panels shown in the Plan.

Modify the following as necessary to identify mounting requirements and/or panel spacing (if not identified in Plan).

For concrete barrier mounted linear delineation panels:

Place each of the linear delineation panels along the concrete barrier, spaced (edge-to-edge) at the distance apart specified in the Plans.

Vertically space the linear delineation panels 2” below the top edge of concrete barrier to the top edge of the panel.

Prior to installing the linear delineation panels, prepare and clean the concrete surface per

the manufacturer’s detail. Attach the linear delineation panels to concrete barrier surface

with anchor bolts AND adhesive caulk per the manufacturer’s detail.

For guardrail mounted linear delineation panels:

Place each of the linear delineation system panels along the inside vertical surface of the guardrail

trough, spaced (center-to-center) at the distance apart specified in the Plans.  Center each panel horizontally between guardrail post bolt heads.

The Contractor may adjust the spacing of panels if the specified panel distance and placement conflicts with guardrail post bolt heads.  If spacing is adjusted, the Contractor shall ensure approximately uniform spacing between the panels.

Prior to installing the linear delineation panels, prepare and clean the guardrail surface per

the manufacturer’s detail. Attach the linear delineation system panels to the guardrail surface with

adhesive tape per the manufacturer’s detail.

### Infiltration Area Marker

The provisions of 2564.3O, “Infiltration Area Marker” are modified as follows:

Furnish and install a new three wall base and riser post at the location approved by the Engineer.  Attach the furnished sign panel to the flanged channel post with new mounting hardware at a 4 foot mounting height, measured from the highest point of the ground surface below the sign panel to the bottom of the sign panel.

### Post-Installed Adhesive Anchorage

Use this provision when installing Type C or D sign to concrete structures

This section is applicable for any post-installed adhesive anchorage used to facilitate installation of signs to in-place concrete structures.

Except when part of a proprietary anchorage assembly, ensure threaded rods and bolts meet the requirements of 3385, "Anchor Rods," and 3391," Fasteners," respectively.

- Post-Installed Adhesive Anchorages

Adhesive anchorage installers must hold current ACI Adhesive Anchor Installer Certification credentials.  Installers are required to check depth, diameter and condition of the drilled hole, clean the hole, and install the anchorage per the Manufacturer’s Printed Installation Instructions (MPII).  Record the name(s) of all certified installers on the RECORD OF CONTRACTOR/INSTALLER ACI CERTIFICATION form available on the MnDOT Bridges and Structures website under "Bridge Construction; Construction forms and tools".

Furnish only one of the systems listed on the Department’s "Approved/Qualified Products List for Bridge Products, Concrete Adhesive Anchorages for Structural Applications".  Verify that the adhesive has an uncracked characteristic bond strength as specified in the plan.  Install all anchors as specified by the MPII.  Install in sound concrete to a depth equal to the minimum depth specified in the plan or as specified by the manufacturer, whichever is greater.

Meet the following conditions prior to installation and testing:

- Concrete is greater than 14 days old;

- Concrete surface is free of water prior to drilling;

- The hole is dry, as defined below; and

- Any additional requirements listed in the Manufacturer’s Printed Installation Instructions.

A dry hole is defined as:  a hole with no water present within the hole.  If the hole is filled with water, partially filled with water, or water entered the hole during drilling, blow out the water using compressed air and allow a minimum of 24 hours dry-out time before cleaning the hole and installing the anchorage.

It is essential that the adhesive material completely fill the hole in the concrete for proper anchorage performance.  Ensure that the hole is completely filled to the top of the concrete surface in which the anchorage is installed.  Do not permit the adhesive to overtop the concrete surface in a way that will interfere with the placement of the elements.

- Testing of Post-Installed Anchorages

Perform all testing by an independent third party testing agency.  Testing agent must have current ACI Adhesive Anchor Inspector Certification credentials.

Verify the anchor strength and installation procedures by proof testing anchorages in accordance with this specification.  Perform all testing in accordance with ASTM E488, Standard Test Methods for Strength of Anchors in Concrete Elements.  Set up the tension testing device such that no portion of the device bears on the concrete surface within a distance equal to one and a half times the anchorage embedment depth.  Test anchorages to not less than the required proof load as provided in the plan (if no anchor proof load is provided in the plan, contact the Engineer).  Failure criteria of an anchorage test are defined in ASTM E488.

Ensure that nothing interferes with the testing apparatus during the proof test.  Do not perform any caulk prior to testing.

Verify the anchor strength and installation procedure by demonstrating the anchorage system at the first site of field installation.  One passing demonstration is required to be able to move to the remaining production anchorage installations.  Include a proof test in each demonstration installation.  Failure of a proof test will require a modification of installation procedures or use of a different anchorage system and an additional demonstration of the modified or substituted system.   Demonstration anchorages may be used as production anchorages.  The Contractor assumes all liability for repairs that may need to be performed as a result of a failed test.  Record all demonstration results on the PRE-PRODUCTION ANCHORAGES QUALIFICATION TEST REPORT available on the www.dot.state.mn.us/bridge/construction.html under "Construction forms and tools," and furnish the original of the completed form to the Engineer.

Notify the Engineer immediately after any failure.  Provide a non-conformance anchorage replacement plan, to be accepted by the Engineer.  Once accepted by the Engineer:

- Remove all anchorages that fail the field test without damage to the surrounding concrete;

- Redrill holes to remove adhesive bonding material;

- Install replacement anchorages in accordance with the MPII; and,

- Test anchors using the method listed above.

Perform replacement of failed anchorages to the satisfaction of the Engineer and at no cost to the Department.

Payment for all costs associated with furnishing, testing, and installing the anchorages are included with the sign installation.

### Sign Panels Type C and D

For ground mounted signs,

Sign panel punching codes, stringer details, and panel joint details have been updated to correspond with new square tube post standards and crashworthy requirements.

Field Punch salvaged sign panels as needed based on the new Punching Codes and Stringer Detail file found on MnDOT’s Standard Signs and Markings Manual (2020) website:

http://www.dot.state.mn.us/trafficeng/publ/signsmanual/index.html

Aluminum stringers shall be extruded aluminum made with 6061-T6 Alloy.  The stringer shape shall be a channel with the legs folded back upon themselves to form an enlarged area for mounting to the post.  The stringer shall be two inches wide with a milled surface that the sign will be mounted upon.  The stringer shall have 3/8 inch holes provided at one inch intervals on center.  The legs of the stringer shall be 7/8 inch long to the folded portion.  The nominal thickness of the stringer shall be 1/8 inch.

Use this provision when installing street name Type D Signs on bridge structures

For street name Type D signs on bridge mounted structures,

Furnish sign with either steel stringer and attachment angles or an aluminum z-bar as shown in the plans.

Attach each new sign support structure to the in-place concrete structure with new mounting hardware and post-installed anchors as detailed on the Plans and as described in these Special Provisions.

Installation of bridge mounted structures for street name signs includes all costs associated with providing, installing and testing the mounting structure, including the post-installed adhesive anchorages.

Use this provision when installing minor guide sign Type D Signs on bridge structures

For minor guide sign Type D signs on bridge mounted structures,

Furnish sign with the steel stringers, mounting posts, and connection brackets as shown in the plans.

Attach each new sign support structure to the in-place concrete structure with new mounting hardware and post-installed anchors as detailed on the Plans and as described in these Special Provisions.

Installation of bridge mounted structures for minor guide signs includes all costs associated with providing, installing and testing the sign panel and mounting structure, including the post-installed adhesive anchorages.

Use this provision when installing Type C or Type D signs on Concrete Rail Mounted Sign Support structures

For signs on concrete rail mounted structures,

Attach each new sign support structure to the in-place concrete barrier with new mounting hardware and post-installed anchors as detailed on the Plans and as described in these Special Provisions.

Installation of signs on an individual Concrete Rail Mounted Sign Support includes the cost of the sign panel(s) including mounting hardware and all costs associated with providing, installing and testing the mounting structure, including the post-installed adhesive anchorages and bearing pads.

### Sign Panels Type Overlay

Use this provision when including an organization’s official logo/pictograph and attaching them to the panel with rivets .

(pictograph = government agency; logo = private organization)

For Sign(s)/Sign Panel(s) A/D/OH-XX,

Use the official name of logo here for the logo/pictograph design on the sign panel.

If design requirements are publicly available:

Access the logo/pictograph's graphic design standards on the following website:

Insert website here

If design requirements are NOT publicly available:

Contact Company Name to obtain the logo/pictograph’s graphic design standards:

Contact Name

Company Name

Telephone and/or email

Fabricate the sign panel(s) with the logo/pictograph at the size and location shown on Sheet No(s). XX on the Plans.

Attach the logo/pictograph to the sign panel according to 2564.3H, “Traffic Signs and Devices: Construction Requirements: Sign Panels”, paragraph 1.

### Install Sign Type Special

Install each salvaged Sign Type Special:

At the location approved by the Engineer

At the same embedment depth as the salvaged sign.

### Delineator/Marker Panel

Provide delineator/marker sign panel as required by the contract.

Attach the sign panel with new mounting hardware required by the contract.

Affix a Department-provided warning sticker to the backside of each marker sign panel directly above the fabrication sticker. Warning stickers are available at the Department's Transportation District Office specified in the Contract. Provide thirty calendar days advance notice before picking up the stickers.

The Engineer will measure delineator/marker panel as a complete unit, including new mounting hardware and new sign panel.

### Delineator/Marker Sign

Provide delineator/marker sign panel as required by the contract.

Provide and install the square tubular sign post and attach the sign panel with new mounting hardware required by the contract.

Affix a Department-provided warning sticker to the backside of each marker sign panel directly above the fabrication sticker. Warning stickers are available at the Department's Transportation District Office specified in the Contract. Provide thirty calendar days advance notice before picking up the stickers.

The Engineer will measure delineator/marker sign as a complete unit, including new post, new mounting hardware, and new sign panel.

## METHOD OF MEASUREMENT AND BASIS OF PAYMENT

The Engineer will measure each item according to the Contract and the 2564, “Traffic Signs and Devices: Construction Requirements” section of these Special Provisions.

The Department will include all work described in the Contract and the 2564, “Traffic Signs and Devices: Construction Requirements” section of these Special Provisions as part of the contract unit price per unit of measure.

The contract square foot prices for Sign Panels Type EA and Type EO includes the cost of providing and installing the flanged channel or S4x7.7 panel mounting posts.

The Department will pay for traffic signs and devices on the basis of the following schedule:

Include only pay items that contain work specified in these special provisions.

| Item No.: | Item: | Unit: |
| --- | --- | --- |
| 2564.502 | Install Sign Panel Type _ | Each |
| 2564.502 | Install Sign Type _ | Each |
| 2564.502 | Delineator Type Special | Each |
| 2564.502 | Infiltration Area Marker X3-6A | Each |
| 2564.518 | Sign Panels Type _ | Square foot |
| 2564.518 | Sign Panel Overlay Type _ | Square foot |
| 2564.602 | Delineator/Marker Panel | Each |
| 2564.602 | Delineator/Marker Sign | Each |
| 2564.602 | Install Sign Type Special | Each |

The Department’s payment for each item shall be compensation in full for all work, material, and costs involved in performing the work specified on the Plans and these Special Provisions.

# (3352) SIGNS, DELINEATORS, AND MARKERS

## DESCRIPTION

The Contractor shall furnish signs, delineators, and markers in accordance with 3352, “Signs, Delineators, and Markers,” except as modified in these Special Provisions.

The provisions of 3352.2A.1 “Sheet Aluminum” are modified and supplemented as follows:

The following table replaces Table 3352.2-1

| Table 3352.2-1 Sheet Aluminum Thickness |  |
| --- | --- |
| Length of Longest Side | Thickness |
| < 18 | 0.063 + 0.004 |
| >18 - 30 | 0.080 + 0.005 |
| >30 | 0.100 + 0.005 |
| Overlays | 0.063 + 0.004 |
| X4-3 and X4-3a | 0.040 + 0.005 |

### Aluminum Stringers

Add under section 3352.2:

A.8  Aluminum Stringers

Use extruded aluminum alloy 6061-T6 with mill finished surface for stringers used to mount sign panels to square tube posts. The stringer shall have 3/8 inch holes provided at one inch intervals on center.  The nominal thickness of the stringer shall be 1/8 inch.

### Stainless Steel Clamp

Add under section3352.2:

A.9  Stainless Steel Clamps

To clamp stringers to square tube posts use 11 gauge Type 304, #2B finished stainless steel with 3/8"-16 x 2" carriage bolt & serrated flange nut.

### Wraparound Delineators

Add under section 3352.2:

A.10  Wraparound Delineators

Use either cylinder or angular style.

For the cylinder style use either 0.040 or 0.080 inch thick sheet aluminum bent into a cylindrical shape of 6 inches diameter.  Height is 9 inches.  Use either sign sheeting type V, VIII, or XI.

For the angular style use 4 inches wide by 9 inch height by 0.040 inches thick sheet aluminum.  Bend the aluminum sheet 90 degrees along the long axis.  Use either sign sheeting type V or VIII.

### Stainless Steel bolts

Add under section 3352.2:

A.7.c  Stainless Steel Bolts

Use stainless steel bolts as specified in 3391.2E, “Fasteners: Requirements: Stainless Steel Bolts,” with zinc coated steel nylon insert lock nuts. When used to attach sign panels place a stainless steel washer and nylon washer on the sign sheeting surface.

### Galvanized Steel Screw Anchor Bolts

Add under section 3352.2:

A.7.d  Galvanized Steel Screw Anchor Bolts

Use galvanized screw anchor bolts as specified in plans. Galvanize screw ancho bolts in accordance with 3392, “Galvanized Hardware.”

# (3402) SQUARE TUBULAR SIGN POSTS

## DESCRIPTION

The Contractor shall furnish square tubular sign posts in accordance with 3402, “Square Tubular Sign Posts,” except as modified in these Special Provisions.

The provisions of 3402.2C “Weight” are modified and supplemented as follows:

The following replaces the entire section:

Use posts required by the contract and in accordance with Table 3402.2-1.

| Table 3402.2-1 Properties |  |  |
| --- | --- | --- |
| Size | USS Gauge | Weight |
| 1-3/4 inches by 1-3/4 inches | 14 | 1.71 pounds/foot |
| 1-1/2 inches by 1-1/2 inches | 12 | 1.7 pounds/foot |
| 1-3/4 inches by 1-3/4 inches | 12 | 2.06 pounds/foot |
| 2 inches by 2 inches | 12 | 2.42 pounds/foot |
| 2-1/4 inches by 2-1/4 inches | 12 | 2.77 pounds/foot |
| 2-1/2 inches by 2-1/2 inches | 12 | 3.14 pounds/foot |
| 2-3/16 inches by 2-3/16inches | 10 | 3.43 pounds/foot |
| 2-1/2 inches by 2-1/2 inches | 10 | 4.01 pounds/foot |

The post weight shall be within 7 percent of the weight shown for the specified post size and gauge.